JNEOPALLIUM · INDUSTRIAL LOOP GUARDIAN TRAINING GUIDE

Training Guide

FMI Skid Demo — build, run, train, and produce a deployable supervisory network

A complete, copy-paste walkthrough of the training half of the pipeline: install the toolchain, build the platform, run the skid scenarios, train the four diagnostic heads, and produce the ready-to-deploy supervisory network the Deployment Guide consumes.

| Document | Model Training Guide |
| --- | --- |
| Product | Jneopallium Industrial Loop Guardian (FMI Skid Demo) |
| Model | industrial-loop-guardian 1.0.0 |
| Trainer | scripts/demo-industrial-fmi/train_loop_guardian_model.py |
| Output | Deployable JNeopallium layer/neuron config |
| Author | Dmytro Rakovskyi — Kharkiv, Ukraine |
| Date | 23 June 2026 |
| License | BSD 3-Clause |

Contents

1.  What you will achieve

2.  Where training ends and deployment begins

3.  Prerequisites and system requirements

4.  Step 1 — Get the source and build

5.  Step 2 — Run the FMI skid scenarios

6.  Step 3 — Understand the generated outputs

7.  Step 4 — Train the Loop Guardian model

8.  Step 5 — Fast Python model verification

9.  Step 6 — Generate training and run reports

10.  Step 7 — The generated artifacts: a deployable network

11.  Step 8 — Verify the run against the gates

12.  Troubleshooting

13.  Appendix — training cheat-sheet

1  What you will achieve

By the end of this guide you will have: a built Jneopallium platform; deterministic FMI skid runs across nine scenarios; a freshly trained Industrial Loop Guardian maintenance-and-energy model; and — the key output — a complete, deployable JNeopallium network (five layer files, real runtime classes, embedded trained heads, a fixed safety gate, and a ready-made production context) that the companion Deployment Guide loads directly.

Safety ceiling for the whole pipeline Everything the trained model does is supervisory and ADVISORY. PLC/PID/SIS controls remain deterministic and authoritative; the trainer encodes this — the response layer is ADVISORY, hard safety gates are fixed configuration, and OPC UA actuator writes stay separate.

2  Where training ends and deployment begins

| Phase | What happens | Document |
| --- | --- | --- |
| Training (this guide) | Build, run scenarios, train on data, emit the deployable network | Training Guide |
| Deployment | Load the emitted network, configure event sources, run the advisory worker | Deployment Guide |

The hand-off is a single directory of generated artifacts (Step 7). Training produces it; deployment consumes it. The trainer writes JNeopallium-ready layer and neuron configuration *and* a production context file, so the same files that record your training run are the files the worker boots from.

3  Prerequisites and system requirements

Toolchain

| Tool | Version | Why |
| --- | --- | --- |
| Java JDK | 17 or newer (LTS) | Build and run the worker / controller |
| Apache Maven | 3.9 or newer | Multi-module build of master + worker |
| Python | 3.10+ (3.13 tested) | Runs the trainer, the demo runner, and report generator |
| Git | any recent | Clone the repository |
| Docker + Mosquitto (optional) | any recent | Only for the full OPC UA + MQTT protocol path |

The default one-command runner produces deterministic evidence without Docker, a broker, or an OPC UA server. The full protocol path (gateway + IndustrialFmiDemoMain) additionally needs Maven, Mosquitto/Docker, the Python gateway dependencies, and a built FMU.

Verify the toolchain

java -version # expect 17 or newer mvn -version # expect 3.9 or newer python --version # expect 3.10 or newer

4  Step 1 — Get the source and build

git clone https://github.com/rakovpublic/jneopallium.git cd jneopallium mvn clean install

The build installs com.rakovpublic.jneopallium:worker:1.0. The worker already contains the real industrial neuron, processor, and signal classes the trained network references (com.rakovpublic.jneuropallium.worker.net.neuron.impl.industrial.* and friends), so no extra hand-written classes are required to train.

5  Step 2 — Run the FMI skid scenarios

The deterministic runner advances the simulated skid and writes evidence for each scenario. Run all nine, or one at a time:

Linux / macOS

scripts/demo-industrial-fmi/run_demo.sh all scripts/demo-industrial-fmi/run_demo.sh pump-wear scripts/demo-industrial-fmi/run_demo.sh high-temperature-interlock

Windows (PowerShell)

powershell -ExecutionPolicy Bypass -File scripts/demo-industrial-fmi/run_demo.ps1 all

The nine scenarios are normal, load-disturbance, oscillation, pump-wear, temperature-sensor-drift, mqtt-outage, opcua-outage, high-temperature-interlock, and operator-override. Each exercises a different part of the supervisory and safety behaviour.

6  Step 3 — Understand the generated outputs

Each run writes a per-scenario folder under target/:

target/jneopallium-industrial-fmi/<scenario>/ manifest.json process_trace.csv controller_results.jsonl advisory_findings.jsonl model_advisory_findings.jsonl heuristic_advisory_findings.jsonl opcua_audit.jsonl mqtt_audit.jsonl alarms.jsonl interventions.jsonl metrics.json comparison.json

| File | What it holds |
| --- | --- |
| metrics.json | Scenario KPIs: IAE, overshoot, settling, energy, interlock latency, availability |
| comparison.json | Baseline-versus-Jneopallium comparison |
| model_advisory_findings.jsonl | Advisories from the trained model |
| advisory_findings.jsonl | Advisories with owning neuron, economic basis, safety envelope, boundary |
| opcua_audit.jsonl / mqtt_audit.jsonl | Every command/telemetry event, for the protocol boundary |

7  Step 4 — Train the Loop Guardian model

The trainer needs no third-party Python packages. Train and export the maintenance-and-energy model with the production-scale reference command:

python scripts/demo-industrial-fmi/train_loop_guardian_model.py \ --reference-multiplier 1000 \ --target-corpus-bytes 100gb \ --max-corpus-bytes 100gb

It writes the full artifact set to worker/src/main/resources/model/industrial-loop-guardian/ (see Step 7). The trainer fits four diagnostic heads over 39 engineered features, evaluates them on a leakage-safe split, and prints a summary including per-finding precision/recall/F1.

What the flags mean --reference-multiplier deterministically expands the bundled skid corpus. --target-corpus-bytes records the logical production-scale target (100 GiB) reached by replication. --max-corpus-bytes is a hard guardrail that aborts rather than inflate the repository — the fitted sample stays a few tens of MiB so the run completes on a workstation.

The machine-health condition-monitoring subsystem

Alongside the four maintenance-and-energy heads, the updated model includes a multimodal machine-health subsystem (acoustic + vibration) implemented as a Java runtime layer. Its feature extractors, operating-regime normalisation, baseline / domain-shift scoring, fault hypotheses, and advisory gate are designed to be trained and validated on public condition-monitoring datasets:

| Dataset | Role |
| --- | --- |
| MIMII | Industrial machine sound under normal and faulty conditions |
| DCASE machine-condition | Acoustic anomaly-detection benchmark |
| MIMII DUE / DG | Domain-shift / domain-generalisation variants |
| Paderborn Bearing Data Center | Labelled bearing-fault vibration data |
| FMI pump simulator | Bundled deterministic skid signals for repeatable evidence |
| Authorised site telemetry | Your own machines, once a data-handling review is in place |

The subsystem is read-only / shadow / advisory by construction: it emits a MachineHealthAdvisorySignal (health score, per-fault probabilities, unknown-anomaly, domain-shift, uncertainty) and never writes an actuator command. The domain-shift and uncertainty outputs let it say "I am not sure" on an unfamiliar machine instead of over-claiming — which is what real condition-monitoring validation requires before any production reliance.

8  Step 5 — Fast Python model verification

For a quick, build-free check of the model logic, run the Python unit tests:

python -m unittest discover -s scripts/demo-industrial-fmi/tests

These exercise the trainer and the loop-guardian model end to end without Maven, a broker, or a built FMU — ideal for a fast feedback loop while iterating on features or the corpus.

9  Step 6 — Generate training and run reports

After training and replay, render consolidated reports:

python scripts/demo-industrial-fmi/generate_run_reports.py

This collates the per-scenario metrics and the trained-model evidence into reproducible report artifacts you can attach to a pilot assessment.

10  Step 7 — The generated artifacts: a deployable network

This is the pipeline's hand-off. The training run writes a directory of JNeopallium-style configuration that is ready to deploy as-is — the actual layer-and-neuron network the worker boots from, plus a ready-made production context:

| Artifact | What it is |
| --- | --- |
| trained-industrial-loop-guardian-model.json | The compact trained model: features, four heads, weights, metrics |
| model-descriptor.json | Whole-network descriptor: 5 layers, 17 neurons, frequency map, runtime classes |
| layer-0.json | Plant + supervisory-context input boundary |
| layer-1-fast-telemetry.json | Fast telemetry validation and loop state — 7 neurons |
| layer-2-maintenance-energy.json | The four trained diagnostic finding heads |
| layer-3-advisory-planning.json | Economic advisory planning + the fixed safety gate — 4 neurons |
| result-layer.json | Industrial advisory JSONL output |
| trained-model-update.json | Latest trained snapshot: status, checksum, weights, training summary |
| quantitative-summary.json | Scale, metrics, and top positive / negative feature weights per head |
| source-mapping.json | Which typed signals each data source maps onto |
| production-context.json | A ready-to-run advisory-worker context for deployment |

The network the trainer builds

model-descriptor.json records a five-layer network of 17 real neurons, with 156 trainable weights and 4 biases, every neuron referencing a concrete class from the worker's industrial package:

| Layer | Size | Role |
| --- | --- | --- |
| 0 — Input | 0 | Plant / OPC UA / MQTT / FMI replay / Kafka boundary |
| 1 — Fast telemetry | 7 | Validation, interlocks, override, fast loop state |
| 2 — Diagnostic heads | 4 | Pump wear, oscillation/stiction, sensor drift, energy |
| 3 — Advisory planning | 4 | Maintenance scheduling, bounded trim, economic basis, safety gate |
| 4 — Result | 2 | Maintenance and energy advisories as JSONL |

Each diagnostic head embeds its trained weights, the feature gate it is allowed to use (the oscillation head, for example, is gated to 17 of the 39 features), and its logical role and owned reasoning. The descriptor also carries a signalFrequencyMap giving each signal its loop cadence — fast tags every loop, degradation/energy/setpoint every 10, maintenance windows every 60 — the multi-timescale design made concrete.

Why this matters Because training emits real, inspectable, deployable configuration, there is no translation step between "the model we trained" and "the network we run." The Deployment Guide simply points the worker at this directory and loads production-context.json.

11  Step 8 — Verify the run against the gates

1.  All expected source families are present in source-mapping.json (FMI replay, OPC UA, MQTT, Kafka shadow, CMMS, energy meter).

2.  Per-finding F1 is at or near 1.0, and the energy head's false-positive rate is within budget (reference run: macro-F1 ≈ 0.9995, macro false-positive rate ≈ 0.0005).

3.  The five layer files, trained-model-update.json, and production-context.json were written and reference the expected runtime classes.

4.  Every scenario produced deterministic traces and a baseline comparison.

5.  Advisory JSON includes the owning neuron, recommendation code, economic basis, safety-envelope result, and the PLC/PID/SIS-versus-Jneopallium boundary.

Reproducibility check The reference run is deterministic. Re-running the same command must produce the same training checksum (trained-model-update.json) and the same per-finding metrics. A drift in either signals that an input or the toolchain changed.

12  Troubleshooting

| Symptom | Likely cause and fix |
| --- | --- |
| java -version shows < 17 | Install a JDK 17+ and set JAVA_HOME |
| Trainer cannot find scenarios | Run from the repo root; scenarios live in scripts/demo-industrial-fmi/config/scenarios/ |
| "estimated canonical corpus exceeds --max-corpus-bytes" | Lower --reference-multiplier/--target-corpus-bytes or raise the guardrail deliberately |
| FMU / gateway errors | The default runner needs none of these; only the full protocol path requires the FMU + broker |
| Layer files not written | Confirm --output-dir is writable; the trainer writes all artifacts there |
| Unit tests fail after a change | A feature or corpus edit changed behaviour; review the diff and expected metrics |

13  Appendix — training cheat-sheet

Build

git clone https://github.com/rakovpublic/jneopallium.git && cd jneopallium && mvn clean install

Run scenarios

scripts/demo-industrial-fmi/run_demo.sh all # Linux/macOS powershell -File scripts/demo-industrial-fmi/run_demo.ps1 all # Windows

Train, verify, report

python scripts/demo-industrial-fmi/train_loop_guardian_model.py \ --reference-multiplier 1000 --target-corpus-bytes 100gb --max-corpus-bytes 100gb python -m unittest discover -s scripts/demo-industrial-fmi/tests python scripts/demo-industrial-fmi/generate_run_reports.py

Jneopallium Industrial Loop Guardian · Training Guide. The trainer emits a deployable JNeopallium network plus a production context; see the Deployment Guide to run it. Supervisory above PLC/PID/SIS. Safety mode: ADVISORY. License: BSD 3-Clause.