Building Models with CANopen and Jneopallium

User Manual

Industrial CAN bus · advisory ceiling · forbidden-index + allow-list defence

Version: 1.0-SNAPSHOT

License: BSD 3-Clause

Author: Dmytro Rakovskyi

# 1. Introduction

[CANopen](https://www.can-cia.org/) is the higher-layer protocol that sits on top of CAN (Controller Area Network) and structures it into a usable application protocol. CAN itself is the physical and data-link layer that has been the dominant in-vehicle bus since the late 1980s — modern cars carry dozens of CAN bus segments per vehicle, modern industrial machines wire dozens of drives, IO modules, and sensors onto shared CAN buses, and modern battery management systems (BMS) speak CAN to the rest of the system. CANopen, maintained by CAN in Automation (CiA), provides the Object Dictionary model, the device profiles (CiA-401 generic I/O, CiA-402 drives and motion control, CiA-418 battery packs), and the PDO/SDO communication patterns that turn raw CAN frames into a structured device model.

The CANopen bridge connects Jneopallium to industrial CAN networks. It speaks CAN frames over two transport options — Linux SocketCAN for native Linux hosts on real industrial controllers, and USB-CAN dongles (PEAK PCAN-USB, Kvaser, Intrepid) for cross-platform development and lab work. The bridge parses EDS (Electronic Data Sheet) files at startup to validate Object Dictionary paths, subscribes to TPDO (Transmit PDO) messages for cyclic data, supports SDO (Service Data Object) reads for non-cyclic queries, and publishes advisory recommendations as TPDO writes to allow-listed OD indices only. The decoded values become typed Jneopallium signals: ProprioceptiveSignal for CiA-402 drive states (encoder position, velocity, torque), MeasurementSignal for generic CiA-401 I/O and sensor PDOs, EnergySignal for CiA-418 battery pack metrics, BatchStateSignal for drive state-machine snapshots (statusword decoded), and AlarmSignal for bridge-level liveness or fault events.

The bridge is advisory by structural ceiling. This is the first bridge in the directory with a two-layer write defence:

First, a hard-coded forbidden OD index list rejects writes to safety-critical indices unconditionally. The current set is FORBIDDEN_WRITE_OD_INDICES = { 0x6040 } — the CiA-402 controlword. Autonomous cyclic writes to the controlword would arm a drive without external supervision; the bridge structurally cannot do this regardless of any other configuration. A write binding targeting 0x6040 fails to load with IllegalArgumentException and a message naming 13-CANOPEN.md §3, §5.

Second, a per-node write-index allow-list narrows the surface further. Every write binding must satisfy writeIndexAllowList[nodeId].contains(odIndex), even if the OD index isn't on the forbidden list. The operator who deploys the bridge declares per-node which indices are advisory-writable in the YAML; bindings targeting un-allow-listed (nodeId, odIndex) pairs fail to load with a message naming 13-CANOPEN.md §6. This is the "structural defence" of §10 R3: every advisory write must clear two independent gates. The forbidden list is hard-coded; the allow-list is operator-declared per deployment. Together they make accidentally autonomous control of a motor or brake structurally impossible.

This manual explains how to install the dependency, configure the bridge against a CAN bus (real industrial line or virtual development CAN), parse EDS files, design PDO read bindings and structurally-defended advisory write bindings, validate end to end, and understand the regulatory posture. It assumes a Java engineer who knows Maven, has read the universal bridge framework (00-FRAMEWORK.md), and has at least basic familiarity with CANopen concepts (the Object Dictionary, PDO/SDO transfers, CiA-402 statusword/controlword, NMT states, EDS files).

# 2. Six rules you must internalise before writing any code

The universal six from 00-FRAMEWORK.md §0, specialised for CANopen:

- No raw write to a safety-critical OD index from anywhere. The hard-coded FORBIDDEN_WRITE_OD_INDICES set rejects 0x6040 (controlword) at config load unconditionally. Future additions to the set (DSP-402 mode of operation 0x6060, torque-limited modes, etc.) extend the unconditional defence. There is no environment variable, no flag, no test-mode bypass.

- Per-node allow-listing is mandatory for every advisory write. Even non-forbidden indices must be on writeIndexAllowList[nodeId]. The operator declares the allow-list per deployment; bindings outside the allow-list fail to load. This makes the operator-side safety analysis explicit and reviewable.

- Interlocks have advisory authority only. A drive-side hardware interlock (E-stop button, limit switch wired to the drive) remains primary. A HarmVetoSignal from Jneopallium becomes an advisory write to an operator-configured advisory OD index (typically a parameter the safety PLC reads); it cannot directly disarm a drive — that authority lives in certified safety logic on the drive itself or on a separate safety PLC.

- Every CAN frame, every SDO transaction, and every advisory write produces an audit record. Records cover decoded PDOs, SDO timeouts, NMT state changes, EMCY (emergency) frames, allow-list rejections at runtime, and bus-off events. Local JSONL only — operator-side bus tracing (PCAN-View, candump, CANalyzer) remains the source of truth for the bus traffic.

- Quality propagates. PDO frames don't carry explicit quality, but heartbeat-derived liveness produces Quality.UNCERTAIN on stale data (no heartbeat within the configured window) and Quality.BAD when a node drops to NMT pre-operational or stopped state. EMCY frames also downgrade quality.

- Wall-clock timestamps come from the receive side. CAN frames don't carry timestamps; the bridge stamps them at receive time from SocketCanClientService or UsbCanClientService. For applications requiring tighter sync, hardware-side time-stamping at the CAN controller is preferred — operator-side configuration.

# 3. Prerequisites

- JDK 17 or later. The bridge uses Java records, java.nio.ByteBuffer for frame encoding, and javax.xml.stream (StAX) for EDS parsing.

- Maven 3.9 or later.

- A CAN bus reachable from the host. Two paths:

- SocketCAN (Linux-native) — vcan0 for development on a Linux host without hardware, or a real can0 / can1 interface on industrial controllers. Requires the can-utils package on the host and a properly-configured kernel CAN module. The bridge uses [JNA](https://github.com/java-native-access/jna) (Java Native Access) to call SocketCAN system calls — production deployments add net.java.dev.jna:jna-platform to the deployment module.

- USB-CAN dongle (cross-platform) — PEAK PCAN-USB, Kvaser Leaf, Intrepid neoVI. The bridge ships an abstract UsbCanClientService that production deployments specialise per vendor SDK. Useful for Windows/macOS development and for industrial deployments where SocketCAN isn't an option.

- An EDS (Electronic Data Sheet) file per node, supplied by the device vendor. The parser only uses the OD index → object name mapping; vendor-specific fields are ignored. Without EDS, the bridge falls back to raw index numbers in logs — usable but harder to read.

- For operational deployment in a real industrial cell: safety analysis per IEC 62061 / ISO 13849 (depending on cell classification), interlock validation, and a maintenance-of-changes process. None of these is a software dependency, but they are non-optional for any deployment where the bridge can influence a motor, brake, or BMS.

# 4. Adding Jneopallium to your project

The CANopen bridge ships in the same worker artifact as the rest of Jneopallium:

<repositories> <repository> <id>gitlab-jneopallium</id> <url>https://gitlab.com/api/v4/projects/44434192/packages/maven</url> </repository> </repositories> <dependencies> <dependency> <groupId>com.rakovpublic.jneopallium</groupId> <artifactId>worker</artifactId> <version>1.0-SNAPSHOT</version> </dependency> </dependencies>

For SocketCAN-backed deployments, add JNA in your deployment module (not the worker):

<dependency> <groupId>net.java.dev.jna</groupId> <artifactId>jna-platform</artifactId> <version>5.14.0</version> <scope>runtime</scope> </dependency>

For USB-CAN dongles, add the vendor SDK in your deployment module — typically as a JAR provided by the dongle manufacturer. The bridge stays unaware of which SDK you use; you supply a UsbCanClientService subclass that delegates to the vendor's Java API.

# 5. Architecture

The bridge introduces sixteen classes under worker.bridge.canopen:

┌──────────────────────────────────┐ CAN frames (PDO/SDO/NMT/EMCY) │ CAN bus segment │ ────────────────────────────────┐ │ │ │ │ Node 1: drive (CiA-402) │ │ │ OD 0x6040: controlword [W*] │ │ │ OD 0x6041: statusword [R] │ │ │ OD 0x6064: actual position │ │ │ OD 0x606C: actual velocity │ │ │ │ │ │ Node 2: IO module (CiA-401) │ │ │ OD 0x6000: digital inputs │ │ │ │ │ │ Node 3: BMS (CiA-418) │ │ │ OD 0x9000: state-of-charge │ │ │ │ ◀──── advisory writes ──────────│ │ [W*] = forbidden index │ (allow-listed indices only) │ └──────────────────────────────────┘ │ ▼ ┌──────────────────────────────────────┐ │ CanopenClientService │ │ • SocketCAN or USB-CAN transport │ │ • EDS parser output │ │ • per-node heartbeat tracking │ │ • PDO decoder pipeline │ │ • SDO request/response state mach. │ │ • per-binding signal queues │ │ • runtime allow-list re-check │ └─┬─────────┬──────────┬────────┬──────┘ │ │ │ │ ┌──────────────▼─┐ ┌─────▼────┐ ┌───▼────┐ ┌─▼─────────────┐ │ CanopenMeas │ │ Canopen │ │ Canopen│ │ CanopenEvent │ │ urementInput │ │ StateInp │ │ Telem? │ │ Input │ │ (Measurement │ │ (Batch- │ │ (Pro- │ │ (Alarm: │ │ + Energy) │ │ State) │ │ prio) │ │ EMCY, NMT) │ └────────────────┘ └──────────┘ └────────┘ └───────────────┘ ▼ [Standard Jneopallium pipeline] ▼ CanopenAdvisoryOutputAggregator │ │ - Forbidden-index runtime re-check │ - Allow-list runtime re-check │ - Clamp + rate-limit │ ▼ CanopenClientService.send() │ ▼ CanopenAuditOutput (JSONL, local)

Components:

| Class | Role |
| --- | --- |
| CanopenBridgeConfig | YAML record (immutable). Rejects forbidden OD indices and un-allow-listed (nodeId, odIndex) pairs at config load. |
| CanopenBridgeConfigLoader | Jackson YAML loader. FAIL_ON_UNKNOWN_PROPERTIES = true. |
| EdsParser | Reads .eds files. Surfaces OD index → object name + type mapping. Vendor-specific fields ignored. |
| ObjectDictionaryEntry | Resolved OD entry — index, subindex, type, name, mapped-to PDO source. |
| CanFrame | Immutable CAN frame value type — CAN id, data, dlc, timestamp. |
| CanopenNodeBinding | Resolved per-node binding identity. |
| CanopenSignalMapper | Pure functions. Decodes PDO/SDO/EMCY frames → typed signals. |
| AbstractCanopenClientService | Base class for transport-specific client implementations. |
| SocketCanClientService | Production: Linux SocketCAN via JNA. |
| UsbCanClientService | Production: abstract base for USB-CAN dongle SDKs. |
| CanopenClientService | Top-level lifecycle owner. Builds the right concrete service per canBus.type. |
| CanopenMeasurementInput | IInitInput. Drains MEASUREMENT and EFFICIENCY bindings (generic I/O + BMS metrics). |
| CanopenStateInput | IInitInput. Drains BATCH_STATE bindings (CiA-402 statusword decoded). |
| CanopenEventInput | IInitInput. Drains ALARM bindings (EMCY, NMT state changes, bus-off). |
| CanopenAdvisoryOutputAggregator | IOutputAggregator. Encodes signals → CAN frames. Re-checks forbidden index + allow-list at runtime. |
| CanopenAuditOutput | Local JSONL append-only audit. |

A note on the input split: there is no CanopenTelemetryInput separate class in the v1 — proprioceptive joint-state signals from CiA-402 drives are handled by CanopenMeasurementInput along with generic measurement bindings, with the mapper choosing the signal class per signalKind. The split could be added in a future version if the per-input neuron-attachment ergonomics warrant it.

# 6. The two-gate write defence

The CANopen bridge has a uniquely layered write defence appropriate for the protocol's safety surface. Every advisory write must clear two independent gates at config load, plus a runtime re-check on each send.

Gate 1 — Hard-coded forbidden OD indices. The set CanopenBridgeConfig.FORBIDDEN_WRITE_OD_INDICES is { 0x6040 } — the CiA-402 controlword. The controlword is the index used to arm, disarm, fault-reset, and command motion on a CiA-402 drive. Autonomous cyclic writes to it would arm a drive without external supervision — exactly the failure mode the structural defence prevents. The set is hard-coded in source; extending it (e.g. to 0x6060 mode of operation under certain configurations) requires a code change and a re-release.

Gate 2 — Per-node allow-list. The operator declares writeIndexAllowList: { 0x21: [0x607A, 0x60FF], 0x22: [0x6071] } per deployment. Every write binding must satisfy writeIndexAllowList[nodeId].contains(odIndex). Bindings outside the allow-list fail to load with a message naming the binding, the (nodeId, odIndex) pair, and the spec section.

Both gates are at config load. The runtime additionally re-checks both in CanopenAdvisoryOutputAggregator.save(): even if a future code path bypasses the config-load check (programmatic construction, serialization round-trips, refactors), the runtime guard still throws. Under correct configuration the runtime check never fires.

The cumulative effect: a Jneopallium deployment cannot, under any configuration, at any moment, write to:

- 0x6040 controlword on any node (gate 1, unconditional).

- Any (nodeId, odIndex) pair the operator hasn't explicitly allow-listed (gate 2, operator-declared).

The operator's allow-list is the operator-side safety analysis encoded in YAML. A reviewer auditing the YAML can see exactly which indices the AI is allowed to advisorily write per node. The list is short and explicit by intent — narrow allow-lists are easier to review than broad ones.

# 7. Configuration

Configuration is YAML, loaded once at startup through CanopenBridgeConfigLoader.load(Path). Hot-reload is not supported. The loader sets FAIL_ON_UNKNOWN_PROPERTIES = true.

## 7.1 Schema

The configuration has nine top-level keys.

- canBus — transport type (SOCKETCAN or USB_CAN), interface name, bitrate.

- nodes — list of CANopen nodes on the bus, each with id, name, and optional EDS file path.

- reads — list of PDO/SDO read bindings.

- events — list of event read bindings (EMCY, NMT state, etc.).

- writes — list of advisory write bindings.

- writeIndexAllowList — mandatory per-node allow-list of writable OD indices.

- audit — local JSONL audit file path.

- perTagSafetyMode — per-binding SHADOW / ADVISORY. AUTONOMOUS rejected at load.

- tickInterval — worker tick cadence. Default PT0.25S.

## 7.2 Minimal example — virtual CAN smoke test

The smallest useful configuration. Uses a Linux vcan0 virtual CAN interface, one node with one read binding, no writes. Suitable as a first integration check.

canBus: type: SOCKETCAN interface: "vcan0" bitrate: 500000 nodes: - id: 0x21 name: "drive-axis-1" edsFile: "/etc/jneopallium/eds/axis1.eds" reads: - bindingId: "AXIS1-POS" nodeId: 0x21 odIndex: 0x6064 # CiA-402 position actual value odSubindex: 0x00 pdoSource: TPDO1 signalKind: PROPRIOCEPTIVE signalTag: "AXIS.1.POS" events: [] writes: [] writeIndexAllowList: {} audit: localAuditFile: "/var/log/jneopallium/canopen-audit.jsonl" tickInterval: "PT0.1S"

## 7.3 Production-like example — multi-node with advisory writes

A more substantial config: a SocketCAN line with three nodes (a CiA-402 drive, a CiA-401 I/O module, a CiA-418 BMS), TPDO bindings for proprioceptive, measurement, and energy signals, plus two advisory write bindings on the drive (target position 0x607A and target velocity 0x60FF — both must appear in the allow-list).

canBus: type: SOCKETCAN interface: "can0" bitrate: 1000000 nodes: - id: 0x21 name: "drive-axis-1" edsFile: "/etc/jneopallium/eds/axis1.eds" - id: 0x40 name: "io-module" edsFile: "/etc/jneopallium/eds/io-module.eds" - id: 0x50 name: "bms-pack-1" edsFile: "/etc/jneopallium/eds/bms-pack.eds" reads: - bindingId: "AXIS1-POS" nodeId: 0x21 odIndex: 0x6064 pdoSource: TPDO1 signalKind: PROPRIOCEPTIVE signalTag: "AXIS.1.POS" - bindingId: "AXIS1-VEL" nodeId: 0x21 odIndex: 0x606C pdoSource: TPDO1 signalKind: PROPRIOCEPTIVE signalTag: "AXIS.1.VEL" - bindingId: "AXIS1-STATE" nodeId: 0x21 odIndex: 0x6041 # statusword pdoSource: TPDO1 signalKind: BATCH_STATE signalTag: "AXIS.1.STATE" - bindingId: "IO-DI" nodeId: 0x40 odIndex: 0x6000 # CiA-401 digital inputs pdoSource: TPDO1 signalKind: MEASUREMENT signalTag: "IO.DI" - bindingId: "BMS-SOC" nodeId: 0x50 odIndex: 0x9000 # CiA-418 state-of-charge pdoSource: TPDO2 signalKind: EFFICIENCY signalTag: "BMS.SOC" events: - bindingId: "AXIS1-EMCY" nodeId: 0x21 eventType: EMCY signalTag: "AXIS.1.EMCY" - bindingId: "AXIS1-NMT" nodeId: 0x21 eventType: NMT signalTag: "AXIS.1.NMT" writes: - bindingId: "AXIS1-TARGET-POS" nodeId: 0x21 odIndex: 0x607A # CiA-402 target position — allow-listed below signalTag: "AXIS.1.RECOMMEND.POS" minClampValue: -1000000.0 maxClampValue: 1000000.0 - bindingId: "AXIS1-TARGET-VEL" nodeId: 0x21 odIndex: 0x60FF # CiA-402 target velocity — allow-listed below signalTag: "AXIS.1.RECOMMEND.VEL" minClampValue: -10000.0 maxClampValue: 10000.0 writeIndexAllowList: 0x21: - 0x607A - 0x60FF audit: localAuditFile: "/var/log/jneopallium/canopen-audit.jsonl" perTagSafetyMode: AXIS1-TARGET-POS: ADVISORY AXIS1-TARGET-VEL: ADVISORY tickInterval: "PT0.05S"

Note the writeIndexAllowList lists exactly the two indices the deployment intends to advisorily write. Adding any other write binding (say, 0x6071 target torque) fails to load until the allow-list is updated. And no listing can ever unlock 0x6040 — that's gate 1's job.

## 7.4 Field reference

canBus.type is SOCKETCAN or USB_CAN. SocketCAN is the universal choice on Linux production hosts; USB-CAN is for cross-platform development and for industrial deployments where SocketCAN isn't an option.

canBus.interface is the OS-level interface name. For SocketCAN: vcan0, can0, can1. For USB-CAN: the vendor-specific identifier (e.g. PCAN_USBBUS1 for PEAK).

canBus.bitrate is the bus bitrate in bits per second. 1 Mbps is typical for industrial cells; 500 kbps for long bus runs or high-noise environments; 125 kbps for legacy systems.

nodes[].id is the CANopen node id (1-127). Conventionally written in hex (0x21 = 33). Cycles must not contain duplicate node ids.

nodes[].edsFile is the local path to the EDS file. Optional but recommended; without it the bridge logs raw OD indices instead of names.

reads[].odIndex / odSubindex is the (index, subindex) pair in the Object Dictionary. Subindex defaults to 0.

reads[].pdoSource is the PDO that carries this value: TPDO1, TPDO2, TPDO3, TPDO4, or SDO for acyclic reads. Cyclic data should always use TPDOs; SDO is for occasional configuration reads.

reads[].signalKind is one of PROPRIOCEPTIVE (CiA-402 drive state), MEASUREMENT (generic I/O), EFFICIENCY (CiA-418 BMS), BATCH_STATE (CiA-402 statusword decoded), or ALARM.

writes[].odIndex is the OD index to write. Must clear gate 1 (not on FORBIDDEN_WRITE_OD_INDICES) and gate 2 (in writeIndexAllowList[nodeId]).

writes[].minClampValue / maxClampValue are pre-write clamps applied by the aggregator. Values outside the range are moved to the nearest bound and audited.

writeIndexAllowList is mandatory for any deployment with writes:. Keys are node ids; values are lists of allow-listed OD indices for that node. Bindings outside the allow-list fail to load.

perTagSafetyMode keys are bindingIds. Legal values: SHADOW, ADVISORY. AUTONOMOUS is rejected with IllegalArgumentException at load — the structural ceiling is ADVISORY.

tickInterval is the worker tick cadence. Cyclic PDOs at 1 ms typical industrial cycle rate require tickInterval: PT0.001S on the worker side; slower processes can use longer intervals.

# 8. Wiring the Java side

A minimal driver:

package com.example.cell; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenAdvisoryOutputAggregator; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenBridgeConfig; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenBridgeConfigLoader; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenClientService; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenEventInput; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenMeasurementInput; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenSignalMapper; import com.rakovpublic.jneuropallium.worker.bridge.canopen.CanopenStateInput; import com.rakovpublic.jneuropallium.worker.bridge.canopen.EdsParser; import com.rakovpublic.jneuropallium.worker.bridge.common.AbstractBridgeAuditOutput; import java.nio.file.Path; public final class CanopenBootstrap { public static void main(String[] args) throws Exception { // 1. Load YAML. Forbidden-index and allow-list rejections happen here. CanopenBridgeConfig cfg = CanopenBridgeConfigLoader.load( Path.of("canopen-bridge.yml")); // 2. Parse EDS files. Optional but recommended. EdsParser.Catalog edsCatalog = EdsParser.parseAll(cfg.nodes()); // 3. Open the audit sink. try (AbstractBridgeAuditOutput audit = new CanopenAuditOutput( Path.of(cfg.audit().localAuditFile()))) { // 4. Build the signal mapper. CanopenSignalMapper mapper = new CanopenSignalMapper(cfg, edsCatalog); // 5. Build the client service. Factory picks SocketCAN or USB-CAN // transport based on canBus.type. CanopenClientService svc = CanopenClientService.build( cfg, mapper, audit); // 6. Start: open CAN interface, register PDO decoders, send NMT // operational to bring nodes into PDO-active state. svc.start(); try { // 7. Build the inputs. Three by signal family. CanopenMeasurementInput meas = new CanopenMeasurementInput( "canopen-meas", svc); CanopenStateInput state = new CanopenStateInput( "canopen-state", svc); CanopenEventInput events = new CanopenEventInput( "canopen-events", svc); // 8. Build the aggregator. Runtime forbidden-index + // allow-list re-checks fire inside send(). CanopenAdvisoryOutputAggregator agg = new CanopenAdvisoryOutputAggregator(cfg, svc, audit); // 9. Register with the worker. worker.registerInput(meas); worker.registerInput(state); worker.registerInput(events); worker.registerOutputAggregator(agg); worker.run(); } finally { svc.close(); } } } }

Three notes worth highlighting. Three inputs by signal family. Measurement (generic I/O + BMS energy), state (CiA-402 statusword-decoded snapshots), events (EMCY + NMT alarms). Different from MAVLink's split because the CANopen surface has a structurally different shape — CiA-402 statusword is rich enough to deserve its own input. `CanopenClientService.build()` is a factory. It picks SocketCanClientService or UsbCanClientService based on canBus.type. The application doesn't see the difference. Runtime defence inside `send()`. Under correct config it never fires; defence in depth ensures it catches any programmatic-construction shortcut that bypassed gate 1 or gate 2 at config load.

# 9. Modeling guidance

## 9.1 What to read

For a typical industrial CANopen deployment:

- CiA-402 drive states from each axis — actual position (0x6064), actual velocity (0x606C), actual torque (0x6077), statusword (0x6041). Map to PROPRIOCEPTIVE and BATCH_STATE.

- Generic I/O modules — digital inputs (0x6000), digital outputs (0x6200), analog inputs (0x6401). Map to MEASUREMENT.

- Battery management systems — state-of-charge (0x9000), pack voltage (0x9001), pack current (0x9002), cell temperatures. Map to EFFICIENCY.

- Drive EMCYs — emergency events from each drive. Map to ALARM via eventType: EMCY.

- NMT state changes — operational, pre-operational, stopped. Map to ALARM via eventType: NMT.

## 9.2 The OD index → signal class mapping

The mapper handles signal kinds according to the binding's signalKind config:

| `signalKind` | Signal class produced | Typical OD source |
| --- | --- | --- |
| PROPRIOCEPTIVE | ProprioceptiveSignal | CiA-402 0x6064 (position), 0x606C (velocity), 0x6077 (torque) |
| MEASUREMENT | MeasurementSignal | CiA-401 0x6000 (DI), 0x6200 (DO), 0x6401 (AI) |
| EFFICIENCY | EnergySignal | CiA-418 0x9000+ (BMS metrics) |
| BATCH_STATE | BatchStateSignal | CiA-402 0x6041 statusword, decoded to state-machine state |
| ALARM | AlarmSignal | EMCY, NMT (via eventType) |

The statusword decoding turns 16 bits of state into a structured state-machine snapshot: NOT_READY_TO_SWITCH_ON, SWITCH_ON_DISABLED, READY_TO_SWITCH_ON, SWITCHED_ON, OPERATION_ENABLED, QUICK_STOP_ACTIVE, FAULT_REACTION_ACTIVE, FAULT. This is more useful than raw integer signals — neurons reasoning about drive state can match on enum values.

## 9.3 Adding an advisory write

The procedure (in operator-controlled order):

- YAML — allow-list update. Add the (nodeId, odIndex) pair to writeIndexAllowList. This is the operator's safety-analysis declaration.

- YAML — write binding. Add the writes: entry with clamp values appropriate for the OD's physical units (motor revolutions, mm, etc.).

- YAML — start in SHADOW. perTagSafetyMode: { NEW-WRITE: SHADOW }. Audit-only.

- Observe SHADOW for an observation period. Inspect audit JSONL. Verify proposed values are sensible per the cell's safety envelope.

- Promote to ADVISORY. Edit YAML, restart. Writes flow to the drive's allow-listed index. The drive itself, the safety PLC, or the operator's HMI decides what to do with the advisory value.

There is no AUTONOMOUS step. The bridge ceiling is structurally ADVISORY.

## 9.4 Why the controlword is forbidden

Among CANopen indices, 0x6040 is uniquely hazardous. It's the index that commands a CiA-402 drive to:

- Switch on (0x0007).

- Enable operation (0x000F).

- Quick stop (0x0002).

- Disable voltage (0x0000).

- Fault reset (0x0080).

- Halt (set bit 8 of an otherwise valid command).

A cyclic autonomous writer to 0x6040 effectively has full motor control. The safety case for any AI-on-CANopen deployment cannot include autonomous control of this index without the AI itself becoming the safety-critical component — which would require IEC 62061 / ISO 13849 certification of the AI, an enormous burden the bridge doesn't and shouldn't take on. Hard-coding 0x6040 in the forbidden set keeps the AI's role advisory regardless of what the YAML or runtime might attempt.

If a future deployment legitimately needs structured drive arming (e.g. for a research robot in a closed cell with redundant safety interlocks), that's a separately certified canopen-control bridge with its own safety case — not a flag flip on this one.

# 10. The advisory egress algorithm

When the worker calls CanopenAdvisoryOutputAggregator.save(), for each signal:

- Resolve the write binding by signalTag. UNBOUND → audit REJECTED reason=UNBOUND_SIGNAL_TYPE. No send.

- Resolve per-tag safety mode. SHADOW → audit REJECTED reason=SHADOW_MODE. No send.

- Runtime gate 1 — re-check the OD index against FORBIDDEN_WRITE_OD_INDICES. If forbidden → audit REJECTED reason=FORBIDDEN_INDEX_RUNTIME. No send. Under correct config this never fires.

- Runtime gate 2 — re-check (nodeId, odIndex) against writeIndexAllowList. If missing → audit REJECTED reason=ALLOW_LIST_RUNTIME. No send. Under correct config this never fires.

- Clamp the value to [minClampValue, maxClampValue]. CLAMPED_HIGH or CLAMPED_LOW recorded in audit if a clamp fired.

- Encode the value as an SDO write or a configured RPDO frame.

- Send the frame via the transport. APPLIED → audit APPLIED reason=ADVISORY_WRITTEN. On bus-off or transport error → audit FAILED reason=SEND_ERROR:<detail>.

Two gates at config load, two gates at runtime, plus clamp. The cumulative defence makes accidental autonomous control structurally impossible.

# 11. Audit and transparency

The bridge writes one JSONL record per decision to the local audit:

{ "ts": 1740000000000, "run": 12345, "bridge": "canopen", "verdict": "APPLIED", "tag": "AXIS.1.RECOMMEND.POS", "nodeId": 33, "odIndex": "0x607A", "proposed": 12345.6, "effective": 12345.6, "reason": "ADVISORY_WRITTEN", "safetyMode": "ADVISORY" }

Records you will see:

| `verdict` | `reason` | Triggered by |
| --- | --- | --- |
| APPLIED | ADVISORY_WRITTEN | Advisory write sent to allow-listed (nodeId, odIndex). |
| APPLIED | PDO_DECODED | Inbound PDO decoded into a signal. |
| APPLIED | SDO_RESPONSE_OK | SDO read response received and decoded. |
| APPLIED | NMT_OPERATIONAL | Node entered NMT operational state. |
| APPLIED | BRIDGE_RECONNECTED | CAN interface recovered after bus-off. |
| APPLIED | CLAMPED_HIGH / CLAMPED_LOW | Value clamped before write. |
| REJECTED | SHADOW_MODE | Binding in SHADOW; no send. |
| REJECTED | UNBOUND_SIGNAL_TYPE | Tag has no writes: entry. |
| REJECTED | FORBIDDEN_INDEX_RUNTIME | Runtime gate 1 caught a forbidden OD index (should never fire under correct config). |
| REJECTED | ALLOW_LIST_RUNTIME | Runtime gate 2 caught an un-allow-listed (nodeId, odIndex) (should never fire). |
| FAILED | SDO_TIMEOUT:<nodeId> | SDO transaction timed out. |
| FAILED | BUS_OFF | CAN controller entered bus-off state. |
| FAILED | EMCY:<errorCode> | Emergency message received from a node. |
| FAILED | SEND_ERROR:<detail> | Transport-level send error. |

The audit file is local. Operator-side CAN bus tracing (PCAN-View, candump, CANalyzer) remains the source of truth for the raw bus traffic. The bridge audit is for bridge-level decisions, not for bus-level duplication.

# 12. Operating procedures

## 12.1 Smoke test with vcan0

The fastest end-to-end check uses Linux's virtual CAN:

- Enable virtual CAN on the host:

``bash    sudo modprobe vcan    sudo ip link add dev vcan0 type vcan    sudo ip link set up vcan0    ``

- Save §7.2 minimal config to canopen-bridge.yml.

- In another shell, generate test PDO traffic:

``bash    cansend vcan0 1A1#0000000000000000   # TPDO1 from node 0x21    ``

- Run CanopenBootstrap (§8).

- The bridge receives the frame; CanopenMeasurementInput.readSignals() returns a ProprioceptiveSignal.

## 12.2 Verifying the two-gate defence

A useful test sequence:

- Forbidden index at config load. Add a writes: binding targeting 0x6040 and an allow-list including 0x6040 for that node. Load. Assert IllegalArgumentException — gate 1 (forbidden list) rejects it regardless of the allow-list.

- Allow-list miss at config load. Add a writes: binding for OD 0x607A but leave 0x607A out of writeIndexAllowList[nodeId]. Load. Assert IllegalArgumentException — gate 2 catches it.

- Both pass. Add a writes: binding for OD 0x607A and include 0x607A in writeIndexAllowList. Load succeeds.

- Runtime gate 1. Programmatically construct a config bypassing the compact constructor (test-only). Try to write to 0x6040. Assert it throws at runtime.

- Runtime gate 2. Same as 4, but with an un-allow-listed (nodeId, odIndex). Assert runtime throws.

Five tests, two structural gates plus their runtime fallbacks. All five should pass on every release.

## 12.3 Bus-off recovery

CAN controllers enter bus-off after many consecutive transmission failures (electrical fault, controller mismatch). The bridge handles this:

- The controller transitions to bus-off; subsequent transmissions fail.

- Audit emits FAILED reason=BUS_OFF.

- The bridge waits the configured backoff and resets the controller.

- After successful reset, audit emits APPLIED reason=BRIDGE_RECONNECTED.

- NMT operational commands re-broadcast to bring nodes back into PDO-active state.

If bus-off recurs immediately after recovery, investigate the bus topology — likely a wiring fault, termination issue, or mismatched bitrate.

## 12.4 EMCY handling

CANopen emergency messages (EMCY) carry an 8-byte payload with error code, error register byte, and 5 manufacturer-specific bytes. The bridge:

- Decodes the EMCY frame on receipt.

- Emits AlarmSignal via CanopenEventInput with the error code as the alarm's reason and severity mapped from the CiA-301 error register.

- Audits FAILED reason=EMCY:<errorCode>.

Downstream neurons consume the alarm signal. The bridge does not take action on the EMCY itself — that's the drive's onboard safety logic and the operator's safety PLC.

# 13. Cybersecurity and safety posture

CAN networks are not secure by design. The bridge respects this:

- No CAN authentication. CAN bus security (CAN XL, CANcrypt) is outside scope. The bridge consumes whatever traffic appears on the bus.

- No bus-level isolation enforcement. The operator's network architecture (VLAN-equivalent bus segmentation, firewall between bus segments) remains primary.

- Local audit only. No external network egress. The bridge connects only to the configured CAN interface.

- Allow-list is the safety-analysis artefact. Reviewing the YAML's writeIndexAllowList shows exactly what the AI can advisorily write. Narrow allow-lists mean shorter safety arguments.

- No replacement for cell safety hardware. E-stop buttons, light curtains, safety relays, safety PLCs remain primary. The bridge does not replace them.

For deployments under IEC 62061 (functional safety of electrical control systems) or ISO 13849 (safety of machinery — control systems): the bridge contributes a structural defence (no controlword writes, allow-list-bounded other writes) that those frameworks recognise. The bridge does not by itself satisfy any SIL or PL classification.

# 14. Limitations and future work

- No writes to 0x6040, ever. Structural. AI-controlled drive arming needs a separately certified canopen-control bridge.

- Single allow-list expansion mechanism. Operators expand the allow-list per-deployment; no runtime grant. Future enhancement could add an audited runtime grant mechanism if a use case warrants it.

- No CAN FD or CAN XL. The bridge speaks classic CAN 2.0A/B. CAN FD (Flexible Data-rate) and CAN XL are not in v1.

- No EDS round-trip. The parser reads EDS for OD metadata; it doesn't write modified EDS files back. EDS editing remains operator tooling.

- One CAN interface per bridge instance. Multi-bus deployments run multiple bridge instances, one per interface, sharing the worker.

- SDO is acyclic. The bridge issues SDO reads on-demand; cyclic SDO at high rates is not supported (use PDOs).

# 15. Regulatory posture

| Aspect | Posture |
| --- | --- |
| Safety ceiling | ADVISORY. Structural — AUTONOMOUS rejected at config load. |
| Default per-binding mode | SHADOW. Every binding must be explicitly promoted to ADVISORY. |
| Forbidden OD indices | 0x6040 (CiA-402 controlword) — hard-coded. Future extension by code change. |
| Allow-list | Mandatory per-node per deployment. Operator's safety-analysis declaration. |
| Interlock authority | Advisory only. Drive-side and safety-PLC-side interlocks remain primary. |
| Operator override | Not applicable to bridge writes. |
| Audit | Local JSONL. No mirror. Bus tracing (PCAN-View, candump, CANalyzer) remains source of truth. |
| MoC | Hot-reload not supported. Allow-list changes follow operator's MoC. |
| IEC 62061 / ISO 13849 | The bridge contributes structural defences those frameworks recognise. It does not by itself satisfy any SIL or PL classification. |

The bridge is appropriate for industrial CANopen deployments where AI advice should flow to operator-configured advisory channels but never directly arm or disarm drives, control torque limits, or otherwise take SIL-classified actions. The two-gate defence makes this guarantee robust against careless future maintenance.

# 16. References

- 13-CANOPEN.md — the bridge specification.

- 00-FRAMEWORK.md — the universal bridge contract.

- docs/canopen-bridge.md — domain-context companion in the repo.

- CAN in Automation (CiA) — https://www.can-cia.org/. Standards body.

- CiA-301 — CANopen application-layer and communication profile.

- CiA-401 — Generic I/O modules device profile.

- CiA-402 — Drives and motion control device profile.

- CiA-418 — Battery modules device profile.

- SocketCAN — https://www.kernel.org/doc/html/latest/networking/can.html. Linux native CAN.

- PEAK PCAN-USB — https://www.peak-system.com/. Cross-platform USB-CAN.

- Kvaser — https://www.kvaser.com/. Cross-platform USB-CAN.

- IEC 62061 — Functional safety of electrical/electronic/programmable safety-related control systems.

- ISO 13849 — Safety of machinery — Safety-related parts of control systems.

- Rakovskyi, D. (2024). Framework for Natural Neuron Network Modeling: The Jneopallium Approach. IJSR 13(7).