Building Models with MAVLink and Jneopallium

User Manual

Drone telemetry · sim-only by default · forbidden-actuator message list

Version: 1.0-SNAPSHOT

License: BSD 3-Clause

Author: Dmytro Rakovskyi

# 1. Introduction

[MAVLink](https://mavlink.io/) (Micro Air Vehicle Link) is the dominant communication protocol for drones, ground-control stations, and autopilot systems. It originated in the open-source UAV community around 2009 and is now used by every major autopilot stack — ArduPilot, PX4, Pixhawk — as the wire protocol between the autopilot, telemetry radio, ground-control station (Mission Planner, QGroundControl), companion computer (Raspberry Pi running ROS or custom code), and any peer aircraft in a swarm.

MAVLink is a binary message protocol over a thin transport layer. UDP is the modern default; serial is still ubiquitous for telemetry radios; TCP is used for some ground-control-station integrations. Messages carry vehicle state (HEARTBEAT, ATTITUDE, GLOBAL_POSITION_INT, BATTERY_STATUS), commands (COMMAND_LONG, COMMAND_INT, SET_MODE), and structured events (STATUSTEXT, EVENT).

The MAVLink bridge connects Jneopallium to MAVLink-speaking systems — autopilots, simulators (Gazebo + SITL, jMAVSim, Microsoft AirSim), and ground stations. It subscribes to telemetry over UDP or TCP, decodes typed messages into Jneopallium signals (ProprioceptiveSignal for own-vehicle pose and battery, PeerObservationSignal for swarm peers, AlarmSignal for status text and events), and publishes neuron-derived advisory recommendations back as MAVLink messages — but only into a simulator by default.

The bridge is structurally sim-only. This is the strictest active-write safety posture in the directory. Two layered defences enforce it:

First, eleven MAVLink message types are explicitly forbidden when simulatorOnly: false. The set in MavlinkBridgeConfig.FORBIDDEN_WRITE_MESSAGE_TYPES covers every actuator-driving message in the protocol: COMMAND_LONG, COMMAND_INT, SET_MODE, MANUAL_CONTROL, RC_CHANNELS_OVERRIDE, plus every MISSION_* write. A write binding targeting any of these against a non-sim deployment fails at config load with a clear message naming the spec sections (§3, §6) that forbid it.

Second, per-tag `AUTONOMOUS` is rejected unless `simulatorOnly: true`. The compact constructor walks perTagSafetyMode and throws if any tag is promoted to AUTONOMOUS while simulatorOnly is false. The error message names the binding and the spec section.

Both gates are independent. There is no environment variable, no forceWrite flag, no flag combination that bypasses both. To write actuator commands to MAVLink at all, you must set simulatorOnly: true. There is no path to writing actuator commands to a physical autopilot in v1. Physical-aircraft control needs a separately certified mavlink-control bridge with its own safety case — a project that involves DO-178C (airborne software certification), MIL-STD-882E (system safety), and operator-side regulatory engagement with the relevant aviation authority. That bridge is not this bridge.

A third layer of structural defence applies to ingress: bindings whose `systemId` does not appear in their connection's `expectedSystems` whitelist are rejected at config load. This protects against silent cross-talk on shared UDP segments (a common operational hazard at fields with multiple aircraft on overlapping radio channels). A typo in systemId is caught at config load with a message naming the connection and the expected list.

This manual explains how to install the dependency, configure the bridge against a SITL simulator or a real airframe (for read-only telemetry), design swarm-aware models with peer observation, validate end to end, and understand the regulatory posture. It assumes a Java engineer who knows Maven, has read the universal bridge framework (00-FRAMEWORK.md), and has at least basic familiarity with MAVLink concepts (system/component ids, message families, the UDP-default heartbeat).

# 2. Six rules you must internalise before writing any code

The universal six from 00-FRAMEWORK.md §0, specialised for MAVLink:

- No actuator-driving write to a physical autopilot, ever. The bridge has no path to write COMMAND_LONG, SET_MODE, MANUAL_CONTROL, or any of the eleven forbidden message types to a non-simulator deployment. The forbidden-message-list check (MavlinkBridgeConfig.FORBIDDEN_WRITE_MESSAGE_TYPES) fails the config load. There is no flag combination that bypasses this.

- Interlocks have advisory authority only. A HarmVetoSignal can drive an advisory STATUSTEXT message into the simulator or into a ground-station's advisory channel; it cannot trigger an emergency stop on a real airframe. Aircraft emergency-stop authority lives on the autopilot itself (its failsafe modes) and on the operator's manual override of RC channels — not on this bridge.

- Operator override does not apply. The bridge writes no regulatory control. OperatorOverrideSignal is ignored.

- Every poll, every write attempt, and every config-load defence trigger produces an audit record. Records cover successful reads, decoder failures, expected-systems mismatches at runtime, simulator-only refusals, forbidden-message-type refusals, and link state changes (heartbeat timeouts, reconnects). Local JSONL only — operator-side telemetry recording (.tlog files in QGroundControl format) remains the operator's responsibility.

- Quality propagates. MAVLink messages carry no explicit quality field, but heartbeat-derived link state is surfaced as Quality.UNCERTAIN on stale messages (no heartbeat received within the configured window) and Quality.BAD when the link is dropped entirely. The mapper does not invent quality from sensor value content.

- Wall-clock timestamps come from the source. MAVLink messages carry time_boot_ms (autopilot uptime) and, in some message types, time_unix_usec (microseconds since epoch). The mapper honours the latter when present and the former when not, anchored against an autopilot-uptime-to-wall-clock offset established on the first received heartbeat. System.currentTimeMillis() is only used when no source timestamp is available.

# 3. Prerequisites

- JDK 17 or later. The bridge uses Java records, java.nio.channels for UDP, and java.net.Socket for TCP.

- Maven 3.9 or later.

- A MAVLink-speaking system reachable from the host. Two paths:

- Simulator — the recommended starting point. ArduPilot SITL (Software In The Loop), PX4 SITL, jMAVSim, Microsoft AirSim, or Gazebo with the appropriate autopilot plugin. All speak MAVLink natively over UDP.

- Real airframe — for read-only telemetry. The bridge can subscribe to UDP messages from a telemetry radio's serial-to-UDP bridge (running on a companion computer) or from a ground-station's MAVLink forwarding. Writing to a real autopilot requires simulatorOnly: false, which structurally rejects the forbidden message types — useful for advisory writes to non-actuator topics only.

- The bridge core compiles without any external MAVLink Java library. The transport seam (MavlinkTransport) is implemented against the JDK's networking APIs; message encode/decode happens in MavlinkSignalMapper against a curated subset of message types. Production deployments wanting full MAVLink 2.0 dialect support can supply an IoMavlinkTransport adapter using io.dronefleet:mavlink or me.drton:jmavlib and the bridge stays unaware.

- For operational deployment near real aircraft: pilot-in-command, airspace authorisation, RC failsafe verification, and aviation-authority engagement per the airframe's class. None of these is a software dependency, but they are non-optional for any deployment within RF range of a real airframe.

# 4. Adding Jneopallium to your project

The MAVLink bridge ships in the same worker artifact as the rest of Jneopallium:

<repositories> <repository> <id>gitlab-jneopallium</id> <url>https://gitlab.com/api/v4/projects/44434192/packages/maven</url> </repository> </repositories> <dependencies> <dependency> <groupId>com.rakovpublic.jneopallium</groupId> <artifactId>worker</artifactId> <version>1.0-SNAPSHOT</version> </dependency> </dependencies>

No external MAVLink library is required. For deployments wanting the full official dialect coverage, add io.dronefleet:mavlink in your deployment module:

<dependency> <groupId>io.dronefleet</groupId> <artifactId>mavlink</artifactId> <version>1.1.11</version> <scope>runtime</scope> </dependency>

The bridge stays unaware of this dependency — your deployment supplies a custom MavlinkTransport adapter.

# 5. Architecture

The bridge introduces twelve classes under worker.bridge.mavlink:

┌──────────────────────────────────┐ UDP / TCP MAVLink messages │ MAVLink-speaking endpoint │ ────────────────────────────────┐ │ │ │ │ SITL simulator (ArduPilot, │ │ │ PX4, jMAVSim, AirSim) │ │ │ │ │ │ - or - │ │ │ │ │ │ Real airframe (READ-ONLY, │ │ │ simulatorOnly=false rejects │ │ │ actuator writes) │ ◀──── advisory writes ──────────│ │ │ (STATUSTEXT, PARAM_SET │ │ HEARTBEAT, ATTITUDE, │ to non-forbidden types) │ │ GLOBAL_POSITION_INT, │ │ │ BATTERY_STATUS, STATUSTEXT, │ │ │ COMMAND_ACK, EVENT │ │ └──────────────────────────────────┘ │ ▼ ┌──────────────────────────────────────┐ │ MavlinkClientService │ │ • per-connection UDP/TCP transport │ │ • heartbeat tracking per system │ │ • per-binding decoders │ │ • expected-system whitelist filter │ │ • bounded advisory queue │ │ • runtime forbidden-type re-check │ └─┬───────┬──────────┬─────────────────┘ │ │ │ ┌─────────────────▼─┐ ┌───▼──────┐ ┌─▼───────────────┐ │ MavlinkTelemetry │ │ Mavlink │ │ MavlinkEvent │ │ Input │ │ SwarmIn │ │ Input │ │ (IInitInput → │ │ (Peer- │ │ (IInitInput → │ │ Proprio / │ │ Observ) │ │ Alarm) │ │ Efficiency) │ │ │ │ │ └───────────────────┘ └──────────┘ └─────────────────┘ ▼ [Standard Jneopallium pipeline] ▼ MavlinkAdvisoryOutputAggregator │ │ AUTONOMOUS writes ONLY if simulatorOnly:true │ Forbidden message types rejected at runtime │ ▼ MavlinkClientService.send() │ ▼ MavlinkAuditOutput (JSONL, local)

Components:

| Class | Role |
| --- | --- |
| MavlinkBridgeConfig | YAML record (immutable). Rejects forbidden message types and AUTONOMOUS at config load when simulatorOnly=false. Validates systemId against expectedSystems whitelist. |
| MavlinkBridgeConfigLoader | Jackson YAML loader. FAIL_ON_UNKNOWN_PROPERTIES = true. |
| MavlinkTransport | Test-seam interface. UDP and TCP transports. connect / send / setReceiveHandler / close. |
| MavlinkConnectionBinding | Resolved connection identity (host, port, transport, expected systems). |
| MavlinkMessageBinding | Resolved per-message binding (connection + system id + message type + signal tag). |
| MavlinkSignalMapper | Pure functions. Decodes MAVLink messages → typed signals. Handles HEARTBEAT, ATTITUDE, GLOBAL_POSITION_INT, BATTERY_STATUS, STATUSTEXT, EVENT, and the peer-observation patterns. |
| MavlinkClientService | Owns transports, per-system heartbeat tracking, bindings, advisory queue, audit channel, runtime forbidden-type re-check. AutoCloseable. |
| MavlinkTelemetryInput | IInitInput. Drains PROPRIOCEPTIVE and EFFICIENCY bindings (own-vehicle state and battery). |
| MavlinkSwarmInput | IInitInput. Drains PEER_OBSERVATION bindings (positions of other systems in the swarm). |
| MavlinkEventInput | IInitInput. Drains ALARM bindings (STATUSTEXT, EVENT messages). |
| MavlinkAdvisoryOutputAggregator | IOutputAggregator. Encodes typed signals → MAVLink messages. Enforces simulatorOnly re-check on every send. |
| MavlinkAuditOutput | Local JSONL append-only audit. |

# 6. The structural defences

The MAVLink bridge has the strictest active-write safety posture in the directory. Three layered defences guard against accidental writes to a real airframe.

Defence 1 — eleven forbidden actuator message types. The set MavlinkBridgeConfig.FORBIDDEN_WRITE_MESSAGE_TYPES contains every MAVLink message that drives an actuator: COMMAND_LONG, COMMAND_INT, SET_MODE, MANUAL_CONTROL, RC_CHANNELS_OVERRIDE, MISSION_ITEM, MISSION_ITEM_INT, MISSION_COUNT, MISSION_CLEAR_ALL, MISSION_SET_CURRENT, MISSION_WRITE_PARTIAL_LIST. When simulatorOnly: false, the compact constructor walks writes: and throws IllegalArgumentException for any binding targeting one of these. The exception message names the binding, the message type, and the spec sections (§3, §6).

Defence 2 — AUTONOMOUS gated by simulatorOnly. Even if a binding doesn't target a forbidden message type, promoting any tag to AUTONOMOUS requires simulatorOnly: true. The compact constructor walks perTagSafetyMode and throws for any AUTONOMOUS entry while simulatorOnly is false. ADVISORY remains usable for non-actuator writes (status text, parameter reads, advisory channels) — the gate is specifically on AUTONOMOUS promotion.

Defence 3 — expected-systems whitelist for cross-talk protection. Every ConnectionConfig has an expectedSystems list. Every binding's systemId must appear in that list, or config load fails. This catches the most common operational hazard at multi-aircraft fields: a typo in systemId that silently consumed cross-talk from a different drone on the same UDP segment. The error message names the connection's expectedSystems list and the binding's mismatching systemId.

The first two defences are about preventing the bridge from controlling a physical airframe. The third is about preventing the bridge from accidentally listening to or acting on the wrong airframe. Together they form the safety contract: writes are sim-only, reads are scoped to expected systems.

These are config-load checks. The runtime adds a fourth: MavlinkClientService.send() re-checks both forbidden message type and simulatorOnly on every call, throwing if either gate would have caught it at load. This is defence in depth — every layer holds even if the others fail.

# 7. Configuration

Configuration is YAML, loaded once at startup through MavlinkBridgeConfigLoader.load(Path). Hot-reload is not supported. The loader sets FAIL_ON_UNKNOWN_PROPERTIES = true.

## 7.1 Schema

The configuration has eight top-level keys.

- connections — list of transport endpoints (UDP or TCP), each with id, host, port, and expectedSystems whitelist.

- simulatorOnly — boolean. Default true (the constructor treats null as true). When false, the structural defences in §6 apply to the loaded writes: and perTagSafetyMode.

- reads — list of telemetry-read bindings.

- events — list of event-message read bindings.

- writes — list of advisory-write bindings.

- audit — local JSONL audit file path.

- perTagSafetyMode — per-binding SHADOW / ADVISORY / AUTONOMOUS (latter only with simulatorOnly: true).

- tickInterval — worker tick cadence. Default PT0.25S.

## 7.2 Minimal example — SITL smoke test

The smallest useful configuration. Connects to a local ArduPilot SITL over UDP, reads telemetry from system 1, no writes, no events.

connections: - id: "sitl-local" transport: UDP host: "127.0.0.1" port: 14550 expectedSystems: [1] simulatorOnly: true reads: - bindingId: "VEHICLE-POSE" connectionId: "sitl-local" systemId: 1 messageType: "ATTITUDE" signalKind: PROPRIOCEPTIVE signalTag: "DRONE.1.ATTITUDE" - bindingId: "VEHICLE-BATT" connectionId: "sitl-local" systemId: 1 messageType: "BATTERY_STATUS" signalKind: EFFICIENCY signalTag: "DRONE.1.BATTERY" events: [] writes: [] audit: localAuditFile: "/var/log/jneopallium/mavlink-audit.jsonl" tickInterval: "PT0.25S"

## 7.3 Production-like example — swarm-aware simulator deployment

A more substantial config: a three-drone swarm SITL setup, telemetry from each, peer observations across the swarm, status-text alarms, and advisory writes (AUTONOMOUS allowed because simulatorOnly: true).

connections: - id: "sitl-multi" transport: UDP host: "127.0.0.1" port: 14550 expectedSystems: [1, 2, 3] simulatorOnly: true reads: - bindingId: "DRONE1-POSE" connectionId: "sitl-multi" systemId: 1 messageType: "ATTITUDE" signalKind: PROPRIOCEPTIVE signalTag: "DRONE.1.ATTITUDE" - bindingId: "DRONE1-POS" connectionId: "sitl-multi" systemId: 1 messageType: "GLOBAL_POSITION_INT" signalKind: PROPRIOCEPTIVE signalTag: "DRONE.1.GLOBALPOS" - bindingId: "DRONE1-BATT" connectionId: "sitl-multi" systemId: 1 messageType: "BATTERY_STATUS" signalKind: EFFICIENCY signalTag: "DRONE.1.BATTERY" - bindingId: "PEER-2-POS" connectionId: "sitl-multi" systemId: 2 messageType: "GLOBAL_POSITION_INT" signalKind: PEER_OBSERVATION signalTag: "PEER.2.GLOBALPOS" - bindingId: "PEER-3-POS" connectionId: "sitl-multi" systemId: 3 messageType: "GLOBAL_POSITION_INT" signalKind: PEER_OBSERVATION signalTag: "PEER.3.GLOBALPOS" events: - bindingId: "DRONE1-STATUS" connectionId: "sitl-multi" systemId: 1 messageType: "STATUSTEXT" signalTag: "DRONE.1.STATUS" severityMap: EMERGENCY: "URGENT" ALERT: "URGENT" CRITICAL: "URGENT" ERROR: "HIGH" WARNING: "HIGH" NOTICE: "LOW" INFO: "JOURNAL" DEBUG: "JOURNAL" writes: - bindingId: "ADVISORY-WAYPOINT" connectionId: "sitl-multi" systemId: 1 messageType: "MISSION_ITEM_INT" signalTag: "DRONE.1.WAYPOINT" - bindingId: "ADVISORY-MODE" connectionId: "sitl-multi" systemId: 1 messageType: "SET_MODE" signalTag: "DRONE.1.MODE" audit: localAuditFile: "/var/log/jneopallium/mavlink-audit.jsonl" perTagSafetyMode: ADVISORY-WAYPOINT: AUTONOMOUS ADVISORY-MODE: ADVISORY tickInterval: "PT0.1S"

## 7.4 Field reference

connections[].id is the bridge's local identifier for the connection. Bindings reference connections by id.

connections[].transport is UDP or TCP. UDP is the universal default in MAVLink ground-station integrations; TCP is used for some bridging scenarios.

connections[].host / port define the remote endpoint. For SITL the default UDP port is 14550 (Mission Planner / QGroundControl convention). Multiple SITL instances usually run on 14550, 14551, 14552.

connections[].expectedSystems is the whitelist of MAVLink system IDs accepted on this connection. Every binding's systemId must appear here. Empty list disables the whitelist (not recommended in any operational deployment).

simulatorOnly defaults to true if absent. Set to false only when the deployment is doing pure read-only telemetry from a real airframe — and even then, the forbidden-message-list check still rejects any actuator write at config load.

reads[].messageType is the MAVLink message type name (e.g. ATTITUDE, GLOBAL_POSITION_INT, BATTERY_STATUS). The mapper's curated subset covers the common telemetry types; extending it to additional types requires extending MavlinkSignalMapper.

reads[].signalKind is PROPRIOCEPTIVE (own-vehicle state) or EFFICIENCY (battery, link quality). For peer-observation entries, PEER_OBSERVATION is also valid.

events[].severityMap maps MAVLink STATUSTEXT.severity enum values to AlarmPriority. The MAVLink severity enum follows syslog convention (EMERGENCY=0 through DEBUG=7); the map translates to Jneopallium's URGENT/HIGH/LOW/JOURNAL.

writes[].messageType is the MAVLink message type the bridge will send. The forbidden-type check rejects bindings targeting any of the eleven actuator types unless simulatorOnly: true.

perTagSafetyMode keys are bindingIds. Legal values: SHADOW, ADVISORY, AUTONOMOUS. AUTONOMOUS is rejected at load unless simulatorOnly: true.

# 8. Wiring the Java side

A minimal driver:

package com.example.uav; import com.rakovpublic.jneuropallium.worker.bridge.common.AbstractBridgeAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkAdvisoryOutputAggregator; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkBridgeConfig; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkBridgeConfigLoader; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkClientService; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkEventInput; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkSignalMapper; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkSwarmInput; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkTelemetryInput; import com.rakovpublic.jneuropallium.worker.bridge.mavlink.MavlinkTransport; import java.nio.file.Path; public final class MavlinkBootstrap { public static void main(String[] args) throws Exception { // 1. Load YAML. Forbidden-type and AUTONOMOUS rejections happen here. MavlinkBridgeConfig cfg = MavlinkBridgeConfigLoader.load( Path.of("mavlink-bridge.yml")); // 2. Open the audit sink. try (AbstractBridgeAuditOutput audit = new MavlinkAuditOutput( Path.of(cfg.audit().localAuditFile()))) { // 3. Build the transport. The default JDK-based transport handles // UDP and TCP. Production deployments wanting full MAVLink 2.0 // dialect support supply an adapter that delegates to // io.dronefleet:mavlink or similar. try (MavlinkTransport transport = new DefaultMavlinkTransport(cfg)) { // 4. Build the signal mapper and the client service. MavlinkSignalMapper mapper = new MavlinkSignalMapper(cfg); MavlinkClientService svc = new MavlinkClientService( cfg, transport, mapper, audit); // 5. Start: open all connections, register decoders. // Heartbeat tracking starts immediately; first heartbeat // received on each system establishes the wall-clock offset. svc.start(); try { // 6. Build the inputs. Three by signal family. MavlinkTelemetryInput tlm = new MavlinkTelemetryInput( "mavlink-tlm", svc); MavlinkSwarmInput swarm = new MavlinkSwarmInput( "mavlink-swarm", svc); MavlinkEventInput events = new MavlinkEventInput( "mavlink-events", svc); // 7. Build the aggregator. Runtime forbidden-type and // simulatorOnly re-checks fire inside send(). MavlinkAdvisoryOutputAggregator agg = new MavlinkAdvisoryOutputAggregator(cfg, svc, audit); // 8. Register with the worker. worker.registerInput(tlm); worker.registerInput(swarm); worker.registerInput(events); worker.registerOutputAggregator(agg); worker.run(); } finally { svc.close(); } } } } }

Three implementation notes worth highlighting. Three inputs by signal family. Telemetry (PROPRIOCEPTIVE + EFFICIENCY), swarm (PEER_OBSERVATION), events (ALARM from STATUSTEXT/EVENT messages). The split mirrors how swarm-aware AI naturally categorises data: own-state, peer-state, world-events. Heartbeat tracking is automatic. The first HEARTBEAT message from each system establishes that system's time_boot_ms-to-wall-clock offset and registers the system as live. Subsequent reads carry quality information derived from heartbeat freshness. The runtime defence fires inside `send()`. Under correct config it never fires — defences 1 and 2 catch issues at load. Under any code path that bypasses the config check (programmatic construction, serialization round-trips, future refactors), the runtime gate still throws. Defence in depth.

# 9. Modeling guidance

## 9.1 What to read

For a typical swarm-aware deployment:

- Own-vehicle attitude from ATTITUDE — pitch, roll, yaw. One binding per vehicle.

- Own-vehicle global position from GLOBAL_POSITION_INT — lat/lon/alt. One binding per vehicle.

- Own-vehicle battery from BATTERY_STATUS — voltage, current, remaining. One binding per vehicle.

- Peer positions from GLOBAL_POSITION_INT with peer systemId and signalKind: PEER_OBSERVATION. One binding per peer.

- Status text from STATUSTEXT — autopilot messages with severity levels. One event binding per vehicle.

For solo-deployment, drop the peer bindings. For mission-critical monitoring, add more bindings: SYSTEM_STATUS for component health, GPS_RAW_INT for satellite count, RC_CHANNELS for stick positions, etc.

## 9.2 The messageType → signal class mapping

The mapper handles the curated subset:

| `messageType` | Signal class produced | Signal kind |
| --- | --- | --- |
| HEARTBEAT | (internal — establishes link state, no signal) | — |
| ATTITUDE | ProprioceptiveSignal | PROPRIOCEPTIVE |
| GLOBAL_POSITION_INT | ProprioceptiveSignal or PeerObservationSignal (depending on signalKind) | PROPRIOCEPTIVE / PEER_OBSERVATION |
| LOCAL_POSITION_NED | ProprioceptiveSignal | PROPRIOCEPTIVE |
| BATTERY_STATUS | EnergySignal | EFFICIENCY |
| RADIO_STATUS | EnergySignal (link efficiency proxy) | EFFICIENCY |
| STATUSTEXT | AlarmSignal | ALARM |
| EVENT | AlarmSignal | ALARM |

For message types not in this table, extend MavlinkSignalMapper in your deployment module.

## 9.3 Swarm-aware modeling

The PEER_OBSERVATION signal kind is the key abstraction for swarm-aware AI. A binding with signalKind: PEER_OBSERVATION produces a PeerObservationSignal carrying the peer's systemId, position, velocity (if available), and a timestamp. Downstream neurons reasoning about peer state read these signals just like they'd read sensor signals — except the data is about another aircraft.

Three operational notes for peer observation:

- Heartbeat staleness applies independently per peer. If peer 2 stops heartbeating, signals from binding PEER-2-POS become Quality.UNCERTAIN after the configured staleness window and Quality.BAD after link timeout. The downstream model sees the degradation.

- The whitelist applies to peers too. Peer systemIds must appear in expectedSystems. Adding a peer means updating both the connection's whitelist and adding a binding.

- Peer observations are still on the same UDP segment. The bridge doesn't initiate connections to peers; it subscribes to the same broadcast traffic. In typical SITL multi-instance setups, all vehicles publish on the same port; the system-id filter does the routing.

## 9.4 Promoting an advisory binding

For non-actuator writes (advisory STATUSTEXT messages, PARAM_REQUEST_READ for telemetry expansion):

- YAML: add a writes: entry with the non-forbidden message type. Start in SHADOW.

- Observe SHADOW for an observation period. Inspect audit JSONL.

- Promote to ADVISORY. Restart. Messages flow into the simulator.

- AUTONOMOUS requires `simulatorOnly: true`. And even then, only for non-forbidden message types when simulatorOnly: false — but in simulatorOnly: true, all message types including the eleven forbidden are unlocked because nothing actuates a real aircraft.

The forbidden-type rule is specifically about preventing actuator writes to a real airframe. In simulator mode, those same writes go to a simulator process that can be killed or paused without consequence — so they're unlocked.

# 10. The advisory egress algorithm

When the worker calls MavlinkAdvisoryOutputAggregator.save(), for each signal:

- Resolve the write binding by signalTag. UNBOUND → audit REJECTED reason=UNBOUND_SIGNAL_TYPE. No send.

- Resolve per-tag safety mode. SHADOW → audit REJECTED reason=SHADOW_MODE. No send.

- Runtime defence 1 — check the binding's messageType against FORBIDDEN_WRITE_MESSAGE_TYPES. If forbidden AND simulatorOnly: false → audit REJECTED reason=FORBIDDEN_TYPE_RUNTIME. No send. Under correct config this never fires.

- Runtime defence 2 — check the per-tag mode. If AUTONOMOUS AND simulatorOnly: false → audit REJECTED reason=AUTONOMOUS_BLOCKED. No send. Under correct config this never fires.

- Encode the signal as a MAVLink message of the binding's messageType via MavlinkSignalMapper.encode().

- Call MavlinkClientService.send(connectionId, message). The transport layer sends via UDP or TCP.

- On success → audit APPLIED reason=MESSAGE_SENT. On transport failure → audit FAILED reason=SEND_ERROR:<detail>.

There is no clamp, no rate-limit, no interlock fail-safe — MAVLink messages are discrete commands, not continuous values. The simpler algorithm reflects the discrete-message nature of the protocol.

# 11. Audit and transparency

The bridge writes one JSONL record per decision to the local audit:

{ "ts": 1740000000000, "run": 12345, "bridge": "mavlink", "verdict": "APPLIED", "tag": "DRONE.1.WAYPOINT", "connectionId": "sitl-multi", "systemId": 1, "messageType": "MISSION_ITEM_INT", "reason": "MESSAGE_SENT", "safetyMode": "AUTONOMOUS", "simulatorOnly": true }

Records you will see:

| `verdict` | `reason` | Triggered by |
| --- | --- | --- |
| APPLIED | MESSAGE_SENT | Outbound MAVLink message sent. |
| APPLIED | MESSAGE_DECODED | Inbound message decoded into a signal. |
| APPLIED | HEARTBEAT_RECEIVED | Heartbeat from expected system; link state updated. |
| APPLIED | BRIDGE_RECONNECTED | Transport recovered after a drop. |
| REJECTED | SHADOW_MODE | Binding in SHADOW; no send. |
| REJECTED | UNBOUND_SIGNAL_TYPE | Tag has no writes: entry. |
| REJECTED | FORBIDDEN_TYPE_RUNTIME | Runtime check (defence 1) caught a forbidden type with simulatorOnly=false. |
| REJECTED | AUTONOMOUS_BLOCKED | Runtime check (defence 2) caught AUTONOMOUS with simulatorOnly=false. |
| REJECTED | UNEXPECTED_SYSTEM | Inbound message from a systemId not in expectedSystems. |
| FAILED | SEND_ERROR:<detail> | Transport error during send. |
| FAILED | DECODE_ERROR:<detail> | Inbound message failed to decode. |
| FAILED | LINK_TIMEOUT:<system> | No heartbeat from system for the timeout window. |

The audit file is local. The operator's existing telemetry recording (QGroundControl .tlog files, custom flight-data recorders) remains the source of truth for the MAVLink traffic itself.

# 12. Operating procedures

## 12.1 Smoke test with ArduPilot SITL

The fastest end-to-end check:

- Install ArduPilot SITL per the [ArduPilot docs](https://ardupilot.org/dev/docs/sitl-setup-landingpage.html).

- Start SITL with sim_vehicle.py -v ArduCopter --console. By default it listens on UDP 14550.

- Save §7.2 minimal config to mavlink-bridge.yml.

- Run MavlinkBootstrap (§8).

- Within the first second, the bridge starts receiving HEARTBEAT messages. MavlinkTelemetryInput.readSignals() returns ProprioceptiveSignals for attitude.

- Inspect the audit JSONL: HEARTBEAT_RECEIVED, MESSAGE_DECODED records appear.

## 12.2 Verifying the structural defences

A useful test sequence:

- Forbidden type at config load. Set simulatorOnly: false and add a writes: binding for COMMAND_LONG. Load. Assert IllegalArgumentException with the message naming the binding and §3, §6.

- AUTONOMOUS at config load. Set simulatorOnly: false and add perTagSafetyMode: { BINDING: AUTONOMOUS }. Load. Assert IllegalArgumentException.

- Expected-systems mismatch. Set connections[0].expectedSystems: [1, 2] and add a binding with systemId: 99. Load. Assert IllegalArgumentException with the message naming the connection and the expected list.

- Runtime forbidden type. Programmatically construct a config that bypasses the compact constructor (test-only). Call svc.send() with a forbidden type. Assert it throws.

Four tests, three structural defences plus the runtime fallback. All four should pass on every release.

## 12.3 Verifying expected-systems cross-talk rejection

Operational tip: at multi-aircraft fields, several drones share UDP segments. To verify the bridge correctly ignores cross-talk:

- Configure expectedSystems: [1].

- Run SITL with two vehicles (system IDs 1 and 2) on the same port.

- Inspect the audit JSONL. Messages from system 2 produce REJECTED reason=UNEXPECTED_SYSTEM records and don't appear in any input.

If you see signals tagged with system 2's data, the whitelist isn't working — debug before any operational deployment.

## 12.4 Verifying heartbeat-based quality

- Run the smoke test (§12.1).

- Confirm MavlinkTelemetryInput.readSignals() returns signals with Quality.GOOD.

- Kill the SITL process. Watch for LINK_TIMEOUT:1 audit record after the configured staleness window.

- Subsequent reads return signals with Quality.UNCERTAIN, then Quality.BAD.

- Restart SITL. First heartbeat triggers BRIDGE_RECONNECTED; quality returns to GOOD.

The quality propagation is what lets downstream models react to link loss without writing flight-aware code themselves.

## 12.5 Promoting from sim to real airframe (read-only)

To use the bridge for read-only telemetry from a real airframe:

- Confirm the deployment is read-only. No writes: entries, or all writes: target non-forbidden message types.

- Set `simulatorOnly: false`. This activates the structural defences against actuator writes.

- Wire a telemetry-only path to the bridge. Typically: airframe → telemetry radio → ground-station → MAVLink forwarding to UDP port → bridge.

- Run and monitor. All reads work normally. Any actuator write attempted falls through to the runtime defence and audits as REJECTED.

The bridge is appropriate for read-only AI monitoring of real airframes — the structural defences guarantee no actuator command can reach the autopilot. For closed-loop AI control of real airframes, the answer remains: separately certified mavlink-control bridge with its own safety case.

# 13. Cybersecurity and safety posture

MAVLink deployments span a wide range of safety contexts — from desktop simulators to civil-aviation airframes. The bridge's posture:

- Sim-only by default. Every actuator write requires simulatorOnly: true. The bridge cannot accidentally command a real airframe.

- No external network egress from the bridge. The bridge connects only to configured endpoints. No telemetry to cloud, no auto-update, no remote support.

- Audit-only for real airframe deployments. When deployed read-only on a real airframe, the bridge contributes signals to AI reasoning but cannot affect the airframe directly. AI decisions flow through other bridges (advisory file, ground-station alerts, separate certified channels).

- No replacement for autopilot failsafe modes. The autopilot's own failsafe modes (RTL on low battery, RTL on link loss, geofence enforcement) remain primary. AI is supplemental.

- No replacement for pilot-in-command. Real-airframe operation always has a pilot-in-command per aviation regulation. The bridge does not affect this responsibility.

For deployments in civil airspace under EASA, FAA, or equivalent regulators: the airframe's certification and the operator's authorisation define what's allowed. The bridge contributes structural defences that those regulators care about (no actuator write authority), but it does not by itself constitute certification of any kind.

# 14. Limitations and future work

- No write to real airframes, ever. Structural. AI-assisted airframe control needs a separately certified mavlink-control bridge.

- Curated message-type subset. The mapper handles common telemetry types but not the full MAVLink 2.0 dialect. Extensions are straightforward via mapper extension; the framework is in place.

- No serial transport. UDP and TCP only. Serial (the historical telemetry-radio default) is left to the operator's serial-to-UDP bridge running on the companion computer.

- No signing or authentication. MAVLink 2.0 supports message signing; the bridge does not currently verify signatures or sign outbound messages. For deployments needing this, extend MavlinkSignalMapper's encode path.

- One advisory queue per binding. No per-system rate limiting. For high-rate advisory writes in dense swarms, consider per-tick batching at the neuron level.

# 15. Regulatory posture

| Aspect | Posture |
| --- | --- |
| Safety ceiling | `SIM-ONLY` by default. Forbidden-type list (eleven actuator message types) and AUTONOMOUS gate, both keyed on simulatorOnly. |
| Default per-binding mode | SHADOW. AUTONOMOUS requires simulatorOnly: true for any binding. |
| Forbidden message types | COMMAND_LONG, COMMAND_INT, SET_MODE, MANUAL_CONTROL, RC_CHANNELS_OVERRIDE, plus every MISSION_* write. Eleven types total. |
| Expected-systems whitelist | Mandatory at config load. Cross-talk rejected at runtime. |
| Interlock authority | Advisory only. Autopilot failsafe modes remain primary. |
| Operator override | Not applicable to bridge writes; remains the pilot-in-command's authority for the airframe itself. |
| Audit | Local JSONL. No mirror. Operator's telemetry recording (.tlog) remains source of truth for MAVLink traffic. |
| MoC | Hot-reload not supported. YAML changes follow operator's MoC. |
| Aviation regulatory | The bridge does not certify any deployment. Airworthiness, airspace authorisation, and pilot-in-command requirements are operator and regulatory matters. |

The bridge is appropriate for: simulator-based development and validation of swarm AI, read-only AI monitoring of real airframes, ground-station-side AI advisory pipelines, and CI/CD pipelines validating AI flight policies against SITL. The bridge is not appropriate for: closed-loop AI control of physical airframes, autonomous mission execution without human-in-command, or any deployment that would require certification of AI authority over flight controls.

# 16. References

- 12-MAVLINK.md — the bridge specification.

- 00-FRAMEWORK.md — the universal bridge contract.

- docs/mavlink-bridge.md — domain-context companion in the repo.

- MAVLink — https://mavlink.io/. Protocol specification.

- ArduPilot — https://ardupilot.org/. Open-source autopilot.

- PX4 — https://px4.io/. Open-source autopilot.

- QGroundControl — http://qgroundcontrol.com/. Cross-platform ground-control station.

- Mission Planner — https://ardupilot.org/planner/. ArduPilot's primary ground-control station.

- jMAVLib — https://github.com/DrTon/jMAVlib. Java MAVLink library.

- io.dronefleet:mavlink — https://github.com/dronefleet/mavlink. Modern Java MAVLink library.

- DO-178C — https://www.rtca.org/. Software Considerations in Airborne Systems and Equipment Certification.

- MIL-STD-882E — https://en.wikipedia.org/wiki/MIL-STD-882. System safety standard.

- Rakovskyi, D. (2024). Framework for Natural Neuron Network Modeling: The Jneopallium Approach. IJSR 13(7).