Building Models with FMI/FMU and Jneopallium

User Manual

Simulation-driven testing · digital twins · closed-loop CI

Version: 1.0-SNAPSHOT

License: BSD 3-Clause

Author: Dmytro Rakovskyi

# 1. Introduction

[FMI](https://fmi-standard.org/) (Functional Mock-up Interface) is a free standard for exchanging dynamic simulation models. An FMU (Functional Mock-up Unit) is a ZIP container with an XML model description, native C binaries for the supported platforms, and optionally model source. FMI is supported by 280+ tools — Modelica, Simulink, Dymola, OpenModelica, Amesim, GT-SUITE, JModelica, FMPy — which means any of them can compile a plant model that this bridge will drive.

For Jneopallium this is the most important bridge in the directory. Every other bridge spec — OPC UA, PLC4X, MQTT, ROS 2, all of them — references the FMI bridge in its acceptance scenarios. It is the testbench. It lets you prove the entire pipeline (measurements → safety gate → harm gate → planning → setpoints) against a physically realistic simulator before any wire is connected to a real PLC or robot. The bridge runs the FMU in the same JVM (or at least the same machine) as the worker; there is no network, no reconnect, no source-system quality coming back from a protocol.

The bridge is unusual in three ways. First, the "external system" is co-located, so the framework's universal six rules apply differently — there are no protocol reconnects, no quality codes, no operator-override-vs-interlock distinction across a network. Second, the bridge has a clock — AS_FAST_AS_POSSIBLE for CI determinism, REAL_TIME for hardware-in-the-loop (HIL) demos. Third, it is the only bridge whose write path goes into a simulator, so the safety ceiling is AUTONOMOUS by default — there is no real actuator to protect, and the whole point is closed-loop testing.

This manual explains how to install the dependency, parse an FMU's modelDescription.xml, configure the bridge, design a closed-loop test, run it as part of mvn verify, and interpret the audit log. It assumes a Java engineer who knows Maven, has read the universal bridge framework (00-FRAMEWORK.md), and has basic familiarity with FMI co-simulation (the instantiate → setupExperiment → enterInit → exitInit → doStep → terminate lifecycle).

# 2. Six rules — restated for a simulation context

The universal six from 00-FRAMEWORK.md §0 apply with three notable specialisations for FMI:

- No raw write to a field actuator from neuron output. The pipeline PlanningNeuron → SafetyGateNeuron → SafetyMode check → OperatorOverrideSignal check → FmuCommandOutputAggregator still applies. The aggregator extends AbstractBridgeOutputAggregator so every clamp / rate-limit / interlock / override defence is in place. The difference: when an "actuator write" happens, it modifies an FMU input variable. No human can get hurt; the only thing at stake is the test's signal-to-noise ratio.

- Interlocks have direct authority. An InterlockSignal with tripped=true writes the binding's failSafeValue regardless of mode — useful for testing your interlock neurons themselves. The simulator should expose an "interlock trip" output that wires into a downstream InterlockNeuron; that completes the loop.

- Operator override wins for regulatory control. Useful for simulating operator interventions during long-running scenario sweeps. The OverrideRegistry is exposed via aggregator.overrideRegistry() for test code that wants to inject overrides at known wall-clock instants.

- Every write produces an audit record. The audit JSONL is the test report. A run that produces no verdict=APPLIED records during a 60-second simulation means the safety ladder rejected every command — exactly what a SHADOW-mode acceptance test wants to see.

- Quality propagates — but the FMU always returns `Quality.GOOD`. The simulator is deterministic; there are no sensor errors unless you model them. To test quality propagation, configure the FMU itself to emit a "quality output" boolean and bind it as a second MeasurementSignal whose quality your downstream code derives.

- Wall-clock timestamps come from the source. In FMI the "source" is the simulator's wall-clock at the moment of doStep. The bridge's FmuClientService uses System.currentTimeMillis() for the actual timestampMs on emitted signals — appropriate because the FMU's internal time is simulation time (seconds since clock.startTime), not real time.

# 3. Prerequisites

- JDK 17 or later. The integration uses Java records and the FMI 2.0 / 3.0 C API via JNA bindings.

- Maven 3.9 or later.

- An FMU file. For your first run, download an open-source test FMU from the [FMI Standard reference list](https://fmi-standard.org/tools/) or the OpenModelica examples — the [VanDerPol](https://github.com/modelica/Reference-FMUs) reference FMU is a good starter.

- For the bundled test FMU bootstrap: nothing. The repo ships StubFmuDriver — a pure-Java tank-temperature model that emulates the FMI co-simulation contract without ever touching a native binary, the file system, or an FMU ZIP. Every integration test in FmuBridgeIntegrationTest runs against it.

- For real FMUs: a JNA-based FMI adapter you supply, or javafmi if available in your registry. The bridge core compiles without any of those — all FMI calls go through the FmuDriver seam.

# 4. Adding Jneopallium to your project

The FMI bridge ships in the same worker artifact as the rest of Jneopallium:

<repositories> <repository> <id>gitlab-jneopallium</id> <url>https://gitlab.com/api/v4/projects/44434192/packages/maven</url> </repository> </repositories> <dependencies> <dependency> <groupId>com.rakovpublic.jneopallium</groupId> <artifactId>worker</artifactId> <version>1.0-SNAPSHOT</version> </dependency> </dependencies>

For a real FMU adapter, add JNA in your test or runtime classpath:

<dependency> <groupId>net.java.dev.jna</groupId> <artifactId>jna</artifactId> <version>5.14.0</version> <scope>test</scope> </dependency>

javafmi is an option too, but its release cadence is inconsistent — verify before merging.

# 5. Architecture

The bridge introduces ten boundary classes under worker.bridge.fmi:

┌──────────────────────────────────┐ │ FmuClientService │ │ • owns the FMU lifecycle │ │ • realCache / boolCache / int │ │ • pending writes (buffered to │ │ next doStep) │ │ • clock: REAL_TIME or AFAP │ └──────┬───────────────────┬───────┘ │ │ ┌──────────▼─┐ ┌───────▼─────────┐ │ FmuMeasure │ │ FmuEventInput │ │ mentInput │ │ (IInitInput → │ │ (IInitInput│ │ AlarmSignal) │ │ → Measu- │ │ │ │ rement- │ │ │ │ Signal) │ │ │ └────────────┘ └─────────────────┘ ▼ [Standard Jneopallium pipeline — your model under test] ▼ FmuCommandOutputAggregator (wraps AbstractBridgeOutputAggregator) │ │ inner.save() applies §2.2 algorithm │ outer.save() calls svc.step() afterwards ▼ FmuClientService.setReal() / .setBoolean() ▼ FmuClientService.step() → driver.doStep(t, h) │ ▼ FmuAuditOutput (JSONL)

Components:

| Class | Role |
| --- | --- |
| FmiBridgeConfig | YAML record (immutable). Validates tolerance >= 0 and stepSize > 0 at construction. |
| FmiBridgeConfigLoader | Jackson YAML loader. FAIL_ON_UNKNOWN_PROPERTIES = true. |
| FmuDriver | Test-seam interface: instantiate / setupExperiment / enterInit / exitInit / getReal / setReal / getBoolean / setBoolean / doStep / terminate / close. |
| FmuModelDescription | Parses modelDescription.xml from the FMU ZIP. Surfaces variable names, value references, types, and the FMI version. |
| FmuVariableBinding | Resolved binding identity (bindingId, fmuVariable, signalTag, valueReference, failSafeValue, rampRate, clamps). Implements BridgeBinding. |
| FmuSignalMapper | Pure functions. Real → MeasurementSignal(Quality.GOOD); Boolean true → AlarmSignal with configured severity. |
| FmuClientService | Owns the FMU lifecycle, the latest-value cache, the pending-writes buffer, and the clock. AutoCloseable — calls terminate() then close() on the driver. |
| FmuMeasurementInput | IInitInput that emits one MeasurementSignal per reads: binding from the cache. |
| FmuEventInput | IInitInput that polls Boolean event bindings; emits one AlarmSignal per true flag. |
| FmuCommandOutputAggregator | IOutputAggregator that wraps an inner AbstractBridgeOutputAggregator (decorator pattern). The inner aggregator applies the safety algorithm; the outer adds svc.step() after every tick. |
| FmuAuditOutput | Local JSONL append-only audit. |

Two design choices worth highlighting. The decorator pattern on the aggregator. The universal AbstractBridgeOutputAggregator.save() is final — by contract, that algorithm cannot be subclassed mid-flow. But the FMI bridge needs to call svc.step() after every tick, including ticks where no results came in (otherwise simulation time freezes). So FmuCommandOutputAggregator wraps an inner anonymous subclass that does the safety work, then itself implements IOutputAggregator.save() to call the inner and then step(). Writes are buffered. svc.setReal() accumulates pending writes into a HashMap; the next svc.step() flushes them all to the driver, then calls doStep, then refreshes the cache. This means a single tick can write to multiple FMU inputs atomically — the FMU sees the simultaneous input vector that the safety pipeline computed.

# 6. The FMU model description

Every FMU contains a modelDescription.xml at its root, defining the variables the simulator exposes. Before configuring the bridge, you must know:

- the model name (used to instantiate the FMU);

- the variable names for every input/output you intend to bind;

- the value references (integers that the FMI C API uses internally) — though the bridge resolves these via FmuModelDescription, you don't have to type them into YAML;

- the FMI version — 2.0 (the dominant in-production version) or 3.0 (newer, fewer tools support it).

The minimum useful subset of a modelDescription.xml:

<?xml version="1.0" encoding="UTF-8"?> <fmiModelDescription fmiVersion="2.0" modelName="TankTemperature"> <CoSimulation modelIdentifier="TankTemperature" canHandleVariableCommunicationStepSize="true"/> <ModelVariables> <ScalarVariable name="tank.T" valueReference="1" causality="output" variability="continuous"> <Real start="20.0" unit="degC"/> </ScalarVariable> <ScalarVariable name="heater.Q" valueReference="2" causality="input" variability="continuous"> <Real start="0.0" unit="W"/> </ScalarVariable> <ScalarVariable name="alarm.over_temperature" valueReference="3" causality="output" variability="discrete"> <Boolean start="false"/> </ScalarVariable> </ModelVariables> </fmiModelDescription>

That schema defines a tank with one continuous output (tank.T), one continuous input (heater.Q), and one Boolean alarm output (alarm.over_temperature). The bridge's FmuModelDescription parser surfaces these as a map; you reference them in YAML by name, not by value reference.

# 7. Configuration

Configuration is YAML, loaded once at startup through FmiBridgeConfigLoader.load(Path). Hot-reload is not supported. The loader sets FAIL_ON_UNKNOWN_PROPERTIES = true.

## 7.1 Schema

The configuration has seven top-level sections.

- fmu — FMU file path, logging flag, tolerance.

- clock — clock mode (REAL_TIME or AS_FAST_AS_POSSIBLE), start time, step size in seconds.

- reads — list of fmuVariable → MeasurementSignal bindings.

- writes — list of signalTag → fmuVariable write bindings with the full safety ladder (clamp, ramp, fail-safe).

- events — list of Boolean fmuVariable → AlarmSignal bindings.

- audit — local JSONL audit file path and writeRejectedToAudit toggle.

- perTagSafetyMode — per-binding-id SHADOW / ADVISORY / AUTONOMOUS.

## 7.2 Minimal example — closed-loop tank temperature test

The smallest useful configuration for CI. One FMU input, one output, no events, AS_FAST_AS_POSSIBLE clock. Suitable as a closed-loop integration test.

fmu: path: "./testdata/tank_temperature.fmu" loggingOn: false toleranceDefined: true tolerance: 1.0e-6 clock: mode: AS_FAST_AS_POSSIBLE startTime: 0.0 stepSize: 0.25 # seconds per simulated tick reads: - bindingId: "TANK-TEMP" fmuVariable: "tank.T" signalTag: "PLANT.TANK01.TEMP" writes: - bindingId: "HEATER-Q" fmuVariable: "heater.Q" signalTag: "PLANT.HEATER01.SP" failSafeValue: 0.0 minClampValue: 0.0 maxClampValue: 50000.0 # Watts events: [] audit: localAuditFile: "./out/fmu-audit.jsonl" writeRejectedToAudit: true perTagSafetyMode: TANK-TEMP: AUTONOMOUS # safe — the "field" is a simulator

## 7.3 Production-like example — REAL_TIME demo with interlock

A configuration shaped like an HIL demonstration: real-time clock, both continuous and Boolean outputs (output for telemetry, alarm for the interlock), one writable heater input. Useful for hands-on demos where an operator can interact with a running closed-loop system.

fmu: path: "/opt/jneopallium/fmus/tank_temperature.fmu" loggingOn: false toleranceDefined: true tolerance: 1.0e-6 clock: mode: REAL_TIME startTime: 0.0 stepSize: 0.25 # 250 ms per tick reads: - bindingId: "TANK-TEMP" fmuVariable: "tank.T" signalTag: "PLANT.TANK01.TEMP" writes: - bindingId: "HEATER-Q" fmuVariable: "heater.Q" signalTag: "PLANT.HEATER01.SP" failSafeValue: 0.0 rampRateMaxPerSec: 5000.0 # 5 kW per second max ramp minClampValue: 0.0 maxClampValue: 50000.0 events: - bindingId: "OVERTEMP" fmuVariable: "alarm.over_temperature" signalTag: "PLANT.TANK01.OVERTEMP" severity: URGENT audit: localAuditFile: "/var/log/jneopallium/fmu-audit.jsonl" writeRejectedToAudit: true perTagSafetyMode: TANK-TEMP: AUTONOMOUS HEATER-Q: AUTONOMOUS

## 7.4 Field reference

fmu.path is the file-system path to the FMU ZIP. The bridge does not download FMUs over HTTP; if you need that, the bootstrap can fetch first.

fmu.tolerance is the integrator tolerance the FMU should use, when toleranceDefined: true. Default 0 with toleranceDefined: false lets the FMU use its internal default. For most engineering FMUs 1.0e-6 is a good starting point — tight enough to be reproducible across runs, loose enough that the simulator doesn't grind.

clock.mode is REAL_TIME or AS_FAST_AS_POSSIBLE. The choice is critical and the same model will report wildly different stability properties under the two clocks. CI tests should use `AS_FAST_AS_POSSIBLE` — deterministic, reproducible byte-identical across runs (per acceptance scenario S12 in 03-FMI-FMU.md). HIL demos use REAL_TIME.

clock.startTime is the simulation's initial time in seconds. Almost always 0.0. Non-zero is useful for replaying a scenario from a checkpoint.

clock.stepSize is the seconds the simulation advances on every doStep. The worker's tick rate must match: if the worker ticks at 4 Hz, set stepSize: 0.25. A stepSize smaller than the worker tick produces an under-stepped simulation (the FMU drifts); larger produces an over-stepped one (the controller's view of the world lags).

reads[].fmuVariable is the FMU variable name as declared in modelDescription.xml. The bridge resolves the value reference at startup via FmuModelDescription.resolve(name); a typo'd name fails fast.

writes[].failSafeValue is what the aggregator writes when an InterlockSignal for the same loopId trips. For a heater, 0.0 Watts. For a flow valve on a circulating heat exchanger, this might be a trickle flow that keeps the heat exchanger from boiling — not zero. The plant engineer owns this number even when the plant is simulated.

writes[].rampRateMaxPerSec is the maximum change per second the aggregator permits. A larger requested step is rate-limited and audited with reason=RATE_LIMITED. null disables rate-limiting.

writes[].minClampValue / maxClampValue are hard clamps. Values outside the range are moved to the nearest bound and audited with reason=CLAMPED_LOW or reason=CLAMPED_HIGH.

events[].severity is one of JOURNAL, LOW, HIGH, URGENT. Default HIGH. The AlarmSignal's priority is set from this; a downstream IndustrialAlarmNeuron can route on it.

perTagSafetyMode keys are bindingIds. The legal values are SHADOW, ADVISORY, and AUTONOMOUS. The FMI bridge does not impose a structural ceiling — AUTONOMOUS is allowed by default because the "field" is a simulator. Use SHADOW for tests that should observe but not act.

# 8. Wiring the Java side

The bridge does not ship a main class. A minimal driver looks like this:

package com.example.plant; import com.rakovpublic.jneuropallium.worker.bridge.common.AbstractBridgeAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmiBridgeConfig; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmiBridgeConfigLoader; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmuAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmuClientService; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmuCommandOutputAggregator; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmuDriver; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmuEventInput; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmuMeasurementInput; import com.rakovpublic.jneuropallium.worker.bridge.fmi.FmuModelDescription; import java.nio.file.Path; public final class FmuBootstrap { public static void main(String[] args) throws Exception { // 1. Load YAML. FmiBridgeConfig cfg = FmiBridgeConfigLoader.load(Path.of("fmu-bridge.yml")); // 2. Parse the FMU's modelDescription.xml. FmuModelDescription desc = FmuModelDescription.parse(Path.of(cfg.fmu().path())); // 3. Construct the driver. In production this is your JNA adapter // over the FMI C API. For tests / demos the repo ships // StubFmuDriver in src/test/java. FmuDriver driver = new YourFmuDriver(Path.of(cfg.fmu().path()), desc); // 4. Open the audit sink. try (AbstractBridgeAuditOutput audit = new FmuAuditOutput( Path.of(cfg.audit().localAuditFile()))) { // 5. Construct the client service. Initialize runs the FMI // init sequence and does the first read from every variable. try (FmuClientService svc = new FmuClientService(driver, desc, cfg)) { svc.initialize(); // 6. Build inputs and aggregator. FmuMeasurementInput measIn = new FmuMeasurementInput( "fmu-meas", svc, cfg.reads()); FmuEventInput eventIn = new FmuEventInput( "fmu-events", svc, cfg.events()); try (FmuCommandOutputAggregator agg = new FmuCommandOutputAggregator(svc, cfg, audit)) { // 7. Register with worker. worker.registerInput(measIn); worker.registerInput(eventIn); worker.registerOutputAggregator(agg); worker.run(); } } } } }

Three implementation notes worth highlighting. `svc.initialize()` runs the FMI init sequence (instantiate → setupExperiment → enterInitializationMode → exitInitializationMode) and does a first read from every configured variable. After that, the cache is populated and measIn.readSignals() returns useful signals on the very first tick. The aggregator is `AutoCloseable` even though FmuCommandOutputAggregator.close() is a no-op — the contract is uniform across all bridges. The try block ensures the FMU is terminated cleanly when the worker exits. Order matters: svc.close() is what calls driver.terminate() then driver.close(). Failure to call svc.close() leaks the FMU instance, and on some FMI implementations leaves a lock file in the FMU extraction directory.

# 9. Modeling guidance

## 9.1 Choosing what to bind

For a closed-loop CI test you typically want:

- One read binding per controlled process variable. That's the input to the controller neuron.

- One write binding per manipulated variable. That's the output from the controller neuron.

- One or two event bindings for safety conditions. Over-temperature, over-pressure, sensor-fail flags. These drive InterlockNeurons.

Resist the urge to bind every variable the FMU exposes. The test is more readable when the binding list matches the conceptual control structure of the model.

## 9.2 Designing the FMU

If you have control over the FMU itself (compiling it from Modelica or Simulink), three guidelines:

- Expose plant outputs the controller would actually see, not internal state. The point of FMI is to test the controller as the controller sees the world. If the real plant has a thermocouple at the tank wall, the FMU should output a thermocouple model — including any lag or noise you can simulate — not the ideal bulk temperature.

- Expose alarm flags as Boolean outputs, not as integer codes or floats above a threshold. The FmuEventInput is built for Boolean events; floats-with-thresholds belong in a MeasurementSignal plus a downstream comparison neuron.

- Be careful about discrete events. FMI co-simulation has limited support for events; if your model produces high-frequency events (chattering relays, fast state machines) the FMU may report FmiStatus.DISCARD and require a smaller stepSize. The bridge surfaces non-OK statuses; tests should assert that no DISCARD occurs.

## 9.3 Adding a new bound variable

The three-step procedure:

- YAML: add a reads: or writes: or events: entry with a unique bindingId, the exact fmuVariable name from the model description, a stable signalTag, and (for writes) the safety ladder values.

- Java neurons: add or update neurons that consume the new signal tag.

- Restart. The FMU init sequence runs on every start; the new binding is resolved against the model description.

## 9.4 Writing a closed-loop CI test

The pattern. A minimal @Test:

- Load YAML pointing at a committed test FMU.

- Construct StubFmuDriver if your test is hermetic, or your real driver if it isn't.

- Build the bridge + a small set of neurons under test (e.g. a PIDLoopNeuron for the heater).

- Tick the worker N times with clock.mode: AS_FAST_AS_POSSIBLE.

- Read the audit JSONL and assert:

- No verdict=FAILED records.

- No verdict=INTERLOCK_TRIP unless the test specifically simulates one.

- The last tank.T value is within tolerance of the setpoint.

- Optionally, repeat with the same seed and assert byte-identical audit output (S12 — deterministic replay).

The bundled FmuBridgeIntegrationTest is the reference implementation; consult it for the assertion patterns.

# 10. Clock modes — when to use which

This is the most consequential configuration choice for a new FMI deployment.

AS_FAST_AS_POSSIBLE (default):

- svc.step() calls driver.doStep() and returns immediately.

- The simulation advances as fast as the JVM can run.

- CI-suitable: a 60-second simulated scenario completes in milliseconds.

- Deterministic byte-identical across runs given the same seed.

- Use this for every automated test.

REAL_TIME:

- svc.step() sleeps until the next scheduled wall-clock instant before returning.

- The simulation advances at the same rate as the wall clock.

- HIL-suitable: an operator can interact with the system through an HMI or REPL.

- Non-deterministic by nature (jitter, scheduling).

- Use this for hands-on demos, not for tests.

A CI suite that fails intermittently with REAL_TIME is reporting clock jitter, not a bug in the controller. Move it to AS_FAST_AS_POSSIBLE and the flake goes away.

# 11. The egress safety algorithm

When the worker calls FmuCommandOutputAggregator.save(), control flows like this:

- The outer aggregator delegates to the inner anonymous subclass of AbstractBridgeOutputAggregator.

- The inner runs the universal §2.2 algorithm:

- Partition results into interlocks, overrides, and commands.

- For every tripped InterlockSignal: resolve bindings whose loopId matches via bindingsForInterlock(); call issueWrite() with the binding's failSafeValue regardless of mode. Audit INTERLOCK_TRIP.

- For every OperatorOverrideSignal: refresh OverrideRegistry for the tag with TTL.

- For every command (ActuatorCommandSignal):

- Resolve binding by tag. UNKNOWN_TAG → audit REJECTED, no write.

- Override active for tag → audit REJECTED reason=OVERRIDE_HOLD.

- SHADOW → audit REJECTED reason=SHADOW_MODE.

- ADVISORY and (execute=false OR no operator confirmation) → audit REJECTED reason=ADVISORY_HOLD.

- Clamp to [minClampValue, maxClampValue].

- Rate-limit by rampRateMaxPerSec * dt.

- Diff-suppress writes within ε of the last applied value within 5 s.

- Call issueWrite(binding, value) which forwards to svc.setReal(binding.fmuVariable(), value).

- Audit APPLIED (with modify reason if clamped/rate-limited) or FAILED on driver exception.

- After the inner returns, the outer calls `svc.step(timestampMs)`. This flushes all buffered writes to the driver (driver.setReal(vref, value) for each), calls driver.doStep(simTime, stepSize), then refreshes the cache by calling driver.getReal(vref) / driver.getBoolean(vref) / driver.getInteger(vref) for every configured variable.

Step 3 is what makes the FMI bridge unique. Every other bridge's aggregator writes asynchronously to a network; the FMI bridge writes and advances the simulator's clock in a single bounded operation. That's why doStep happens even when no results came in — otherwise simulation time would freeze.

# 12. Audit and transparency

The bridge writes one JSONL record per decision, conforming to BridgeAuditRecord (00-FRAMEWORK.md §4):

{ "ts": 1740000000000, "run": 12345, "bridge": "fmi", "verdict": "APPLIED", "loopId": "HEATER-Q", "tag": "PLANT.HEATER01.SP", "proposed": 45000.0, "effective": 45000.0, "reason": "", "safetyMode": "AUTONOMOUS", "evidenceNeurons": ["PIDLoop-1"] }

Records you will see:

| `verdict` | `reason` | Triggered by |
| --- | --- | --- |
| APPLIED | (empty) | Command applied verbatim to the FMU input. |
| APPLIED | CLAMPED_HIGH / CLAMPED_LOW | Value pinned to clamp boundary. |
| APPLIED | RATE_LIMITED | Step exceeded rampRateMaxPerSec. |
| APPLIED | DIFF_SUPPRESSED | Value identical within ε to last applied within 5 s; no FMU write. |
| REJECTED | UNKNOWN_TAG | No write binding for the command's tag. |
| REJECTED | SHADOW_MODE | Binding in SHADOW. |
| REJECTED | OVERRIDE_HOLD | Operator override active for tag. |
| INTERLOCK_TRIP | (empty) | Tripped interlock wrote failSafeValue. |
| FAILED | FMU_EXCEPTION:<detail> | svc.setReal() threw FmuException. |
| FAILED | EXCEPTION:<class> | Other runtime exception in the write path. |

The audit log is the test report. A clean run shows a steady stream of APPLIED records; any FAILED or unexpected INTERLOCK_TRIP is a test failure.

# 13. Operating procedures

## 13.1 First closed-loop run with StubFmuDriver

The fastest path to a working closed loop, no native code:

- mvn -pl worker package.

- Copy §7.2 minimal config to fmu-bridge.yml. Change path to anything — StubFmuDriver ignores it.

- Write a bootstrap that constructs new StubFmuDriver() instead of a real driver. Construct an in-memory FmuModelDescription matching the stub's variable names (tank.T, heater.Q).

- Add a PIDLoopNeuron configured with target=60.0, kp=2000, ki=200, tag=PLANT.HEATER01.SP, source PLANT.TANK01.TEMP. Promote TANK-TEMP and HEATER-Q to AUTONOMOUS.

- Tick the worker. Watch the audit JSONL: within ~30 simulated seconds the tank.T cache should settle near 60 °C.

This is the FmuBridgeIntegrationTest scenario S7. The test runs in milliseconds; consult its source for the exact assertion code.

## 13.2 Adding a real FMU

For a real FMU file:

- Open the FMU ZIP and read modelDescription.xml. Note the model name, the variable names, and the FMI version.

- Implement an FmuDriver adapter that wraps your FMI library (JNA + libfmi, or javafmi).

- Build FmuModelDescription via FmuModelDescription.parse(fmuPath).

- Wire as in §8.

- Test the FMU instantiation first before plugging in a controller. A failed instantiation usually means platform mismatch — the FMU was compiled for win64 and you're on linux64. The bridge surfaces this as a clean exception.

## 13.3 Promoting from SHADOW to AUTONOMOUS

For an FMI bridge, the promotion path is collapsed because the "field" is a simulator:

- Run in SHADOW first to verify your controller produces sensible commands. Inspect the audit JSONL.

- Promote directly to AUTONOMOUS. There is no ADVISORY rung for FMU bridges — operator confirmation in a simulation doesn't add real-world safety, and ADVISORY mode requires a confirmation UI that nobody wants to build for testing.

## 13.4 Diagnosing an unstable closed loop

If tank.T oscillates or diverges:

- Check clock.stepSize matches the worker tick. A mismatch is the most common cause of simulated instability.

- Lower the PID gain.

- Inspect the audit log for RATE_LIMITED or CLAMPED_HIGH/LOW reasons. A rate-limited controller saturates and becomes unstable.

- If using REAL_TIME mode, check the host load. The bridge logs warnings when a tick takes longer than stepSize wall-clock seconds.

# 14. Limitations and future work

- No multi-FMU compositions. A single bridge instance manages exactly one FMU. Multi-FMU scenarios (a plant FMU + a battery FMU + an inverter FMU glued together by Jneopallium) work today by instantiating multiple bridges, each with its own YAML, sharing the same worker. A future enhancement would be a single FmiBridgeConfig that names several FMUs with cross-references.

- FMI for Model Exchange is not supported. The bridge implements co-simulation only. ME requires an external ODE solver and a fundamentally different lifecycle; that's an open question for a future bridge.

- No checkpointing. FMI 2.0+ supports fmiGetFMUstate / fmiSetFMUstate for snapshotting a simulation. The bridge does not currently expose these. Useful for scenario sweeps; out of scope for v1.

- No event-mode for co-simulation. FMI 3.0 distinguishes step-mode from event-mode; the bridge treats every step as step-mode. Models that emit a high event density may produce DISCARD statuses requiring smaller stepSize.

- The audit log accumulates fast. A 1 kHz tick with 10 write bindings produces 10 000 audit lines per simulated second. Rotate the log or set audit.writeRejectedToAudit: false for long-running scenario sweeps.

# 15. Regulatory posture

| Aspect | Posture |
| --- | --- |
| Safety ceiling | AUTONOMOUS. By design — the "field" is a simulator. |
| Default per-loop mode | SHADOW (inherited from safetyMode() returning SHADOW for unlisted bindings). Tests must explicitly promote to AUTONOMOUS. |
| Interlock authority | Direct. Useful for testing your interlock neurons themselves. |
| Operator override | Functional via aggregator.overrideRegistry(). Useful for scenario tests that simulate operator interventions. |
| Audit | One JSONL line per decision. Schema documented in 00-FRAMEWORK.md §4. |
| Determinism | AS_FAST_AS_POSSIBLE clock + identical seed + identical FMU = byte-identical audit log. Required for CI replay (acceptance scenario S12). |
| Real-world deployment | None. The FMU bridge is for testing. A production deployment connecting Jneopallium to live plant equipment uses OPC UA or PLC4X — possibly with an FMU running in parallel as a digital twin via this bridge, against which the worker's decisions can be verified before hitting the real bridge. |

A common production pattern: run the OPC UA bridge against live equipment, and run an FMI bridge against a digital-twin FMU of the same equipment, with both inputs feeding the same controller. Compare the audit logs from the two aggregators in post-processing to detect controller-model divergence. The FMI bridge is the calibration anchor.

# 16. References

- 03-FMI-FMU.md — the bridge specification.

- 00-FRAMEWORK.md — the universal bridge contract.

- FMI standard — https://fmi-standard.org/.

- Modelica Standard Library and reference FMUs — https://github.com/modelica/Reference-FMUs.

- OpenModelica — https://openmodelica.org/. Free Modelica compiler suitable for generating test FMUs.

- FMI tool catalog — https://fmi-standard.org/tools/. List of 280+ tools that import/export FMUs.

- javafmi — https://bitbucket.org/siani/javafmi/. One Java FMI library; release cadence is inconsistent.

- JNA — https://github.com/java-native-access/jna. The dependency for hand-rolled FMI adapters.

- Rakovskyi, D. (2024). Framework for Natural Neuron Network Modeling: The Jneopallium Approach. IJSR 13(7).