Building Models with Lab Streaming Layer and Jneopallium

User Manual

Neural data integration · stimulation safety gate · pseudonymised audit

Version: 1.0-SNAPSHOT

License: BSD 3-Clause

Author: Dmytro Rakovskyi

# 1. Introduction

[Lab Streaming Layer](https://labstreaminglayer.org/) (LSL) is the open-source middleware ecosystem used in BCI and physiology research. It standardises streaming, time-synchronisation, and recording of neural, physiological, and behavioural data across heterogeneous acquisition devices — EEG amplifiers, EMG sensors, eye-trackers, motion-capture systems, custom hardware. The Lab Streaming Layer is what most BCI labs and several commercial BCI products use to glue together hardware from different vendors.

LSL is unusual among the protocols Jneopallium integrates with. There is no central broker. Devices announce streams via mDNS multicast on the local subnet; clients discover and pull from them. The Java binding is liblsl-Java — JNI bindings over the native liblsl shared library, which itself is C++ with platform-specific binaries for linux64, osx-arm64, win64. A worker that uses this bridge in production must ship the matching liblsl binary, which is why the bridge core compiles without the jar and goes through an LslTransport seam.

The bridge maps inbound LSL chunks to existing typed Jneopallium signals — LFPSignal, NeuralSpikeSignal, ECoGSignal, ProprioceptiveSignal, InteroceptiveSignal, AppraisalSignal, ThermalSignal, CalibrationSignal — and publishes egress markers and numeric samples on named outlets that stimulator-driver software subscribes to (Jneopallium-Stim-Advisory, Jneopallium-Intent, Jneopallium-Risk). The structural ceiling is ADVISORY. Stimulation commands are routed through the existing StimulationSafetyGateNeuron — a separate cognitive primitive that applies the Shannon criterion, charge-balance check, frequency safe-operating-area, and seizure/thermal lockouts. If the gate vetoes a command, the outlet is not written and the audit records verdict=REJECTED reason=GATE_VETO.

Two non-functional concerns dominate the design. Privacy. Marker text — which can contain subject identifiers in research data — is hashed before it crosses into Jneopallium's signal vocabulary. The raw subject_42_calibration_start marker becomes the sessionId MARKER-<hex>. The audit JSONL stays on the local host; there is no cloud egress. Consent. The bridge does not enforce consent; the operator's runbook does. The manual is explicit about both.

This manual explains how to install the dependency, set up an LSL publisher, configure the bridge against neural and physiological streams, wire the stimulation safety gate, and validate end to end. It assumes a Java engineer who knows Maven, has read the universal bridge framework (00-FRAMEWORK.md), and has at least passing familiarity with one BCI acquisition tool (OpenViBE, OpenBCI's GUI, BCI2000, or a custom pylsl script).

# 2. Six rules you must internalise before writing any code

The universal six from 00-FRAMEWORK.md §0, specialised for LSL:

- No raw write to a field actuator from neuron output. The "field actuator" here is electrical stimulation hardware — TES, tCS, or implanted stimulators. The bridge never drives any of them directly; it writes to named LSL outlets (Jneopallium-Stim-Advisory) that separately certified stimulator-driver software subscribes to. The certified driver decides whether to forward the advice to electrodes. Defence in depth: per-tag AUTONOMOUS is rejected at config load, and every StimulationCommandSignal is routed through StimulationSafetyGateNeuron before any outlet push.

- Interlocks have direct authority. Seizure-risk neurons emit interlock-like signals (SeizureRiskSignal) that cause the aggregator to write the binding's failSafeValue (typically 0 amplitude) regardless of any other state. Useful for the seizure-prevention case.

- Operator override wins for advisory control. The operator (research assistant, BCI clinician) can hold a stimulation-advisory tag for a configured TTL via OperatorOverrideSignal. The downstream driver software sees no advisory updates for that tag while the override is active.

- Every outlet push and every gate veto produces an audit record. Records cover accepted pushes, SHADOW-dropped commands, GATE_VETO, GATE_UNCONFIGURED, operator overrides, ramp limits, and decoder failures on ingress. Records are written to a local JSONL file. No SOC mirror; the audit is local-only per LSL privacy posture.

- Quality propagates. LSL inlets carry sample timestamps; the bridge stamps Quality.GOOD when the inlet has a live publisher (isAlive() == true) and Quality.UNCERTAIN after stream loss until the cache is refreshed by a fresh sample. The bridge does not synthesise quality from sample value content.

- Wall-clock timestamps come from the source. LSL provides a global clock via LslTransport.localClockSeconds() plus per-inlet timeCorrectionSeconds() — the canonical time alignment mechanism in BCI research. The mapper combines them per sample to produce a corrected timestamp; the framework's System.currentTimeMillis() is only the fallback when no LSL time correction is available.

# 3. Prerequisites

- JDK 17 or later.

- Maven 3.9 or later.

- For real LSL traffic: a working LSL installation on the host. The native liblsl binary must match the JVM platform (linux64, osx-arm64, win64). The Java binding is edu.ucsd.sccn:liblsl-Java.

- For development / CI without LSL: nothing. The bridge ships InMemoryLslTransport in src/test/java; every integration test in LslBridgeIntegrationTest runs against it.

- For end-to-end smoke testing: a Python with pylsl installed, or any of OpenViBE / OpenBCI GUI / BCI2000 / LabRecorder that can publish a stream on the local subnet.

- For ethics: consent capture infrastructure consistent with your institution's IRB or ethics board. This is a procedural prerequisite, not a software dependency, but it is non-optional for any deployment involving human subjects.

# 4. Adding Jneopallium to your project

The LSL bridge ships in the same worker artifact as the rest of Jneopallium:

<repositories> <repository> <id>gitlab-jneopallium</id> <url>https://gitlab.com/api/v4/projects/44434192/packages/maven</url> </repository> </repositories> <dependencies> <dependency> <groupId>com.rakovpublic.jneopallium</groupId> <artifactId>worker</artifactId> <version>1.0-SNAPSHOT</version> </dependency> </dependencies>

For production deployments add the LSL Java binding only in the deployment module, not in the worker:

<dependency> <groupId>edu.ucsd.sccn</groupId> <artifactId>liblsl-Java</artifactId> <version>1.16.0</version> </dependency>

The bridge core ignores this dependency at compile time — every LSL call goes through LslTransport. Your deployment supplies an LiblslLslTransport adapter that delegates to edu.ucsd.sccn.LSL.StreamInlet and StreamOutlet; the bridge does not know it exists. Tests inject InMemoryLslTransport so the integration suite runs offline.

# 5. Architecture

The bridge introduces eleven boundary classes under worker.bridge.lsl:

┌────────────────────────────┐ resolve (mDNS multicast) │ LSL multicast subnet │ ◀───────────────────────────────────┐ │ │ │ │ EEG@OpenViBE │ ──── samples ─────────┐ │ │ EMG@OpenBCI │ ──── samples ─────────┤ │ │ HRV@Polar-H10 │ ──── samples ─────────┤ │ │ Markers@Experiment │ ──── samples ─────────┤ │ │ │ │ │ │ Jneopallium-Stim-Advisory │ ◀── push (numeric) ── │ │ │ Jneopallium-Intent │ ◀── push (markers) ── │ │ │ Jneopallium-Risk │ ◀── push (numeric) ── │ │ └────────────────────────────┘ │ │ ▼ ▼ ┌───────────────────────────────────────┐ │ LslClientService │ │ • owns the transport │ │ • inlet ring buffers (per binding) │ │ • outlet handles (per binding) │ │ • marker-cue regex match → calib │ │ • time-correction cache │ └─┬────────┬──────────────────┬─────────┘ │ │ │ ┌──────────────▼─┐ ┌────▼──────────┐ ┌─────▼──────────┐ │ LslNeuralInput │ │ LslPhysiology │ │ LslMarkerInput │ │ (IInitInput → │ │ Input │ │ (IInitInput → │ │ LFP / Spike / │ │ (IInitInput → │ │ Calibration / │ │ ECoG) │ │ Interocep / │ │ bridge events)│ │ │ │ Appraisal / │ │ │ │ │ │ Thermal / │ │ │ │ │ │ Proprioceptive)│ │ │ └────────────────┘ └───────────────┘ └────────────────┘ ▼ [Standard Jneopallium pipeline] ▼ LslAdvisoryOutputAggregator │ │ StimulationCommandSignal → │ StimulationSafetyGateNeuron.allows() ? │ ▼ LslClientService.pushNumeric() / .pushMarker() │ ▼ LslAuditOutput (JSONL, local-only)

Components:

| Class | Role |
| --- | --- |
| LslBridgeConfig | YAML record (immutable). Enforces ADVISORY ceiling at construction. |
| LslBridgeConfigLoader | Jackson YAML loader. FAIL_ON_UNKNOWN_PROPERTIES = true. |
| LslTransport | Test-seam interface. resolveInlet / openOutlet / localClockSeconds / close. Plus inner Inlet, Outlet, Sample types. |
| LslStreamBinding | Resolved binding identity used at runtime by the service. |
| LslSignalMapper | Pure functions. Maps inbound LSL chunks → typed signals; hashes marker text into pseudonymised session ids. |
| LslClientService | Owns the transport, the per-binding ring buffers, the outlet handles, the time-correction cache, the audit channel. AutoCloseable. |
| LslNeuralInput | IInitInput. Drains LFP / NeuralSpike / ECoG bindings. |
| LslPhysiologyInput | IInitInput. Drains Interoceptive / Appraisal / Thermal / EMG-proprioceptive bindings. |
| LslMarkerInput | IInitInput. Drains marker bindings (pseudonymised → CalibrationSignal) plus bridge events. |
| LslAdvisoryOutputAggregator | IOutputAggregator. Routes StimulationCommandSignal through the safety gate, publishes to advisory outlets, audits every decision. Does NOT extend AbstractBridgeOutputAggregator because the gate-veto path makes the universal §2.2 algorithm too narrow. |
| LslAuditOutput | Local JSONL append-only audit. |

The aggregator deliberately diverges from AbstractBridgeOutputAggregator. The reason: BCI stimulation safety lives in a domain-specific gate neuron (StimulationSafetyGateNeuron), not in a universal clamp-and-rate-limit chain. The aggregator's algorithm interleaves the universal §2.2 steps with a gate.allows(command) check that has no analogue in industrial bridges. Implementing the LSL-specific safety logic directly — rather than inheriting and trying to override the universal one — keeps the code easy to audit.

# 6. The stimulation safety gate

Two facts about brain stimulation make it different from any other actuator. First, safe stimulation parameters are non-trivially constrained. The Shannon criterion (k > 1.7 where k = log10(D · Q / A²), with D charge density, Q charge per phase, A electrode area) is the empirical safety boundary against electrode-tissue damage; below it, electrodes and tissue both degrade. Charge balance (every cathodal phase compensated by an anodal one within the inter-phase interval) prevents DC injection. Frequency safe-operating-area limits prevent thermal damage and seizure induction. Second, a bad stimulation event is hard to reverse. An overshoot of mechanical position can be undone by reversing motion; an overstimulation event causing a seizure cannot be undone.

The bridge addresses both by routing every StimulationCommandSignal through an IStimulationSafetyGateNeuron. The repository ships StimulationSafetyGateNeuron as the canonical implementation; sites with different stimulator hardware or different IRB constraints write their own and inject it. The contract is small: boolean allows(StimulationCommandSignal cmd, IContext ctx). The gate sees the command's amplitude, frequency, pulse width, electrode pair, and the current cognitive context (seizure risk, thermal model, charge-balance state, time since last stimulation, lockout windows).

If the gate is not configured, every `StimulationCommandSignal` is rejected with `reason=GATE_UNCONFIGURED`. This is structural: there is no code path from a stimulation command to an outlet that bypasses the gate. Wiring is in your bootstrap (see §8); accidentally leaving the gate unset is the safe failure mode.

# 7. Configuration

Configuration is YAML, loaded once at startup through LslBridgeConfigLoader.load(Path). Hot-reload is not supported. The loader sets FAIL_ON_UNKNOWN_PROPERTIES = true.

## 7.1 Schema

The configuration has six top-level keys.

- discovery — resolve timeout and the list of expected stream names. Fail-fast if any expected stream is absent.

- reads — list of inlet bindings.

- writes — list of outlet bindings.

- audit — local JSONL file path.

- perTagSafetyMode — per-binding SHADOW / ADVISORY. AUTONOMOUS is rejected at load.

- tickInterval — worker tick cadence (ISO-8601). Default PT0.25S.

## 7.2 Minimal example — EEG smoke test

The smallest useful configuration. One EEG inlet binding, no writes, suitable as a first integration check against a pylsl publisher.

discovery: resolveTimeoutMs: 3000 expectedStreams: - "OpenViBE-EEG-256Hz" reads: - bindingId: EEG-CZ-FZ-PZ-OZ streamName: "OpenViBE-EEG-256Hz" streamType: EEG channels: ["Cz", "Fz", "Pz", "Oz"] chunkLengthSamples: 32 targetSignal: LFP signalTag: "BCI.EEG.MIDLINE" writes: [] audit: localAuditFile: "/tmp/jneopallium-lsl-audit.jsonl" tickInterval: "PT0.1S"

## 7.3 Production-like example — full BCI session

A configuration shaped like a research deployment: EEG ingress for LFP, HRV for interoception, eye-tracking for appraisal, markers for calibration cues; advisory outlets for intent and risk plus a stimulation-gated advisory outlet for stim parameters.

discovery: resolveTimeoutMs: 5000 expectedStreams: - "OpenViBE-EEG-256Hz" - "Polar-H10-HRV" - "Pupil-Capture-Gaze" - "Experiment-Markers" reads: - bindingId: EEG-MIDLINE streamName: "OpenViBE-EEG-256Hz" streamType: EEG channels: ["Cz", "Fz", "Pz", "Oz"] chunkLengthSamples: 64 targetSignal: LFP signalTag: "BCI.EEG.MIDLINE" decimateBy: 2 - bindingId: HRV-RR streamName: "Polar-H10-HRV" streamType: HRV channels: ["RR"] chunkLengthSamples: 1 targetSignal: INTEROCEPTIVE signalTag: "PHYSIO.HRV.RR" - bindingId: EYE-GAZE streamName: "Pupil-Capture-Gaze" streamType: Eye channels: ["gaze_x", "gaze_y", "pupil_diameter"] chunkLengthSamples: 1 targetSignal: APPRAISAL signalTag: "PHYSIO.EYE.GAZE" - bindingId: CAL-MARKERS streamName: "Experiment-Markers" streamType: Markers targetSignal: CALIBRATION_MARKER signalTagPrefix: "CAL" calibrationCueRegex: "^(subject_\\d+_)?(calibration¦baseline)_start$" writes: - bindingId: INTENT-OUT outletName: "Jneopallium-Intent" type: MARKERS signalTag: "BCI.INTENT" - bindingId: RISK-OUT outletName: "Jneopallium-Risk" type: NUMERIC nominalSrate: 4.0 signalTag: "BCI.SEIZURE.RISK" minClampValue: 0.0 maxClampValue: 1.0 - bindingId: STIM-ADVISORY outletName: "Jneopallium-Stim-Advisory" type: NUMERIC nominalSrate: 1.0 signalTag: "BCI.STIM.PARAMS" minClampValue: 0.0 maxClampValue: 5.0 # mA rampRateMaxPerSec: 0.5 failSafeValue: 0.0 stimulationGated: true audit: localAuditFile: "/var/log/jneopallium/lsl-audit.jsonl" perTagSafetyMode: INTENT-OUT: ADVISORY RISK-OUT: ADVISORY STIM-ADVISORY: ADVISORY tickInterval: "PT0.25S"

## 7.4 Field reference

discovery.resolveTimeoutMs is the per-stream resolve timeout, in milliseconds. Default 2000. The bridge fails fast at start-up if any expectedStreams entry is not resolved within this — the stream simply isn't there, and silently running without it would produce missing data downstream.

discovery.expectedStreams lists stream names that must be present. A stream named in reads but not in expectedStreams is resolved best-effort; a stream named in expectedStreams causes the bridge to refuse to start without it.

reads[].streamName is the LSL name metadata field of the publisher. Must match exactly.

reads[].streamType is the LSL type metadata field — EEG, EMG, HRV, Eye, Markers, etc. Used as a sanity check against the resolved inlet; a mismatch fails fast.

reads[].channels is the explicit channel allow-list. Resolution against the inlet's reported channel layout fails fast on a missing label (per acceptance scenario S10). Order is significant: the i-th channel becomes the i-th column of the emitted signal payload.

reads[].chunkLengthSamples is the number of samples per pulled chunk. Higher gives larger neuron payloads with lower per-tick CPU; lower gives finer time resolution. EEG at 256 Hz with chunkLengthSamples: 32 produces one chunk every 125 ms.

reads[].targetSignal is one of LFP, NEURAL_SPIKE, ECOG, EMG_PROPRIOCEPTIVE, INTEROCEPTIVE, APPRAISAL, THERMAL, CALIBRATION_MARKER. The kind determines the emitted signal class — see §9.

reads[].decimateBy drops all but every Nth sample. Useful when subscribing to a high-rate stream (1 kHz EEG) but the model ticks slower.

reads[].calibrationCueRegex is valid only for CALIBRATION_MARKER bindings. It matches incoming marker text; when matched, the bridge emits a CalibrationSignal whose sessionId is a hash of the raw marker text (pseudonymisation per §10 R4). Unmatched markers do not produce signals.

reads[].ringBufferMaxSamples is the bridge's per-binding ring buffer cap. Samples beyond this are dropped with one audit per overflow. Default 4096.

writes[].outletName is the LSL outlet name advertised on the multicast network. By convention, every Jneopallium-published outlet starts with Jneopallium-.

writes[].type is MARKERS (string payload, irregular rate) or NUMERIC (double payload, fixed nominal rate). nominalSrate is required for NUMERIC and ignored for MARKERS.

writes[].stimulationGated: true routes the corresponding signalTag's StimulationCommandSignal through StimulationSafetyGateNeuron before any push. Set on every stimulation-related write binding.

perTagSafetyMode keys are bindingIds. Legal values: SHADOW, ADVISORY. AUTONOMOUS is rejected with IllegalArgumentException at load time. Unlisted bindings default to SHADOW.

# 8. Wiring the Java side

The bridge does not ship a main class. A minimal driver:

package com.example.bci; import com.rakovpublic.jneuropallium.worker.bridge.common.AbstractBridgeAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslAdvisoryOutputAggregator; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslBridgeConfig; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslBridgeConfigLoader; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslClientService; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslMarkerInput; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslNeuralInput; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslPhysiologyInput; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslSignalMapper; import com.rakovpublic.jneuropallium.worker.bridge.lsl.LslTransport; import com.rakovpublic.jneuropallium.worker.net.neuron.impl.bci.IStimulationSafetyGateNeuron; import com.rakovpublic.jneuropallium.worker.net.neuron.impl.bci.StimulationSafetyGateNeuron; import java.nio.file.Path; public final class LslBootstrap { public static void main(String[] args) throws Exception { // 1. Load YAML (AUTONOMOUS rejected at load). LslBridgeConfig cfg = LslBridgeConfigLoader.load(Path.of("lsl-bridge.yml")); // 2. Open audit sink. try (AbstractBridgeAuditOutput audit = new LslAuditOutput( Path.of(cfg.audit().localAuditFile()))) { // 3. Build the transport. In production this is your // LiblslLslTransport wrapping edu.ucsd.sccn.LSL; in dev/test // the repo ships InMemoryLslTransport. try (LslTransport transport = new LiblslLslTransport()) { // 4. Construct the signal mapper and the client service. LslSignalMapper mapper = new LslSignalMapper(); LslClientService svc = new LslClientService(cfg, transport, mapper, audit); // 5. Start: resolve every expected inlet, open every outlet. // Fails fast on missing expectedStreams. svc.start(); try { // 6. Build the inputs. Three of them, by signal family. LslNeuralInput neural = new LslNeuralInput("lsl-neural", svc); LslPhysiologyInput physio = new LslPhysiologyInput("lsl-physio", svc); LslMarkerInput marker = new LslMarkerInput("lsl-marker", svc); // 7. Build the stimulation safety gate. This is the // canonical implementation; sites with different // hardware write their own and inject it here. IStimulationSafetyGateNeuron gate = new StimulationSafetyGateNeuron(); // 8. Build the aggregator. Pass the gate explicitly — // if you forget, every StimulationCommandSignal // is rejected with reason=GATE_UNCONFIGURED. LslAdvisoryOutputAggregator agg = new LslAdvisoryOutputAggregator(svc, gate, audit); // 9. Register with the worker. worker.registerInput(neural); worker.registerInput(physio); worker.registerInput(marker); worker.registerOutputAggregator(agg); worker.run(); } finally { svc.close(); } } } } }

Three things worth highlighting. Three inputs, by signal family. Neural (LFP / spike / ECoG), physiology (interoception / appraisal / thermal / EMG-proprioceptive), markers (calibration + bridge events). The split mirrors how a BCI experiment naturally segments its data and lets downstream neurons attach to the appropriate input. The gate is explicit and mandatory. It is not constructed inside the bridge; you pass it in. If you write a one-line bootstrap that forgets the gate parameter (and the constructor allows null for testing), every stimulation command will be rejected. That's the structural safety mechanism — there is no override. `AutoCloseable` is non-negotiable. Closing the service closes every inlet and outlet. Failure to close leaks liblsl resources and on some platforms causes mDNS multicast announcements to persist beyond the process lifetime — confusing for the next run.

# 9. Modeling guidance

## 9.1 Choosing what to bind on ingress

Start with the minimum a controller actually needs. For a typical motor-imagery BCI:

- One EEG binding (LFP) — neural activity over motor cortex.

- One marker binding (CALIBRATION_MARKER) — experimental cues defining trial onsets.

For a physiological-feedback or affective-computing model, add:

- One HRV binding (INTEROCEPTIVE) — heart rate variability proxy for arousal.

- One eye-tracking binding (APPRAISAL) — gaze position and pupil diameter.

For a closed-loop stimulation model, add:

- One advisory stim outlet (NUMERIC, stimulationGated: true).

- One risk outlet (NUMERIC) — emit SeizureRiskSignal-derived values.

- One intent outlet (MARKERS) — emit decoded intent labels.

Resist the urge to bind every channel of every stream. EEG headsets often expose 64+ channels; subscribing to all of them and running them through neurons that only need 4 wastes CPU.

## 9.2 The targetSignal mapping table

The bridge maps each inlet kind to a Jneopallium signal class:

| `targetSignal` | Signal class produced | Typical LSL stream type |
| --- | --- | --- |
| LFP | LFPSignal | EEG |
| NEURAL_SPIKE | NeuralSpikeSignal | Spikes, MEA |
| ECOG | ECoGSignal | ECoG |
| EMG_PROPRIOCEPTIVE | ProprioceptiveSignal | EMG |
| INTEROCEPTIVE | InteroceptiveSignal | HRV, GSR, BVP |
| APPRAISAL | AppraisalSignal | Eye, Gaze |
| THERMAL | ThermalSignal | Temperature |
| CALIBRATION_MARKER | CalibrationSignal (when calibrationCueRegex matches) | Markers |

A research lab streaming a new modality (e.g. fNIRS) can extend the mapper rather than the bridge core — implement a custom LslSignalMapper subclass and wire it in.

## 9.3 Pseudonymisation of markers

CALIBRATION_MARKER bindings deserve careful handling. LSL marker text is often subject-identifying in research data — subject_42_calibration_start, participant_alice_trial_3. The bridge hashes the raw marker text before producing a CalibrationSignal; the signal carries sessionId = "MARKER-" + hex(hash(rawText)). Downstream code sees the hash, not the identifier. The raw text never crosses into Jneopallium's signal vocabulary.

This is a one-way reduction. There is no way to reverse the hash; if your downstream model needs to attach metadata to a subject across sessions, that mapping lives in your experiment-management software, not in Jneopallium. The bridge intentionally cannot bridge that gap.

## 9.4 Adding a new advisory write

The procedure:

- YAML — write binding. Set outletName (Jneopallium- prefix recommended), type (NUMERIC for continuous, MARKERS for discrete events), nominalSrate (NUMERIC only), signalTag for the source signal, and (for stimulation) stimulationGated: true.

- YAML — start in SHADOW. Every command audited as REJECTED reason=SHADOW_MODE, no outlet push.

- Run. Inspect the audit JSONL. Verify the bridge would push sensible values.

- Promote to ADVISORY. perTagSafetyMode: { STIM-ADVISORY: ADVISORY }. Restart.

- Stay in ADVISORY. AUTONOMOUS is structurally unreachable on this bridge.

# 10. The advisory egress algorithm

When the worker calls LslAdvisoryOutputAggregator.save(), the aggregator does this in order:

- Partition results into interlocks, overrides, commands.

- For every SeizureRiskSignal (treated as an interlock-like signal): find every write binding with the matching loopId; push the failSafeValue (typically 0 mA) on each outlet regardless of mode. Audit INTERLOCK_TRIP reason=SEIZURE_RISK.

- For every OperatorOverrideSignal: refresh the override registry for the tag with TTL.

- For every StimulationCommandSignal:

- Resolve binding by tag. UNKNOWN_TAG → audit REJECTED, no push.

- Override active → audit REJECTED reason=OVERRIDE_HOLD.

- Mode SHADOW → audit REJECTED reason=SHADOW_MODE.

- If `stimulationGated: true` for this binding and `gate == null` → audit `REJECTED reason=GATE_UNCONFIGURED`.

- If `stimulationGated: true` and `gate.allows(cmd, ctx)` returns false → audit `REJECTED reason=GATE_VETO:<gate-reason>`. No push. This is the safety boundary.

- Clamp amplitude to [minClampValue, maxClampValue].

- Ramp-limit by rampRateMaxPerSec * dt.

- Diff-suppress writes within ε of the last applied value within 5 s.

- Push numeric sample to outlet via LslClientService.pushNumeric(binding, payload).

- Audit APPLIED reason=STIM_ADVISORY_PUSHED (with modify reason if clamped / rate-limited).

- For every IntentSignal:

- Resolve binding (MARKERS outlet).

- Same SHADOW / override / threshold check (intent confidence threshold).

- Push marker text via LslClientService.pushMarker(binding, text).

- For every TransparencyLogSignal:

- Write to local JSONL. (No outlet mirror — LSL local-only privacy posture.)

The gate is the headline novelty. Every other bridge's safety is structural-then-clamp; this one inserts a domain-specific veto between override and clamp. The order matters: a gate veto stops the algorithm before the value reaches the network.

# 11. Audit and transparency

The bridge writes one JSONL record per decision, conforming to BridgeAuditRecord:

{ "ts": 1740000000000, "run": 12345, "bridge": "lsl", "verdict": "REJECTED", "tag": "BCI.STIM.PARAMS", "proposed": 3.5, "effective": null, "reason": "GATE_VETO:Shannon_below_threshold", "safetyMode": "ADVISORY", "evidenceNeurons": ["StimulationDecoder-1"] }

Records you will see:

| `verdict` | `reason` | Triggered by |
| --- | --- | --- |
| APPLIED | STIM_ADVISORY_PUSHED | Stimulation advisory pushed to outlet. |
| APPLIED | INTENT_PUSHED | Intent marker pushed to outlet. |
| APPLIED | RISK_PUSHED | Risk numeric pushed to outlet. |
| APPLIED | BRIDGE_RECONNECTED | Stream lost and re-resolved. |
| APPLIED | CLAMPED_HIGH / CLAMPED_LOW | Value clamped before push. |
| APPLIED | RATE_LIMITED | Step exceeded rampRateMaxPerSec. |
| REJECTED | SHADOW_MODE | Binding in SHADOW. |
| REJECTED | UNKNOWN_TAG | Signal tag has no writes: entry. |
| REJECTED | OVERRIDE_HOLD | Operator override active for tag. |
| REJECTED | GATE_UNCONFIGURED | Stimulation gate not provided to aggregator. |
| REJECTED | GATE_VETO:<reason> | Stimulation gate refused the command. |
| INTERLOCK_TRIP | SEIZURE_RISK | Seizure-risk signal forced failSafeValue push. |
| FAILED | LSL_STREAM_LOST | Inlet's publisher disappeared (alarm only). |
| FAILED | OUTLET_PUSH_ERROR:<detail> | LSL push failed at the transport layer. |

The audit file is local-only. There is no MQTT or syslog mirror. This is by design — the audit may contain timing data correlated with subject identity, and the privacy posture is that nothing leaves the host.

# 12. Operating procedures

## 12.1 Smoke test with pylsl

The fastest end-to-end check uses a Python publisher:

pip install pylsl python -c " import pylsl, time, math info = pylsl.StreamInfo('OpenViBE-EEG-256Hz', 'EEG', 4, 256, 'float32', 'demo-eeg') ch = info.desc().append_child('channels') for label in ['Cz','Fz','Pz','Oz']: ch.append_child('channel').append_child_value('label', label) outlet = pylsl.StreamOutlet(info) t = 0 while True: sample = [math.sin(2*math.pi*10*t/256.0)] * 4 outlet.push_sample(sample) t += 1 time.sleep(1/256.0) "

This advertises an EEG stream on the local subnet. Use §7.2 minimal config, run LslBootstrap (§8) with the production LiblslLslTransport. Within a couple of seconds the bridge resolves the stream and LslNeuralInput.readSignals() returns LFPSignals.

## 12.2 In-memory tests without LSL

For CI without liblsl:

- Construct new InMemoryLslTransport() instead of LiblslLslTransport.

- Pre-load it with test samples via the transport's API.

- Tick the worker. The bridge sees the in-memory inlets as if they were real.

This is what LslBridgeIntegrationTest does for every acceptance scenario.

## 12.3 Verifying the gate veto

A useful unit test pattern:

- Wire the bridge with a stub IStimulationSafetyGateNeuron whose allows() always returns false.

- Promote the stim binding to ADVISORY.

- Push a StimulationCommandSignal through the aggregator.

- Assert the audit shows verdict=REJECTED reason=GATE_VETO:<your-stub-reason> and no outlet push happened.

Then repeat with allows() returning true; assert verdict=APPLIED reason=STIM_ADVISORY_PUSHED and an outlet push observed on the in-memory transport.

## 12.4 Stream loss and reconnection

LSL is multicast — publishers come and go. The bridge handles this:

- When a publisher disappears, Inlet.isAlive() == false on the next read.

- The bridge emits AlarmSignal(HIGH, LSL_STREAM_LOST) on the event channel via LslMarkerInput.

- Subsequent reads on that binding produce Quality.UNCERTAIN.

- When the publisher returns, resolveInlet() succeeds again; the bridge emits AlarmSignal(LOW, BRIDGE_RECONNECTED) and quality returns to GOOD.

The signal-level guarantee is that stale data is never silently promoted to fresh.

## 12.5 Promoting from SHADOW to ADVISORY

Identical to other bridges:

- Run for an observation period in SHADOW.

- Inspect the audit JSONL: every advisory should appear as REJECTED reason=SHADOW_MODE with the proposed value.

- Confirm with the BCI experimenter that the proposed stimulation parameters are sensible.

- Edit YAML, promote, restart. AUTONOMOUS is structurally unreachable.

# 13. Privacy and ethics

This bridge is the only one in the directory that primarily handles personal physiological and neural data. The structural posture:

- Pseudonymisation by default. Marker text is hashed; raw subject identifiers never enter Jneopallium signals. There is no way to disable this in the YAML.

- Local-only audit. No cloud egress for audit records. No SIEM mirror by default.

- No subject-identifying logs. SLF4J output uses binding ids, never raw marker text.

- The bridge does not enforce consent. That belongs in your IRB/ethics process, the experiment-management software, and the operator runbook.

- Data minimisation. The channels allow-list and decimateBy knob exist partly for privacy: don't pull more data than your downstream code needs.

For deployments involving human subjects, the operator runbook MUST document: consent capture, data retention policy, access controls on the audit file, and the procedure for subject withdrawal (deleting the corresponding session-id hashed records).

# 14. Limitations and future work

- No outlet for raw neural data. The bridge consumes raw neural data on ingress, but its egress outlets are advisory-only (intent markers, risk numerics, stim parameters). A downstream tool that wants to record the typed signals as their own LSL stream needs a separate publisher.

- No time-correction for outlets. Inbound timeCorrectionSeconds() is honoured on ingress; outbound timestamps use LslTransport.localClockSeconds(). For sub-millisecond correlation between bridge outputs and external recorders, configure those recorders to honour Jneopallium's outlet timestamps explicitly.

- `liblsl` platform binaries. The bridge core has no compile dependency, but a production deployment ships one specific liblsl binary per platform. Cross-platform docker images need careful staging.

- Stim gate is binary. The gate either allows or rejects; there is no "allow with modified parameters" path. A future enhancement is a gate that returns a modified command; that requires changing the contract from boolean allows to Optional<Command> evaluate.

- No multi-host LSL. The bridge runs on one host and sees streams announced on its local subnet. Multi-host BCI experiments need an LSL relay (e.g. LabRecorder or LSL-AppDataRoute); the bridge connects to that relay, not directly to the remote streams.

# 15. Regulatory posture

| Aspect | Posture |
| --- | --- |
| Safety ceiling | ADVISORY. Structural — AUTONOMOUS rejected at config load. |
| Default per-binding mode | SHADOW. Every binding must be explicitly promoted to ADVISORY. |
| Stimulation gate | Mandatory. Every StimulationCommandSignal routed through IStimulationSafetyGateNeuron. Gate-not-configured → reject; gate-vetoed → reject; gate-allowed → proceed through clamp/ramp/audit. |
| Interlock authority | SeizureRiskSignal forces failSafeValue push regardless of mode. |
| Operator override | Holds advisory tag for TTL. Defeats neuron-derived advisories; does not defeat the stim gate. |
| Audit | One JSONL line per decision. Local only. No cloud mirror. |
| Pseudonymisation | Mandatory. Marker text hashed; raw subject ids never in signals. |
| MoC | Hot-reload not supported. YAML changes follow site MoC. |
| Consent | Procedural — runbook responsibility, not enforced by code. |
| Hardware certification | The stimulator-driver software downstream of Jneopallium-Stim-Advisory outlet is separately certified per local regulation (IEC 60601 in clinical context; site-specific in research). |

The bridge is a research-grade integration. Clinical deployment requires the stimulator-driver software to be IEC 60601-certified (medical-electrical equipment) and the overall system to satisfy local medical-device regulation (e.g. EU MDR, FDA). The bridge does not claim that certification — it provides the audit trail and structural defences a higher-level safety case needs.

# 16. References

- 05-LSL.md — the bridge specification.

- 00-FRAMEWORK.md — the universal bridge contract.

- docs/lsl-bridge.md — domain-context companion in the repo.

- Lab Streaming Layer — https://labstreaminglayer.org/.

- liblsl GitHub — https://github.com/sccn/liblsl.

- liblsl-Java — https://github.com/labstreaminglayer/liblsl-Java.

- pylsl tutorial — https://github.com/labstreaminglayer/pylsl.

- Shannon, R. V. (1992). A model of safe levels for electrical stimulation. IEEE Trans. Biomed. Eng. 39(4).

- OpenViBE — http://openvibe.inria.fr/. Free BCI acquisition / processing platform.

- OpenBCI — https://openbci.com/. Open hardware EEG / EMG.

- Rakovskyi, D. (2024). Framework for Natural Neuron Network Modeling: The Jneopallium Approach. IJSR 13(7).