Building Models with MQTT, Sparkplug B and Jneopallium

User Manual

Unified-namespace IIoT · advisory-only egress · typed-signal safety

Version: 1.0-SNAPSHOT

License: BSD 3-Clause

Author: Dmytro Rakovskyi

# 1. Introduction

[MQTT](https://mqtt.org/) is the dominant publish/subscribe protocol for industrial IoT — lightweight, broker-mediated, well-supported on edge gateways. [Sparkplug B](https://sparkplug.eclipse.org/) is the Eclipse specification that imposes a topic structure (spBv1.0/<group>/<msg-type>/<edge>/<device>), a Google Protobuf payload schema with rich-typed metrics, and a session-state model (NBIRTH/NDEATH/DBIRTH/DDEATH/NDATA/DDATA/NCMD/DCMD) on top of MQTT. Sparkplug is what most modern unified-namespace deployments standardise on; plain MQTT is supported here as a fallback for greenfield smart-factory and home-IoT devices that don't speak Sparkplug.

The MQTT bridge is positioned differently from the OPC UA and PLC4X bridges. It is read + advisory write only. Brokered, eventually-consistent, multi-client topics are the wrong substrate for safety-critical control writes — if you need autonomous writes, route them through OPC UA or PLC4X. The aggregator never publishes to a DCMD topic that triggers a real field actuator. It publishes only to a configurable advisory namespace (spBv1.0/<group>/DCMD/<edge>/<device>/advisory/...) that the operator HMI subscribes to. The HMI is the human-in-the-loop.

The structural ceiling is ADVISORY and it is enforced in two places: the MqttBridgeConfig compact constructor rejects any per-tag AUTONOMOUS promotion at load time, and the aggregator only differentiates SHADOW (drop) from ADVISORY (publish). There is no code path to AUTONOMOUS.

This manual explains how to install the dependency, configure the bridge, design ingress and advisory egress, run the result against a Mosquitto sandbox, and validate it end to end. It assumes a Java engineer who knows Maven, has read the universal bridge framework (00-FRAMEWORK.md), and has at least basic familiarity with MQTT (QoS, retain, wildcards) and the Sparkplug B session-state model.

# 2. Six rules you must internalise before writing any code

The universal six from 00-FRAMEWORK.md §0, specialised for MQTT/Sparkplug:

- No raw write to a field actuator from neuron output. The aggregator never publishes to a DCMD topic that controls a real device. Every approved command publishes to an advisory topic in the advisory_namespace (default advisory); the HMI consumes those topics and decides whether to translate them into real DCMDs.

- Interlocks have no direct authority on the MQTT side. Because the bridge does not control any actuator directly, an interlock signal does not have a fail-safe value to write through MQTT. Interlock authority lives in the bridge that owns the actuator (typically OPC UA or PLC4X). The MQTT bridge can carry interlock state as advisory AlarmSignals for HMI awareness.

- Operator override does not apply. The bridge does not write regulatory control; operator override is meaningful only for autonomous bridges. The MQTT aggregator ignores OperatorOverrideSignal by design.

- Every advisory publish produces an audit record — accepted, suppressed by SHADOW, clamped, dropped by the bounded advisory queue, or failed at the transport layer. Audits are written to a local JSONL file and, optionally, mirrored to a configurable MQTT audit topic.

- Quality propagates. Sparkplug metric attributes (is_historical, is_transient, is_null) become Quality.UNCERTAIN or Quality.BAD on the derived MeasurementSignal. A device that has just declared DDEATH flags all of its metrics as Quality.UNCERTAIN until the next BIRTH. Plain MQTT JSON payloads can optionally carry a quality field that the mapper honours.

- Wall-clock timestamps come from the source. Sparkplug metric timestamps win over System.currentTimeMillis(). Plain MQTT payloads can carry an optional ts ISO-8601 field that the mapper parses; absent that, the bridge stamps System.currentTimeMillis() at the moment the message is received.

# 3. Prerequisites

- JDK 17 or later. The integration uses Java records and sealed types in dependencies.

- Maven 3.9 or later.

- An MQTT broker reachable from the host — Mosquitto, EMQX, HiveMQ Community Edition, or any Sparkplug-aware broker (HiveMQ Edge, Cirrus Link MQTT Distributor).

- For Sparkplug deployments: a Sparkplug-aware edge emitter (a Tahu sample application, an Ignition Edge gateway, Node-RED with node-red-contrib-sparkplug-b, or a custom Java app using org.eclipse.tahu:tahu-core).

- The bridge core compiles without a broker on the classpath — all MQTT calls go through the MqttTransport seam. Tests inject InMemoryMqttTransport so the integration suite runs without docker.

# 4. Adding Jneopallium to your project

The MQTT bridge ships in the same worker artifact as the rest of Jneopallium:

<repositories> <repository> <id>gitlab-jneopallium</id> <url>https://gitlab.com/api/v4/projects/44434192/packages/maven</url> </repository> </repositories> <dependencies> <dependency> <groupId>com.rakovpublic.jneopallium</groupId> <artifactId>worker</artifactId> <version>1.0-SNAPSHOT</version> </dependency> </dependencies>

The worker already pulls Eclipse Tahu (org.eclipse.tahu:tahu-core) for Sparkplug B encoding and the HiveMQ MQTT client for the production transport. No extra dependencies are needed unless you choose to swap transports.

# 5. Architecture

The bridge introduces twelve boundary classes under worker.bridge.mqtt:

┌───────────────────────────┐ │ MqttClientService │ │ • owns connection state │ │ • BIRTH cache per device │ │ • per-binding queues │ │ • bounded advisory queue │ └─────┬──────────────┬──────┘ │ │ SUBSCRIBE │ │ PUBLISH ───────────────────────────────────────▶│ │◀────────── │ │ ┌──────────────────────┐ │ │ │ MQTT broker │ ─── NBIRTH/DBIRTH ────────────│ │ │ (HiveMQ / Mosquitto/ │ │ │ │ EMQX) │ ─── NDATA/DDATA ──────────────▶ │ │ │ │ │ │ │ ─── NDEATH/DDEATH ────────────▶ │ │ │ │ │ │ │ ◀── PUBLISH (advisory) │ │ │ │ advisory topic only │ │ └──────────────────────┘ │ │ ▼ ▼ ┌──────────────────┐ ┌────────────────────┐ │ MqttMetricInput │ │ MqttEventInput │ │ (IInitInput → │ │ (IInitInput → │ │ Measurement- │ │ AlarmSignal: │ │ Signal) │ │ DEVICE_OFFLINE, │ │ │ │ BRIDGE_RECONNECT) │ └──────────────────┘ └────────────────────┘ ▼ [Standard Jneopallium pipeline] ▼ MqttAdvisoryOutputAggregator │ ▼ MqttClientService.publish() (advisory topic only) │ ▼ MqttAuditOutput (JSONL + optional MQTT mirror)

Components:

| Class | Role |
| --- | --- |
| MqttBridgeConfig | YAML record (immutable). Enforces ADVISORY ceiling at construction. |
| MqttBridgeConfigLoader | Jackson YAML loader. FAIL_ON_UNKNOWN_PROPERTIES = true. |
| MqttTransport | Test-seam interface: connect / subscribe / publish / setHandler / isConnected / close. |
| DefaultMqttTransport | Production wiring on top of the HiveMQ MQTT client. |
| MqttTopicBinding | Resolved binding identity used at runtime by the service. |
| SparkplugMetricResolver | Maps spBv1.0/group/edge/device/metric topic+name to a binding id. Supports per-segment wildcards (+). |
| MqttSignalMapper | Pure functions: Sparkplug B Protobuf decode, plain-MQTT JSON decode via jsonPath, severity-map alarm extraction. |
| MqttClientService | Owns the transport, the per-binding ingress queues, the BIRTH cache, the bounded advisory publish queue, and the audit channel. AutoCloseable. |
| MqttMetricInput | IInitInput that drains a configurable subset of measurement queues each tick. |
| MqttEventInput | IInitInput that drains the bridge's lifecycle event queue (DEVICE_OFFLINE, BRIDGE_RECONNECTED). |
| MqttAdvisoryOutputAggregator | IOutputAggregator. Drops on SHADOW, clamps on ADVISORY, publishes to advisory topics, audits every decision. Does NOT extend AbstractBridgeOutputAggregator because the ADVISORY-only ceiling makes the autonomous-write algorithm moot. |
| MqttAuditOutput | Local JSONL append-only audit plus optional MQTT mirror channel. |

The aggregator deliberately diverges from AbstractBridgeOutputAggregator here, unlike PLC4X. The reason: the universal §2.2 algorithm models a SHADOW → ADVISORY → AUTONOMOUS ladder, but this bridge has no AUTONOMOUS rung. There are no interlock fail-safes to write, no operator override registry to consult, no rate-limiting because we are publishing recommendations not actuating values. The aggregator's job is narrower — drop on SHADOW, clamp on ADVISORY, publish, audit. Implementing only that keeps the code easy to audit; the structural ceiling is enforced by the config record, not by inheritance.

# 6. Configuration

Configuration is YAML, loaded once at startup through MqttBridgeConfigLoader.load(Path). Hot-reload is deliberately not supported. The loader sets FAIL_ON_UNKNOWN_PROPERTIES = true.

## 6.1 Schema

The configuration has nine top-level sections.

- connection — broker URL, client id, clean session, keep-alive, advisory queue size cap.

- security — auth type, credentials env vars, trust store, client certificate.

- sparkplug — enabled flag, group id, edge node id, advisory namespace.

- reads — list of ingress bindings, each either a Sparkplug metric address or a plain MQTT topic with a jsonPath.

- writes — list of advisory egress bindings. Each declares the advisory topic, the signal tag it consumes, clamp range, and QoS.

- audit — local JSONL file, optional MQTT audit topic, audit QoS.

- severityMap — Sparkplug alarm metric name → AlarmPriority.

- perTagSafetyMode — per-binding SHADOW / ADVISORY. AUTONOMOUS is rejected at load.

- tickInterval — worker tick cadence. Default PT0.25S.

## 6.2 Minimal example — read-only smoke test

The smallest useful configuration. Local Mosquitto broker, one Sparkplug ingress binding, no writes. Suitable as a first integration check.

connection: brokerUrl: "tcp://localhost:1883" clientId: "jneopallium-demo" keepAlive: "PT30S" advisoryQueueSize: 10000 security: type: None sparkplug: enabled: true groupId: "Plant1" edgeNodeId: "Jneopallium-Demo" advisoryNamespace: "advisory" reads: - bindingId: "TIC-101" sparkplugMetric: "Plant1/Edge-Reactor/Reactor1/temperature" signalTag: "PLANT.TIC101.PV" signalKind: MEASUREMENT writes: [] audit: localAuditFile: "/tmp/jneopallium-mqtt-audit.jsonl" tickInterval: "PT0.25S"

## 6.3 Production-like example — secure broker, advisory egress

A configuration shaped like a real deployment: TLS-secured broker, username/password from env, Sparkplug edge identity, a measurement plus an alarm metric on ingress, one advisory setpoint on egress, MQTT audit mirror.

connection: brokerUrl: "ssl://broker.plant.local:8883" clientId: "jneopallium-bridge-edge01" cleanSession: false keepAlive: "PT30S" advisoryQueueSize: 10000 security: type: UsernamePassword username: "jneopallium" passwordEnv: "MQTT_PASSWORD" trustStore: "/etc/jneopallium/mqtt-truststore.jks" sparkplug: enabled: true groupId: "Plant1" edgeNodeId: "Jneopallium-Edge-01" advisoryNamespace: "advisory" reads: - bindingId: "TIC-101" sparkplugMetric: "Plant1/Edge-Reactor/Reactor1/temperature" signalTag: "PLANT.TIC101.PV" signalKind: MEASUREMENT - bindingId: "VIBRATION-Z" sparkplugMetric: "Plant1/Edge-Pump/Pump3/vibration_z" signalTag: "PLANT.PUMP3.VIB_Z" signalKind: MEASUREMENT - bindingId: "HIGH-TEMP-ALARM" sparkplugMetric: "Plant1/Edge-Reactor/Reactor1/high_temp" signalTag: "PLANT.TIC101.HIGH" signalKind: ALARM # Plain-MQTT example (not Sparkplug) - bindingId: "AMBIENT-TEMP" plainMqttTopic: "sensors/ambient/temperature" jsonPath: "$.value" signalTag: "FACILITY.AMBIENT.PV" signalKind: MEASUREMENT writes: - bindingId: "TIC-101-ADV" advisoryTopic: "spBv1.0/Plant1/DCMD/Edge-Reactor/Reactor1/advisory/setpoint_temperature" signalTag: "PLANT.TIC101.SP" sparkplugMetric: "Plant1/Edge-Reactor/Reactor1/setpoint_temperature" minClampValue: 0.0 maxClampValue: 100.0 qos: 1 audit: localAuditFile: "/var/log/jneopallium/mqtt-audit.jsonl" mqttAuditTopic: "spBv1.0/Plant1/DCMD/Edge-Reactor/Jneopallium/audit" mqttAuditQos: 1 severityMap: high_temp: HIGH high_high_temp: URGENT perTagSafetyMode: TIC-101-ADV: ADVISORY tickInterval: "PT0.25S"

## 6.4 Field reference

connection.brokerUrl accepts tcp://, ssl://, ws://, and wss:// schemes. Production deployments must use ssl:// (or wss://) — plain tcp:// is fine for laboratory smoke testing only. The bridge does not enforce this; site security review does.

connection.advisoryQueueSize caps the in-memory publish queue. When the broker is slow or unreachable, advisory publishes accumulate; once the cap is reached the oldest entry is dropped with verdict=FAILED reason=ADVISORY_QUEUE_FULL. Default 10 000 entries. Tune up for high-frequency advisory flows; tune down on memory-constrained edges.

security.type is one of None, UsernamePassword, or ClientCertificate. passwordEnv and clientKeyEnv are the names of environment variables — never embed the secret in the YAML file. The config loader only reads the name; the transport reads the variable at start-up.

sparkplug.enabled: true activates Sparkplug B decoding for any sparkplugMetric binding. With it false, only plain-MQTT bindings work. sparkplug.groupId and sparkplug.edgeNodeId define the bridge's own Sparkplug identity for any advisory it publishes — the bridge does not announce itself as an Edge Node (no NBIRTH from the bridge), it just uses the identity to construct advisory topics.

sparkplug.advisoryNamespace is the segment that distinguishes advisory DCMD topics from production DCMD topics. Default advisory. Choose a value the HMI agrees on; the bridge does not enforce a particular convention.

reads[].sparkplugMetric is the four-segment path group/edge/device/metric. Each segment may be + for a single-level wildcard. Example: Plant1/Edge-Pump/+/temperature_* matches every pump's temperature_* metric in Plant1. + wildcards are matched via SparkplugMetricResolver; the resolver maps the actual (group, edge, device, metric) of an inbound message back to the binding id for queueing.

reads[].plainMqttTopic is a plain MQTT topic with optional MQTT wildcards (+, #). Plain-MQTT bindings require a jsonPath (a small dotted/index expression like $.value, $.temperature.current, $.sensors[0].pv); the mapper applies it to the JSON payload.

reads[].signalKind is MEASUREMENT (numeric — produces MeasurementSignal) or ALARM (boolean — produces AlarmSignal if the metric name appears in severityMap). Default MEASUREMENT.

writes[].advisoryTopic is the full MQTT topic that the aggregator publishes to. There is no constraint that it sits under the advisory namespace; the bridge trusts the YAML. By convention, every entry should include /<advisory_namespace>/ in the path.

writes[].sparkplugMetric, when set, makes the aggregator emit a Sparkplug B-encoded payload (one metric named at this path). When null, the payload is plain JSON. Sparkplug encoding requires sparkplug.enabled: true.

writes[].minClampValue / maxClampValue are the clamp range. A value outside the range is moved to the nearest bound and audited with reason=CLAMPED_LOW or reason=CLAMPED_HIGH. null disables that side of the clamp.

writes[].qos is the MQTT publish QoS (0, 1, or 2). Default 1. The bridge ignores invalid values silently and falls back to 1.

severityMap keys are Sparkplug metric names (the leaf segment, not the full path); values are JOURNAL, LOW, HIGH, or URGENT. The mapper checks every incoming boolean metric against the map; a true value whose name appears emits one AlarmSignal with the configured priority.

perTagSafetyMode keys are bindingIds. The only legal values are SHADOW and ADVISORY. AUTONOMOUS is rejected with IllegalArgumentException at load time — that's the structural ceiling. Unlisted bindings default to SHADOW.

# 7. Sparkplug session state

A Sparkplug-aware deployment depends on the session-state model. The bridge tracks it as follows:

- `NBIRTH` — Edge Node announces itself. The bridge does not currently produce metric metadata signals; the BIRTH establishes the edge as alive in the internal cache. Subsequent NDATA messages are valid.

- `DBIRTH` — Device announces itself under an edge node. Same treatment — registers the device alive; subsequent DDATA is valid; metric metadata (type, alias) is cached so future DDATA can be decoded against it.

- `NDATA` / `DDATA` — Data updates. Each numeric metric matching a binding becomes a MeasurementSignal with Quality.GOOD (or UNCERTAIN if the metric carries is_historical=true). Each boolean metric matching a binding and present in severityMap becomes an AlarmSignal.

- `DDEATH` — Device dropped. All cached metrics for that device flag Quality.UNCERTAIN until the next DBIRTH. The bridge emits one AlarmSignal(LOW, DEVICE_OFFLINE) to the event queue.

- `NDEATH` — Edge Node dropped. All devices under that edge are treated as if DDEATH'd.

- `NCMD` / `DCMD` — Command topics. The bridge does not subscribe to these — they are operator/controller-originating writes. The bridge publishes to advisory variants only.

The bridge does not currently surface DBIRTH/DDEATH events as standalone signals beyond the DEVICE_OFFLINE alarm. Downstream neurons that care about device lifecycle can subscribe to the event queue via MqttEventInput.

# 8. Wiring the Java side

The bridge does not ship a main class — it is a set of components you wire into your worker bootstrap. A minimal driver looks like this:

package com.example.plant; import com.rakovpublic.jneuropallium.worker.bridge.common.AbstractBridgeAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.DefaultMqttTransport; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttAdvisoryOutputAggregator; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttBridgeConfig; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttBridgeConfigLoader; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttClientService; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttEventInput; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttMetricInput; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttSignalMapper; import com.rakovpublic.jneuropallium.worker.bridge.mqtt.MqttTransport; import java.nio.file.Path; import java.util.List; public final class MqttBootstrap { public static void main(String[] args) throws Exception { // 1. Load YAML (FAIL_ON_UNKNOWN_PROPERTIES = true, AUTONOMOUS rejected). MqttBridgeConfig cfg = MqttBridgeConfigLoader.load(Path.of("mqtt-bridge.yml")); // 2. Open the audit sink. try (AbstractBridgeAuditOutput audit = new MqttAuditOutput( Path.of(cfg.audit().localAuditFile()))) { // 3. Build the transport. In production this is DefaultMqttTransport // over the HiveMQ client; tests inject InMemoryMqttTransport. MqttTransport transport = new DefaultMqttTransport(cfg); // 4. Build the signal mapper and the client service. MqttSignalMapper mapper = new MqttSignalMapper(cfg); MqttClientService svc = new MqttClientService(cfg, transport, mapper, audit); // 5. Start the connection and subscribe to every read binding. // Idempotent; subsequent reconnects re-subscribe automatically. svc.start(); try { // 6. Build the inputs. Pass the list of binding ids whose // signals you want to drain on each tick. List<String> measBindings = cfg.reads().stream() .filter(r -> r.signalKind() == MqttBridgeConfig.ReadSignalKind.MEASUREMENT) .map(MqttBridgeConfig.ReadBindingConfig::bindingId) .toList(); MqttMetricInput metricIn = new MqttMetricInput("mqtt-meas", svc, measBindings); MqttEventInput eventIn = new MqttEventInput("mqtt-events", svc); // 7. Build the aggregator. SHADOW/ADVISORY only; AUTONOMOUS // was already rejected by the config record. MqttAdvisoryOutputAggregator agg = new MqttAdvisoryOutputAggregator(svc, audit); // 8. Register with the worker. worker.registerInput(metricIn); worker.registerInput(eventIn); worker.registerOutputAggregator(agg); worker.run(); } finally { svc.close(); } } } }

Three things worth highlighting. First, `svc.start()` does the subscription, not connect(). The client service owns the transport's connect call, subscribes to every binding's topic, and registers the message handler in one method. This is the only method-call you need to make to bring up ingress. Second, the same audit sink is shared across the service and the aggregator. Ingress decode failures (a Sparkplug payload that fails to parse) and egress decisions (every advisory publish, every SHADOW drop) land in one JSONL file. Third, `AutoCloseable` is non-negotiable — leaving the HiveMQ client open after process exit can leave a dangling broker session that prevents the next connect for the keep-alive interval. Always wrap in try-with-resources or call svc.close() in a finally.

# 9. Modeling guidance

## 9.1 Choosing what to bind on ingress

A useful test: bind every Sparkplug metric you want a neuron to consume, not every metric the broker carries. Sparkplug deployments often expose hundreds of metrics per edge node; subscribing to all of them at the bridge wastes broker bandwidth and JVM heap. The MQTT broker handles the multiplexing — your binding list is the bridge's contract with the rest of the worker.

Per-segment wildcards (+) are useful when the device count is large but homogeneous: Plant1/Edge-Pump/+/vibration_z binds every pump's vibration metric to one binding id. The SparkplugMetricResolver resolves the actual (group, edge, device) of an inbound message back to a binding id for queueing; the resulting MeasurementSignal has the binding's signalTag plus the device identity embedded as inputName so downstream neurons can disambiguate.

For plain MQTT, prefer a structured JSON payload with an explicit value and optional ts / quality fields over a bare scalar. The jsonPath expression supports dotted access ($.value), indexed access ($.sensors[0].pv), and nested objects. Arrays beyond the first index require explicit indexing — there is no * selector.

## 9.2 Adding a new sensor channel

The three-step procedure for ingress:

- YAML: add a reads: entry with a unique bindingId, the Sparkplug metric path or plain MQTT topic, a stable signalTag following your ISA-95 convention, and (for plain MQTT) a jsonPath.

- Java neurons (optional): add a neuron that consumes the MeasurementSignal with the configured signalTag. If you omit this step, the signal flows in but goes nowhere downstream.

- Restart the worker. The bridge subscribes to new topics at start-up; YAML changes are not picked up live.

The bridge subscribes per binding. Adding 50 bindings against the same edge node creates 50 subscriptions; modern brokers handle thousands of subscriptions per client without trouble.

## 9.3 Adding a new advisory write

The procedure:

- YAML — write binding. Add the writes: entry. Pick advisoryTopic so the segment /<advisory_namespace>/ is present — the HMI will subscribe to that namespace. Set minClampValue and maxClampValue to plausible engineering limits even though they are not safety limits — the HMI is the safety layer, but reasonable clamps avoid sending 9999999 through a typo.

- YAML — start in SHADOW. perTagSafetyMode: { TIC-101-ADV: SHADOW }. Every advisory will be audited as REJECTED reason=SHADOW_MODE, recording what the bridge would have published.

- Run. Observe the audit JSONL. Confirm the proposed advisory values are sensible.

- Promote to ADVISORY. Change YAML, restart. Advisories now publish. The HMI sees them; the operator decides whether to translate any into a real DCMD.

- Stay in ADVISORY. There is no AUTONOMOUS rung on this bridge. If the deployment needs autonomous control, that's a different bridge — OPC UA or PLC4X — with its own write binding to the actuator.

## 9.4 Alarm handling

The bridge produces two distinct alarm channels:

- Sparkplug alarm metrics — boolean metrics whose name appears in severityMap. When a DDATA arrives carrying such a metric set to true, the mapper emits one AlarmSignal with the configured AlarmPriority and a tag derived from the metric name. Stateful: an alarm metric remains "active" until the bridge sees a DDATA with the same metric set to false.

- Lifecycle events — bridge-level alarms: DEVICE_OFFLINE (LOW) on DDEATH, BRIDGE_RECONNECTED (LOW) after a transport reconnect. These come from MqttEventInput, not from MqttMetricInput.

Wire both inputs to your worker; alarms from either flow into the same IndustrialAlarmNeuron if you have one. The two channels are kept separate at the input layer to make the source easy to attribute.

# 10. The advisory egress algorithm

When the worker calls MqttAdvisoryOutputAggregator.save(List<IResult>, long, long, IContext), the aggregator does the following in this order:

- For every IResult whose signal is a SetpointSignal:

- Resolve the binding by tag via MqttClientService.findWriteByTag(tag). If null → audit REJECTED reason=UNBOUND_SIGNAL_TYPE. No publish.

- Resolve the per-binding safety mode. If SHADOW → audit REJECTED reason=SHADOW_MODE. No publish.

- Clamp the value to [minClampValue, maxClampValue]. If a clamp fired, the audit reason will be CLAMPED_LOW or CLAMPED_HIGH.

- Encode the value: Sparkplug B Protobuf payload if writes[].sparkplugMetric is set, otherwise a plain JSON object {"tag": "...", "value": 47.3, "ts": "2026-05-14T12:00:00Z"}.

- Enqueue on the bounded advisory queue. If full → audit FAILED reason=ADVISORY_QUEUE_FULL. No publish.

- Publish via MqttTransport.publish(advisoryTopic, payload, qos, false). On success → audit APPLIED reason=ADVISORY_PUBLISHED (with the modify reason from clamp if applicable). On transport failure → audit FAILED reason=PUBLISH_ERROR:<detail>.

- For every IResult whose signal is an ActuatorCommandSignal:

- Only commands with execute=true are considered — otherwise audit REJECTED reason=EXECUTE_FALSE.

- Same algorithm as SetpointSignal thereafter.

- For every IResult whose signal is a TransparencyLogSignal:

- If audit.mqttAuditTopic is configured, encode as JSON and publish there with audit.mqttAuditQos. Best-effort; failures are logged but not re-audited (the SOC's MQTT mirror is the audit-of-the-audit and the local JSONL is the source of truth).

- Always also write the local JSONL record regardless of MQTT mirror result.

There are no interlocks, no operator override registry, no rate-limit, no diff-suppress on this bridge. The simpler algorithm reflects the simpler authority — the bridge is publishing recommendations to a human consumer, not driving an actuator.

# 11. Audit and transparency

The bridge writes one JSONL record per decision, conforming to the framework's BridgeAuditRecord schema (00-FRAMEWORK.md §4):

{ "ts": 1740000000000, "run": 12345, "bridge": "mqtt", "verdict": "APPLIED", "tag": "PLANT.TIC101.SP", "proposed": 105.0, "effective": 100.0, "reason": "CLAMPED_HIGH", "safetyMode": "ADVISORY", "evidenceNeurons": ["Setpoint-12"] }

Records you will see:

| `verdict` | `reason` | Triggered by |
| --- | --- | --- |
| APPLIED | ADVISORY_PUBLISHED | Advisory published to broker successfully. |
| APPLIED | CLAMPED_HIGH / CLAMPED_LOW | Value clamped before publish; publish succeeded. |
| REJECTED | SHADOW_MODE | Binding in SHADOW mode; no publish. |
| REJECTED | EXECUTE_FALSE | ActuatorCommandSignal with execute=false. |
| REJECTED | UNBOUND_SIGNAL_TYPE | Approved signal's tag has no writes: entry. |
| FAILED | ADVISORY_QUEUE_FULL | Advisory queue at advisoryQueueSize; oldest dropped. |
| FAILED | PUBLISH_ERROR:<detail> | Transport error during publish. |
| FAILED | DECODER_ERROR:<detail> | Inbound Sparkplug or JSON payload failed to decode. |
| APPLIED | BRIDGE_RECONNECTED | Transport reconnected; emitted to the event channel. |

Audit-write failures (disk full, permissions revoked) are isolated per framework S5: the bridge keeps running in degraded mode and exposes MqttAuditOutput.isDegraded() == true for monitoring.

When audit.mqttAuditTopic is set, every record is also published to the broker with the configured QoS. The mirror is best-effort; broker unavailability does not stall the local audit.

# 12. Operating procedures

## 12.1 Smoke test against a local Mosquitto

The fastest path to a working bridge:

- Start a broker: docker run --rm -p 1883:1883 eclipse-mosquitto:2 mosquitto -c /mosquitto-no-auth.conf.

- Copy the §6.2 minimal config to mqtt-bridge.yml. Adjust brokerUrl to tcp://localhost:1883.

- Run a MqttBootstrap per §8.

- From a separate shell, publish a Sparkplug DBIRTH then a DDATA. The simplest way is using a Tahu sample app or a Node-RED flow with node-red-contrib-sparkplug-b. For a quick test, use mosquitto_pub to publish a JSON payload to a plain-MQTT binding instead:

``bash    mosquitto_pub -h localhost -t 'sensors/ambient/temperature' \                  -m '{"value": 23.5, "ts": "2026-05-14T12:00:00Z"}'    ``

- The next metricIn.readSignals() call returns a MeasurementSignal(value=23.5).

## 12.2 Verifying device offline propagation

Sparkplug session state is the trickiest part to verify. The procedure:

- Start the bridge and a Sparkplug emitter. Confirm NBIRTH, DBIRTH, then DDATA produce MeasurementSignals.

- Kill the emitter. After the keep-alive timeout the broker publishes NDEATH/DDEATH (the last-will mechanism Sparkplug specifies).

- The bridge emits AlarmSignal(LOW, DEVICE_OFFLINE) on the event channel.

- Continue running. The broker may stay quiet for a while.

- Restart the emitter. NBIRTH/DBIRTH follow; the next DDATA produces signals again.

If step 3 does not happen, check the emitter's last-will configuration. Sparkplug requires a specific last-will format; some libraries do not set it by default.

## 12.3 Promoting a binding from SHADOW to ADVISORY

- Run for an observation period in SHADOW. Inspect the audit JSONL: every advisory should appear with verdict=REJECTED reason=SHADOW_MODE. Check the values are sensible.

- Confirm with the operator and the HMI team that the advisory topic and payload schema match their subscription.

- Stop the worker. Edit YAML: change the binding's perTagSafetyMode to ADVISORY.

- Restart. Subsequent advisories publish. The HMI starts receiving recommendations.

There is no ADVISORY → AUTONOMOUS promotion on this bridge. That's by design.

## 12.4 Diagnosing a queued-but-unpublished advisory

A verdict=FAILED reason=ADVISORY_QUEUE_FULL audit record means publish backlog hit the cap. Causes, in declining frequency:

- Broker offline. Check MqttTransport.isConnected(); reconnect is automatic but may take seconds.

- Broker slow. Production brokers occasionally throttle; check broker-side metrics.

- Advisory queue cap too low for the model's emission rate. Tune connection.advisoryQueueSize up.

The bridge does not retry dropped advisories. By the time the queue is recovering the data is already stale; emitting yesterday's advice would confuse the HMI.

# 13. Limitations and future work

- No `AUTONOMOUS` ceiling. Structural. If the deployment ever needs autonomous control through MQTT, the right answer is a separately scoped, separately audited bridge with its own structural defences — not a flag flip on this one.

- No structural device-identity validation. A misconfigured shadow client with broker credentials can publish to the same advisory topic the bridge writes to. The bridge audits its own writes but cannot prevent parallel writers. Site-level MQTT ACLs are the only defence.

- Eventual consistency. A subscriber may not receive the latest advisory for hundreds of milliseconds under load. Fine for HMI annotations and dashboards, unsafe for closed-loop control. This is an MQTT property, not a bridge limitation.

- No quality-from-stale-data. A binding whose last update arrived an hour ago still emits Quality.GOOD. If you need staleness-as-uncertainty, wrap MqttMetricInput with a neuron that checks the signal timestamp.

- Plain-MQTT mode is minimal. Only one jsonPath expression per binding. For richer mappings (multiple fields per payload, schema validation), a custom MqttSignalMapper subclass is the extension point.

# 14. Regulatory posture

| Aspect | Posture |
| --- | --- |
| Safety ceiling | ADVISORY. Structural — AUTONOMOUS rejected at config load. |
| Default per-binding mode | SHADOW. Every binding must be explicitly promoted to ADVISORY. |
| Interlock authority | None on the MQTT side. Use OPC UA or PLC4X for actuator interlocks. |
| Operator override | Not applicable; advisory bridges do not write regulatory control. |
| Audit | One JSONL line per decision plus optional MQTT mirror topic. |
| MoC | Hot-reload not supported. YAML changes follow site MoC. |
| Broker trust | The broker is the trust boundary. Use TLS in production; use broker ACLs to restrict who can write to advisory topics. |

For closed-loop control of field actuators, route through the OPC UA or PLC4X bridges, both of which are AUTONOMOUS-capable per-loop with the full clamp / rate-limit / interlock / override ladder enforced before any write. The MQTT bridge is the recommendation channel from the cognitive layer to the human operator; the HMI is the gatekeeper.

# 15. References

- 02-MQTT-SPARKPLUG.md — the bridge specification.

- 00-FRAMEWORK.md — the universal bridge contract.

- docs/mqtt-bridge.md — domain-context companion in the repo.

- MQTT — https://mqtt.org/.

- Sparkplug B specification — https://sparkplug.eclipse.org/specification/.

- Eclipse Tahu (Sparkplug B Java implementation) — https://github.com/eclipse-tahu/tahu.

- HiveMQ MQTT client — https://github.com/hivemq/hivemq-mqtt-client.

- Mosquitto broker — https://mosquitto.org/.

- Rakovskyi, D. (2024). Framework for Natural Neuron Network Modeling: The Jneopallium Approach. IJSR 13(7).