Building Models with Apache PLC4X and Jneopallium

User Manual

Legacy fieldbus integration · typed-signal safety · auditable control

Version: 1.0-SNAPSHOT

License: BSD 3-Clause

Author: Dmytro Rakovskyi

# 1. Introduction

Most factories run a long tail of legacy fieldbus controllers — Siemens S7, Modbus TCP, EtherNet/IP, Beckhoff ADS, Allen-Bradley, Profinet — that pre-date modelled OPC UA servers. Where the OPC UA bridge connects Jneopallium to standards-compliant industrial systems, the PLC4X bridge is its sibling for everything that does not yet speak OPC UA. [Apache PLC4X](https://plc4x.apache.org/) wraps each of these protocols behind one Java PlcConnection API: open by connection string, then read or write protocol-native field expressions.

The PLC4X bridge reuses the same MeasurementSignal / AlarmSignal / ActuatorCommandSignal types and the same AbstractBridgeOutputAggregator-based safety algorithm as the OPC UA bridge — interlocks → operator override → clamp → rate-limit → diff-suppress → audit. The same JSONL audit schema, the same phased rollout (SHADOW → ADVISORY → per-loop AUTONOMOUS), the same worker.bridge.common shared base. The only material difference is the transport: PLC4X drivers are mostly poll-based, so this bridge runs one polling thread per PlcConnection instead of an OPC UA subscription.

This manual explains how to install the dependency, configure the bridge, model your plant in Jneopallium primitives, run the resulting model against a real PLC or a stub, and validate it end to end. It assumes a Java engineer who knows Maven, has read the universal bridge framework (00-FRAMEWORK.md), and has at least basic familiarity with industrial process control (PID loops, setpoints, interlocks) and one of the fieldbus protocols PLC4X supports.

# 2. Six rules you must internalise before writing any code

The bridge enforces six non-negotiable safety invariants — the universal six from 00-FRAMEWORK.md §0. Read them, then read them again. Code that violates any of them must not pass review.

- No raw write to a field actuator from neuron output. Every write goes through the chain PlanningNeuron → SafetyGateNeuron → SafetyMode check → OperatorOverrideSignal check → Plc4xCommandOutputAggregator. The aggregator rejects any ActuatorCommandSignal whose execute=false or whose loop is in shadow mode.

- Interlocks have direct authority. When InterlockSignal.tripped == true, the aggregator writes the loop to its failSafeValue regardless of anything else in the same tick. This is the only permitted bypass.

- Operator override wins for regulatory control. While an OperatorOverrideSignal is active for a tag, no neuron-derived setpoint is applied to that tag. Override does not cancel an interlock.

- Every write produces an audit record — accepted, suppressed, clamped, rate-limited, or rejected. Audits are written to a local JSONL file conforming to the framework's BridgeAuditRecord schema.

- Quality propagates. A MeasurementSignal derived from a PLC4X PlcResponseCode that is not OK is emitted with Quality.UNCERTAIN or Quality.BAD — never silently promoted to GOOD.

- Wall-clock timestamps come from the source. The bridge tags each measurement with System.currentTimeMillis() at the moment the cache is read, because PLC4X drivers do not surface a source-system timestamp; that's the closest the protocol allows. When you wire a Plc4x adapter against a driver that does expose a server timestamp (some newer S7 drivers do), surface it through a custom Plc4xDriver implementation that returns it via the ReadResponse.

# 3. Prerequisites

- JDK 17 or later. The bridge uses Java records and sealed types in dependencies.

- Maven 3.9 or later.

- Network access to your PLC. For a first-run smoke test with no real hardware, the bridge ships with a StubPlc4xDriver that simulates s7:// and modbus-tcp:// schemes in-memory.

- For real-hardware verification: a PLC simulator (OpenPLC, ModbusPal, snap7-server) or an actual PLC on the network.

# 4. Adding Jneopallium to your project

The PLC4X bridge ships in the same worker artifact as the rest of Jneopallium:

<repositories> <repository> <id>gitlab-jneopallium</id> <url>https://gitlab.com/api/v4/projects/44434192/packages/maven</url> </repository> </repositories> <dependencies> <dependency> <groupId>com.rakovpublic.jneopallium</groupId> <artifactId>worker</artifactId> <version>1.0-SNAPSHOT</version> </dependency> </dependencies>

The bridge core compiles without the real PLC4X jars on the classpath — all interactions go through the Plc4xDriver seam. A production deployment adds the PLC4X dependencies itself and supplies a thin Plc4xDriver adapter:

<dependency> <groupId>org.apache.plc4x</groupId> <artifactId>plc4j-api</artifactId> <version>0.12.0</version> </dependency> <dependency> <groupId>org.apache.plc4x</groupId> <artifactId>plc4j-connection-pool</artifactId> <version>0.12.0</version> </dependency> <!-- Pull only the drivers you need --> <dependency> <groupId>org.apache.plc4x</groupId> <artifactId>plc4j-driver-s7</artifactId> <version>0.12.0</version> </dependency> <dependency> <groupId>org.apache.plc4x</groupId> <artifactId>plc4j-driver-modbus</artifactId> <version>0.12.0</version> </dependency>

Verify the latest PLC4X release before merging — PLC4X moves on its own cadence and minor versions occasionally tighten driver behaviour.

# 5. Architecture

The bridge introduces nine boundary classes under worker.bridge.plc4x:

polls (per-binding) ┌──────────────────────┐ ────────────────────────────▶ ┌────────────────────────┐ │ Legacy PLC fleet: │ │ Plc4xClientService │ │ • Siemens S7-1500 │ ◀──── PlcWriteRequest ──────── │ • one scheduler / │ │ • Modbus TCP │ │ PlcConnection │ │ • EtherNet/IP │ │ • latest-value cache │ │ • Beckhoff ADS │ │ • address validation │ └──────────────────────┘ └─────┬─────────────┬────┘ │ │ ┌──────────────▼─┐ ┌───────▼──────┐ │ Plc4xMeasure- │ │ Plc4xEvent- │ │ mentInput │ │ Input │ │ (IInitInput → │ │ (IInitInput │ │ Measurement- │ │ → Alarm- │ │ Signal) │ │ Signal) │ └────────────────┘ └──────────────┘ ▼ [Standard Jneopallium pipeline] ▼ Plc4xCommandOutputAggregator (extends AbstractBridgeOutputAggregator) │ ▼ Plc4xClientService.write() → PlcWriteRequest → audit BridgeAuditRecord

The bridge core does not import the PLC4X jars. Every protocol interaction goes through the Plc4xDriver interface — an open / validate / read / write / close seam. The repo ships two implementations: StubPlc4xDriver (in src/test/java, the in-memory simulator used by every test in Plc4xBridgeIntegrationTest) and a production adapter you supply. This keeps the bridge testable offline and lets you swap drivers without touching the safety algorithm.

Components:

| Class | Role |
| --- | --- |
| Plc4xConfig | YAML record (immutable). Eight nested records for the seven YAML sections plus the read/write/event/connection/audit shape. |
| Plc4xConfigLoader | Jackson YAML loader. FAIL_ON_UNKNOWN_PROPERTIES = true. |
| Plc4xDriver | Open/validate/read/write/close interface. Production adapter implements this against org.apache.plc4x.java.api.PlcDriverManager. |
| Plc4xResponseCode | Bridge-local mirror of PLC4X's PlcResponseCode. Decoupling the bridge from the PLC4X jar. |
| Plc4xSignalMapper | Pure functions. PlcResponseCode → Quality, value-to-double coercion, alarm-word bit decoding against the severity map. |
| Plc4xClientService | Per-connection driver lifecycle, per-binding ScheduledExecutorService, latest-value ConcurrentHashMap, address validation. |
| Plc4xMeasurementInput | IInitInput that emits one MeasurementSignal per read binding from the latest cache. |
| Plc4xEventInput | IInitInput that polls alarm-word bindings and emits one AlarmSignal per matching severity-map bit. |
| Plc4xCommandOutputAggregator | IOutputAggregator extending AbstractBridgeOutputAggregator. Implements only issueWrite(binding, value); every other safety concern (interlocks, override, clamp, rate-limit, diff-suppress, audit) is inherited from the universal base. |
| Plc4xAuditOutput | Local JSONL append-only audit, conforming to BridgeAuditRecord schema. |

The aggregator extending the universal base is the single most important design choice in this bridge. Every protocol-specific concern compresses down to one method — BridgeWriteResult issueWrite(Plc4xFieldBinding, double) — that delegates to Plc4xClientService.write() and maps the response code to BridgeWriteResult.ok() or BridgeWriteResult.failed("PLC4X:" + code.name()). Everything else is the universal §2.2 algorithm.

# 6. Configuration

Configuration is YAML, loaded once at startup through Plc4xConfigLoader.load(Path). Hot-reload is deliberately not supported — bridge changes follow your site's Management of Change procedure with a controlled restart. The loader sets FAIL_ON_UNKNOWN_PROPERTIES = true, so a typo in a key is a startup failure, not a silent fallback.

## 6.1 Schema

The configuration has seven top-level sections.

- connections — list of PlcConnections. Each has an id, a PLC4X connectionString, an optional requestTimeout, and an optional keepAliveInterval.

- reads — list of polled scalar tags producing MeasurementSignal.

- writes — list of writable tags driven by ActuatorCommandSignal. The full clamp / rate-limit / fail-safe ladder lives here.

- events — list of polled alarm words. Each polled value is bit-decoded against a severityMap and produces one AlarmSignal per matching bit.

- audit — local JSONL audit file path plus a writeRejectedToAudit toggle.

- perTagSafetyMode — per-binding-id SHADOW / ADVISORY / AUTONOMOUS. The default for any unlisted binding is SHADOW.

- tickInterval — worker tick cadence. Default PT0.25S.

## 6.2 Minimal example — read-only smoke test

The smallest useful configuration. One Modbus connection, one polled coil, no writes. Suitable as a first integration check.

connections: - id: "OPENPLC" connectionString: "modbus-tcp://127.0.0.1:502?unit-identifier=1" requestTimeout: "PT2S" reads: - bindingId: "DEMO-COIL" connectionId: "OPENPLC" fieldAddress: "coil:0" signalTag: "DEMO.COIL.0" pollIntervalMs: 500 writes: [] events: [] audit: localAuditFile: "/tmp/jneopallium-plc4x-audit.jsonl" writeRejectedToAudit: true perTagSafetyMode: DEMO-COIL: SHADOW tickInterval: "PT0.5S"

## 6.3 Production-like example — multi-driver, with writes

A production-shaped configuration: two connections (Siemens S7 line A, Modbus TCP pump house), a polled measurement, a writable setpoint with full safety ladder, and an alarm-word event binding with a severity map.

connections: - id: "S7-LINE-A" connectionString: "s7://10.10.0.1?remote-rack=0&remote-slot=1" requestTimeout: "PT5S" keepAliveInterval: "PT10S" - id: "MODBUS-PUMPHOUSE" connectionString: "modbus-tcp://10.10.0.2:502?unit-identifier=1" requestTimeout: "PT2S" reads: - bindingId: "TIC-101" connectionId: "S7-LINE-A" fieldAddress: "%DB1.DBD0:REAL" signalTag: "PLANT.TIC101.PV" pollIntervalMs: 250 - bindingId: "PUMP-RUN" connectionId: "MODBUS-PUMPHOUSE" fieldAddress: "coil:0" signalTag: "PLANT.PUMP01.STATE" pollIntervalMs: 500 writes: - bindingId: "TIC-101" connectionId: "S7-LINE-A" fieldAddress: "%DB1.DBD8:REAL" signalTag: "PLANT.TIC101.SP" failSafeValue: 0.0 rampRateMaxPerSec: 2.0 minClampValue: 0.0 maxClampValue: 100.0 events: - bindingId: "TROUBLE-ALARMS" connectionId: "S7-LINE-A" fieldAddress: "%DB100.DBW0:WORD" signalTag: "PLANT.LINE_A.TROUBLE" pollIntervalMs: 1000 severityMap: "0x0001": "LOW" "0x0002": "HIGH" "0x0010": "CRITICAL" audit: localAuditFile: "/var/log/jneopallium/plc4x-audit.jsonl" writeRejectedToAudit: true perTagSafetyMode: TIC-101: SHADOW tickInterval: "PT0.25S"

## 6.4 Field reference

connections[].id is the stable identifier read- and write-bindings reference. connections[].connectionString is the verbatim PLC4X connection string for the driver: s7://10.10.0.1?remote-rack=0&remote-slot=1, modbus-tcp://10.10.0.2:502?unit-identifier=1, ads://10.10.0.3.1.1:851, etc. The bridge does not parse it — PLC4X does.

reads[].fieldAddress is the verbatim PLC4X field expression for the driver. Examples: %DB1.DBD0:REAL for S7 data block 1, dword offset 0, REAL type; coil:0 for Modbus coil 0; holding-register:100:UINT16 for a Modbus holding register. Each address is validated at startup via Plc4xDriver.validate() — a typo'd address fails the bridge before any signal flows.

reads[].pollIntervalMs is the per-binding poll cadence. The bridge enforces a single-thread scheduler per connection, so one slow binding does not delay another on the same controller. A reasonable production floor is 250 ms; small S7 CPUs degrade above ~5 Hz aggregate poll rate, so consider that when adding bindings to one connection.

writes[].failSafeValue is what the aggregator writes when an InterlockSignal for the same loop trips. Mandatory by safety policy on any AUTONOMOUS-promoted loop.

writes[].rampRateMaxPerSec is the maximum change per second the aggregator will permit. A larger requested step is rate-limited (not rejected) and audited with reason=RATE_LIMITED. null disables rate-limiting.

writes[].minClampValue / maxClampValue are hard clamps. A value outside the range is moved to the nearest bound and audited with reason=CLAMPED_LOW or reason=CLAMPED_HIGH. null disables that side of the clamp.

events[].severityMap keys are bit masks parsed as integers — hex prefix 0x or decimal. For each polled value, the mapper checks each map entry: when (value & mask) == mask, it emits one AlarmSignal with the configured AlarmPriority. CRITICAL and URGENT both map to AlarmPriority.URGENT; HIGH to HIGH; LOW to LOW; anything else to JOURNAL.

perTagSafetyMode keys are bindingIds, not signal tags — promotion happens at the binding level. Unlisted bindings default to SHADOW. The full ladder: SHADOW (every command is audited as REJECTED reason=SHADOW_MODE with no protocol write), ADVISORY (only commands with execute=true AND operator confirmation are applied; without confirmation, REJECTED reason=ADVISORY_HOLD), AUTONOMOUS (commands with execute=true are applied subject to the full safety ladder).

# 7. Wiring the Java side

The bridge does not ship a main class — it is a set of components you wire into your worker bootstrap. A minimal driver looks like the following.

package com.example.plant; import com.rakovpublic.jneuropallium.worker.bridge.common.AbstractBridgeAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xClientService; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xCommandOutputAggregator; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xConfig; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xConfigLoader; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xDriver; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xEventInput; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.Plc4xMeasurementInput; import java.nio.file.Paths; public final class Plc4xBootstrap { public static void main(String[] args) throws Exception { // 1. Load YAML (FAIL_ON_UNKNOWN_PROPERTIES = true). Plc4xConfig cfg = Plc4xConfigLoader.load(Paths.get("plc4x-bridge.yml")); // 2. Open the audit sink. try (AbstractBridgeAuditOutput audit = new Plc4xAuditOutput( Paths.get(cfg.audit().localAuditFile()))) { // 3. Construct a Plc4xDriver. In production, this is a thin // adapter over org.apache.plc4x.java.api.PlcDriverManager // that you supply. For smoke tests, the repo provides // StubPlc4xDriver in src/test/java. Plc4xDriver driver = new Plc4xRealDriver(); // your adapter try (Plc4xClientService svc = new Plc4xClientService(driver, cfg)) { // 4. Open all connections, validate every field address. // Fail-fast on a bad address or missing driver. svc.connect(); // 5. Build the measurement input (one MeasurementSignal per // read binding, drawn from the latest cache each tick). Plc4xMeasurementInput measIn = new Plc4xMeasurementInput( "plc4x-meas", svc, cfg.reads()); // 6. Build the event input (one AlarmSignal per matching bit // on each alarm-word binding). Plc4xEventInput eventIn = new Plc4xEventInput( "plc4x-events", svc, cfg.events()); // 7. Build the aggregator. The universal §2.2 algorithm is // inherited; only issueWrite() is bridge-specific. Plc4xCommandOutputAggregator agg = new Plc4xCommandOutputAggregator(svc, cfg, audit); // 8. Register with the worker. Exact registration is your // worker's main-loop concern. worker.registerInput(measIn); worker.registerInput(eventIn); worker.registerOutputAggregator(agg); worker.run(); } } } }

Three things to highlight in this wiring. First, `svc.connect()` is where addresses are validated. Every read/write/event fieldAddress is issued as a one-shot validate() against the open connection. The bridge fails fast if any address is rejected (S9) — better to crash at startup with a useful message than to silently skip a binding at runtime. Second, the same audit sink is shared by the aggregator and the input adapters. Reads do not currently produce audit records, but a misconfigured connection produces operational logs; the aggregator's verdicts and the bridge's lifecycle events land in one JSONL file you can grep chronologically. Third, `AutoCloseable` is non-negotiable — Plc4xClientService.close() cancels every polling task and closes every PlcConnection through the driver. Leaving a PlcConnection open on a Siemens controller eventually exhausts its connection pool.

# 8. Modeling guidance

## 8.1 Choosing what to bind

A useful test: if the variable changes faster than 1 Hz and matters for control, bind it as a read. PID process variables, motor currents, vibration RMS, position encoders — all good candidates. Variables that change slowly (ambient temperature trends, batch counters) work better as events polled once every few seconds. Don't bind hundreds of measurements at 4 Hz against a single small S7 — the controller's CPU load is the limiting factor; the bridge will keep up but the PLC won't.

Start with one read binding per process variable. Once the bridge is producing MeasurementSignals for them, neuron design proceeds as usual — IndustrialSensorNeuron to filter, PIDLoopNeuron for regulatory control, SafetyGateNeuron for harm-veto, ActuatorCommandSignal out the other side.

## 8.2 Adding a new measurement

The three-step procedure:

- YAML: add a reads: entry with a unique bindingId, the connectionId, the verbatim PLC4X fieldAddress, a stable signalTag following your ISA-95 convention, and a pollIntervalMs.

- Java neurons (optional): add a neuron that consumes the MeasurementSignal with the configured signalTag. If you omit this step, the signal flows into the network but goes nowhere downstream.

- Tests: the bridge's own integration tests use StubPlc4xDriver; you can stub your new field address there and assert the signal shape.

No code change to the bridge core is needed for new reads.

## 8.3 Adding a new setpoint

The five-step procedure. Every promotion to autonomous control follows this sequence — there's no fast path:

- YAML — write binding. Add the writes: entry with failSafeValue, clamp range, and ramp rate. Critical: pick failSafeValue carefully. For a heater, it is 0 Watts. For a flow valve on a circulating loop, it might be the trickle flow that keeps a heat exchanger from boiling — not zero. The plant engineer owns this number.

- YAML — start in SHADOW. perTagSafetyMode: { TIC-101: SHADOW } is the safe default. Every command audited, no protocol writes.

- Run. For an observation period typical to your site (24 h is a reasonable minimum), let the model run in SHADOW. Inspect the audit JSONL: every command should appear with verdict=REJECTED reason=SHADOW_MODE, recording the proposed value, the timestamp, and the contributing neurons. This is your "what would the AI do?" log.

- Promote to ADVISORY. If SHADOW behaviour is correct, change the YAML to ADVISORY and restart. The aggregator now applies commands with execute=true only after operator confirmation. (The repo's reference Plc4xCommandOutputAggregator returns false from operatorConfirmed(); sites that need ADVISORY-mode writes subclass it and wire in their HMI's confirmation channel.)

- Promote to AUTONOMOUS. Only after another observation period. AUTONOMOUS applies commands with execute=true directly through the safety ladder. Even now, the structural defences remain: interlock trips overwrite any pending command, operator overrides hold writes, the clamp/rate-limit/diff-suppress ladder applies, every write is audited.

## 8.4 Wiring interlocks

An interlock signal is structurally distinct from any other safety mechanism in the framework. When InterlockSignal.tripped == true for a loopId, the aggregator writes that loop's failSafeValue regardless of any other state in the same tick — no neuron can veto an interlock, no operator override blocks it, no shadow mode masks it. Audited as verdict=INTERLOCK_TRIP.

Wire interlocks at the neuron layer with the same loopId as the write binding's bindingId. The InterlockNeuron produces an InterlockSignal that includes the loopId; the aggregator's bindingsForInterlock(loopId) finds every write binding whose loopId matches and writes them all to fail-safe.

## 8.5 Alarm-word decoding

Many PLCs pack 16 or 32 booleans into one diagnostic word for efficient transfer. The PLC4X bridge supports this directly via events: bindings. Configure the word's address once, then list the bit masks that matter in the severityMap. The mapper polls the word at the configured pollIntervalMs, checks each entry — (word & mask) == mask — and emits one AlarmSignal per matching mask. The mask key string preserves its original form in the emitted tag: a binding PLANT.LINE_A.TROUBLE with mask 0x0010 produces an alarm whose tag is PLANT.LINE_A.TROUBLE.0x0010.

This means a single 16-bit word can drive up to 16 distinct alarm channels into the network. Choose the masks deliberately — listing every bit produces noise; listing only the ones that drive a neuron decision keeps the alarm channel useful.

# 9. The egress safety algorithm

When the worker calls Plc4xCommandOutputAggregator.save(List<IResult>, long, long, IContext), the inherited AbstractBridgeOutputAggregator does the following, in this order. Each step is auditable; the audit reason is the bold word in each bullet.

- Partition the results into interlocks, overrides, and commands.

- For every InterlockSignal with tripped=true: resolve the bindings whose loopId matches via bindingsForInterlock(); write each binding's failSafeValue through issueWrite() regardless of mode. Audit each as INTERLOCK_TRIP.

- For every OperatorOverrideSignal: refresh the OverrideRegistry for its tag with the configured TTL (default 5 minutes). The registry holds active overrides keyed by tag.

- For every command (ActuatorCommandSignal):

- UNKNOWN_TAG: resolve the binding by tag. If binding() returns null, audit REJECTED reason=UNKNOWN_TAG, no write.

- OVERRIDE_HOLD: if the registry has an active override for the tag, audit REJECTED reason=OVERRIDE_HOLD, no write.

- SHADOW_MODE: resolve the effective mode via safetyMode(). If SHADOW, audit REJECTED reason=SHADOW_MODE, no write.

- ADVISORY_HOLD: if mode is ADVISORY and (execute=false OR operatorConfirmed() returns false), audit REJECTED reason=ADVISORY_HOLD, no write.

- Clamp: if the value is outside [minClampValue, maxClampValue], move it to the nearest bound. Will audit APPLIED reason=CLAMPED_LOW or CLAMPED_HIGH at the end.

- Rate-limit: if the change from lastApplied[tag] exceeds rampRateMaxPerSec * dt, move the effective value to within the ramp. Will audit APPLIED reason=RATE_LIMITED.

- Diff-suppress: if the value is within DIFF_EPSILON of lastApplied[tag] and dt < DIFF_WINDOW_MS (5 seconds), audit APPLIED reason=DIFF_SUPPRESSED, no write.

- Protocol write: call issueWrite(binding, effectiveValue). The PLC4X aggregator delegates to Plc4xClientService.write(), which calls the driver. Returns BridgeWriteResult.ok() on PlcResponseCode.OK, or BridgeWriteResult.failed("PLC4X:" + code) on anything else.

- APPLIED or FAILED: audit the verdict. On success, update lastApplied[tag].

The whole algorithm lives in AbstractBridgeOutputAggregator. The Plc4x aggregator overrides only four methods: binding(tag), safetyMode(binding), bindingsForInterlock(interlockId), and issueWrite(binding, value). That is the entire surface area of the bridge against the safety algorithm.

# 10. Audit and transparency

The bridge writes one JSONL record per decision, conforming to the framework's BridgeAuditRecord schema (00-FRAMEWORK.md §4):

{ "ts": 1740000000000, "run": 12345, "bridge": "plc4x", "verdict": "APPLIED", "loopId": "TIC-101", "tag": "PLANT.TIC101.SP", "proposed": 47.3, "effective": 45.0, "reason": "RATE_LIMITED", "safetyMode": "AUTONOMOUS", "evidenceNeurons": ["Setpoint-12", "MPC-3"] }

Records you will see:

| `verdict` | `reason` | Triggered by |
| --- | --- | --- |
| APPLIED | (empty) | Command applied verbatim. |
| APPLIED | CLAMPED_HIGH / CLAMPED_LOW | Value pinned to clamp boundary. |
| APPLIED | RATE_LIMITED | Step exceeded rampRateMaxPerSec. |
| APPLIED | DIFF_SUPPRESSED | Value identical within ε to last applied within 5 s; no protocol write. |
| REJECTED | UNKNOWN_TAG | No write binding for the command's tag. |
| REJECTED | SHADOW_MODE | Loop in SHADOW. |
| REJECTED | ADVISORY_HOLD | Loop in ADVISORY without operator confirmation. |
| REJECTED | OVERRIDE_HOLD | Operator override active for tag. |
| INTERLOCK_TRIP | (empty) | Tripped interlock wrote fail-safe value. |
| FAILED | PLC4X:<code> | Protocol write returned non-OK; <code> is the PLC4X response code name. |

The audit sink is append-only and AutoCloseable. Audit-write failures (full disk, permissions revoked) are isolated per framework S5: the bridge keeps running in degraded mode and surfaces Plc4xAuditOutput.isDegraded() == true for monitoring.

When SHADOW mode produces a sea of verdict=REJECTED reason=SHADOW_MODE records and you only want the audits where the bridge actually changed something, set audit.writeRejectedToAudit: false. Rejections are still counted in the bridge's internal metrics, just not persisted.

# 11. Operating procedures

## 11.1 Smoke test with the stub driver

The fastest verification path uses StubPlc4xDriver, which simulates both s7:// and modbus-tcp:// schemes in-memory:

- Build worker — mvn -pl worker package.

- Copy the minimal example from §6.2 to plc4x-bridge.yml. Adjust connectionString to modbus-tcp://stub:502?unit-identifier=1 (the stub accepts any host).

- Write a tiny bootstrap that constructs new StubPlc4xDriver() instead of a real adapter, calls svc.connect(), then loops calling measIn.readSignals() and printing each signal.

- Verify that within one tick the bridge emits a MeasurementSignal for DEMO.COIL.0. The stub's value evolves on every read so successive ticks show different values.

## 11.2 Smoke test against OpenPLC

For an end-to-end check against a real PLC simulator:

- docker run -p 502:502 -p 8080:8080 openplc/openplc_v3 — exposes a Modbus TCP slave on port 502 plus a web UI on 8080.

- Use the §6.2 minimal YAML pointing at modbus-tcp://127.0.0.1:502?unit-identifier=1.

- Run the bootstrap with your production Plc4xRealDriver adapter — see §12.

- Toggle the OpenPLC coil from the web UI. The printed MeasurementSignal flips between 0 and 1 within one pollIntervalMs.

## 11.3 Promoting a loop from SHADOW to ADVISORY

- Run for at least one observation period (24 h is the typical minimum) in SHADOW. Inspect the audit JSONL: every motor command should appear with verdict=REJECTED reason=SHADOW_MODE, recording the proposed value and the contributing neurons.

- Confirm with the operator that the proposed setpoints are sensible.

- Stop the worker. Edit YAML: change the binding's perTagSafetyMode entry to ADVISORY.

- Subclass Plc4xCommandOutputAggregator and override operatorConfirmed(ActuatorCommandSignal) to consult your HMI's confirmation flag.

- Restart. Subsequent commands appear with verdict=APPLIED when the operator confirms, or verdict=REJECTED reason=ADVISORY_HOLD when they don't.

## 11.4 Promoting from ADVISORY to AUTONOMOUS

Same procedure as 11.3, except: another observation period at ADVISORY, another operator sign-off, and the only structural difference is that execute=true commands no longer need operator confirmation. The interlock / override / clamp / rate-limit ladder is unchanged — those defences operate at all modes.

## 11.5 Diagnosing a stuck binding

If Plc4xMeasurementInput.readSignals() returns nothing for a binding that you expect to be active:

- Check that svc.pollCount(bindingId) > 0. If zero, the polling thread has never returned PlcResponseCode.OK for that binding. Likely causes: wrong address, controller offline, driver scheme mismatch.

- Check svc.latest(signalTag). If non-null but the code is not OK, the latest read failed; the input emits Quality.BAD or UNCERTAIN.

- Check the bridge log for "Plc4x poll threw" warnings. A driver exception caches a REMOTE_ERROR response so the input always emits something per the framework's quality-propagation rule, never silently swallowing the failure.

# 12. Production wiring

The bridge core has no compile-time dependency on the PLC4X jar. Each deployment supplies its own Plc4xDriver adapter. A minimal real-driver adapter:

package com.example.plant; import com.rakovpublic.jneuropallium.worker.bridge.plc4x.*; import org.apache.plc4x.java.DefaultPlcDriverManager; import org.apache.plc4x.java.api.PlcConnection; import org.apache.plc4x.java.api.messages.PlcReadRequest; import org.apache.plc4x.java.api.messages.PlcReadResponse; import org.apache.plc4x.java.api.messages.PlcWriteRequest; import org.apache.plc4x.java.api.types.PlcResponseCode; import java.util.concurrent.ConcurrentHashMap; import java.util.concurrent.ConcurrentMap; public final class Plc4xRealDriver implements Plc4xDriver { private final DefaultPlcDriverManager mgr = new DefaultPlcDriverManager(); private final ConcurrentMap<String, PlcConnection> open = new ConcurrentHashMap<>(); @Override public void open(String id, String connectionString) { try { open.put(id, mgr.getConnection(connectionString)); } catch (Exception e) { throw new Plc4xException(e.getMessage(), e); } } @Override public Plc4xResponseCode validate(String id, String fieldAddress) { return read(id, fieldAddress).code(); } @Override public ReadResponse read(String id, String fieldAddress) { PlcConnection c = open.get(id); if (c == null) return ReadResponse.failure(Plc4xResponseCode.REMOTE_ERROR); try { PlcReadRequest req = c.readRequestBuilder() .addTagAddress("v", fieldAddress).build(); PlcReadResponse resp = req.execute().get(); Plc4xResponseCode code = translate(resp.getResponseCode("v")); return new ReadResponse(code, resp.getObject("v")); } catch (Exception e) { return ReadResponse.failure(Plc4xResponseCode.REMOTE_ERROR); } } @Override public Plc4xResponseCode write(String id, String fieldAddress, Object value) { PlcConnection c = open.get(id); if (c == null) return Plc4xResponseCode.REMOTE_ERROR; try { PlcWriteRequest req = c.writeRequestBuilder() .addTagAddress("v", fieldAddress, value).build(); return translate(req.execute().get().getResponseCode("v")); } catch (Exception e) { return Plc4xResponseCode.REMOTE_ERROR; } } @Override public void close(String id) { PlcConnection c = open.remove(id); if (c != null) try { c.close(); } catch (Exception ignored) {} } @Override public void closeAll() { open.values().forEach(c -> { try { c.close(); } catch (Exception ignored) {} }); open.clear(); } private static Plc4xResponseCode translate(PlcResponseCode code) { return switch (code) { case OK -> Plc4xResponseCode.OK; case ACCESS_DENIED -> Plc4xResponseCode.ACCESS_DENIED; case NOT_FOUND -> Plc4xResponseCode.NOT_FOUND; case INVALID_ADDRESS -> Plc4xResponseCode.INVALID_ADDRESS; case INVALID_DATATYPE -> Plc4xResponseCode.INVALID_DATATYPE; case INVALID_DATA -> Plc4xResponseCode.INVALID_DATA; case INTERNAL_ERROR -> Plc4xResponseCode.INTERNAL_ERROR; case REMOTE_ERROR -> Plc4xResponseCode.REMOTE_ERROR; case RESPONSE_PENDING -> Plc4xResponseCode.RESPONSE_PENDING; default -> Plc4xResponseCode.REMOTE_ERROR; }; } }

The adapter is ~80 lines. The bridge core, including the safety algorithm, audit, and reconnect logic, is over 1500 lines that you do not need to touch.

# 13. Limitations and future work

- No source-system timestamps. PLC4X does not surface a per-read server timestamp the way OPC UA does; the bridge tags each measurement with System.currentTimeMillis() at cache-read time. A custom Plc4xDriver adapter can return its own timestamp via the ReadResponse and a custom Plc4xMeasurementInput can pick it up.

- Polling, not subscribing. Most PLC4X drivers are poll-based. A few (S7 with cyclic services) support push semantics; the bridge does not currently exploit them. The framework IInitInput contract is snapshot-based anyway, so the gain is bounded.

- No PUT/GET on Siemens S7 by default. Production S7 controllers very often disable PUT/GET for security. Verify with the controls engineer before promoting any write binding. The bridge will report ACCESS_DENIED from validate() and fail at startup if PUT/GET is off.

- Mixed-endian PLCs. Most modern drivers handle byte ordering, but some legacy targets do not. Document the per-driver byte-order convention in your site runbook; surface a byteOrder config knob via a custom Plc4xDriver if needed.

- No `Quality.UNCERTAIN` for stale data. A binding whose latest poll succeeded an hour ago still emits Quality.GOOD. If you need staleness-as-uncertainty, wrap Plc4xMeasurementInput with a neuron that checks the signal's timestamp against System.currentTimeMillis() and degrades quality past a threshold.

# 14. Regulatory posture

| Aspect | Posture |
| --- | --- |
| Safety ceiling | AUTONOMOUS per-loop. Reach autonomous only via SHADOW → ADVISORY → AUTONOMOUS with documented observation periods. |
| Default per-loop mode | SHADOW. Every loop must be explicitly promoted. |
| Interlock authority | Direct. A tripped InterlockSignal writes failSafeValue regardless of mode (§0.2). |
| Operator override | Holds the bound tag for the override TTL. Defeats neuron-derived commands; does not defeat interlocks. |
| Audit | One JSONL line per command. Schema documented in 00-FRAMEWORK.md §4. |
| MoC | Hot-reload not supported. YAML changes follow your site's Management of Change procedure. |
| PUT/GET on S7 | Production controllers often disable PUT/GET — confirm with the controls engineer before shipping a write binding. |
| IEC 61508 / 62443 alignment | The bridge does not claim SIL/SL certification by itself. It produces the audit and structural defences a higher-level safety case needs. |

For closed-loop control of safety-instrumented functions (SIS), do not use this bridge. Use a certified protection relay or SIS controller. The PLC4X bridge supports regulatory control with structural defences against rogue writes; it is not a substitute for a certified safety system.

# 15. References

- 01-PLC4X.md — the bridge specification.

- 00-FRAMEWORK.md — the universal bridge contract.

- docs/plc4x-bridge.md — domain-context companion in the repo.

- Apache PLC4X — https://plc4x.apache.org/.

- PLC4X driver feature matrix — https://plc4x.apache.org/users/protocols/index.html. Each driver has a different read/write/subscribe support level — verify before designing a binding.

- OpenPLC — https://www.openplcproject.com/. Free PLC simulator suitable for development.

- Rakovskyi, D. (2024). Framework for Natural Neuron Network Modeling: The Jneopallium Approach. IJSR 13(7).