Building Models with ROS 2 / DDS and Jneopallium

User Manual

Robotics middleware integration · advisory-only egress · auditable swarm decisions

Version: 1.0-SNAPSHOT

License: BSD 3-Clause

Author: Dmytro Rakovskyi

# 1. Introduction

[ROS 2](https://docs.ros.org/) is the dominant middleware for modern robotics. It replaced ROS 1's centralised master with [DDS](https://www.dds-foundation.org/) (Data Distribution Service) — a publish/subscribe transport with rich Quality-of-Service contracts, peer-to-peer discovery, and predictable real-time behaviour. The ROS 2 ecosystem provides perception (LIDAR, camera, IMU drivers), proprioception (JointState, Odometry), navigation, simulation (Gazebo, Ignition, Webots), and a vast catalogue of community packages.

The ROS 2 bridge connects Jneopallium to robots and robotic simulators. It implements Strategy B from the spec: a thin JSON-over-WebSocket adapter on top of [rosbridge_suite](https://github.com/RobotWebTools/rosbridge_suite), backed by the JDK's built-in java.net.http.WebSocket. Strategy A (an embedded rcljava client linking the C/C++ middleware via JNI) is left as a feature flag — present in the source tree as RcljavaClientService but not built into the v1 worker jar; it requires a working ROS 2 installation on the build host, which is not a portable assumption.

The bridge is positioned as advisory-only. It subscribes to perception, proprioception, peer-observation, and battery topics, emits typed SensorySignal / ProprioceptiveSignal / EfficiencySignal / PeerObservationSignal, and publishes neuron-derived MotorCommandSignal / FormationSignal / HarmVetoSignal decisions to a configurable advisory namespace consumed by an external autonomy supervisor. The autonomy supervisor — not the bridge — decides whether to translate advice into a real actuator command. The bridge does not publish to /cmd_vel, /joint_trajectory, or /joint_command in any production environment. This is enforced two ways: a hard-coded forbidden-topic list rejects those exact paths at config load unless simulatorOnly: true is set, and per-tag AUTONOMOUS promotion is similarly rejected unless simulatorOnly is set. There is no flag that bypasses both.

This manual explains how to install the dependency, configure the bridge, design a swarm-aware model, run the result against turtlesim or Gazebo, and validate it end to end. It assumes a Java engineer who knows Maven, has read the universal bridge framework (00-FRAMEWORK.md), and has at least basic familiarity with ROS 2 concepts (topics, message types, the QoS model, ROS_DOMAIN_ID).

# 2. Six rules you must internalise before writing any code

The universal six from 00-FRAMEWORK.md §0, specialised for ROS 2:

- No raw write to a field actuator from neuron output. The bridge never publishes to /cmd_vel, /joint_trajectory, or /joint_command in production. Approved MotorCommandSignals are published to an advisory namespace under operator control (e.g. mission_advice/cmd_vel, formation_advisory/...); the external autonomy supervisor is the gatekeeper. Defence in depth: the config record's compact constructor rejects forbidden-topic writes at load time, and Ros2ClientService.publish() re-checks at runtime.

- Interlocks have no direct ROS 2 authority. Because the bridge does not drive any actuator itself, there is no fail-safe value for an InterlockSignal to write through ROS 2. Interlock authority lives elsewhere — usually a microcontroller on the robot. The bridge can carry interlock state as advisory HarmVetoSignals for the supervisor's awareness.

- Operator override does not apply. Advisory bridges do not write regulatory control. The aggregator ignores OperatorOverrideSignal by design — overrides matter at the OPC UA / PLC4X / FMI layer where actual writes happen.

- Every advisory publish produces an audit record. Records cover accepted publishes, SHADOW-dropped commands, clamped values, oversized-payload drops, decoder failures on ingress, and bridge reconnects. Records are written to a local JSONL file and, optionally, mirrored to a ROS 2 advisory audit topic.

- Quality propagates. ROS 2 messages do not carry an explicit quality field, but the bridge surfaces stale data as Quality.UNCERTAIN when a topic has not produced a message within an expected window (configurable per binding). A topic the bridge has lost (publisher disappeared from discovery) flags Quality.BAD on subsequent reads.

- Wall-clock timestamps come from the source. ROS 2 Header.stamp is honoured when present and parseable; otherwise the bridge stamps System.currentTimeMillis() at the moment the message arrives.

# 3. Prerequisites

- JDK 17 or later. The bridge uses Java records, sealed types in dependencies, and java.net.http.WebSocket.

- Maven 3.9 or later.

- A running rosbridge_websocket instance reachable from the host. Typically launched via ros2 run rosbridge_server rosbridge_websocket inside a ROS 2 Humble/Iron/Jazzy environment.

- A ROS 2 system or simulator to subscribe to: turtlesim (smallest), Gazebo / Ignition (full physics), or a real robot stack.

- The bridge core compiles without any ROS 2 installation on the build host — Strategy B speaks JSON over WebSockets, and Strategy A is not enabled by default. Tests inject an in-memory Ros2Transport so the integration suite runs without rosbridge.

# 4. Adding Jneopallium to your project

The ROS 2 bridge ships in the same worker artifact as the rest of Jneopallium:

<repositories> <repository> <id>gitlab-jneopallium</id> <url>https://gitlab.com/api/v4/projects/44434192/packages/maven</url> </repository> </repositories> <dependencies> <dependency> <groupId>com.rakovpublic.jneopallium</groupId> <artifactId>worker</artifactId> <version>1.0-SNAPSHOT</version> </dependency> </dependencies>

No additional dependencies are needed for Strategy B. The JDK's java.net.http.WebSocket is the transport. JSON encode/decode goes through the worker's existing Jackson dependency.

# 5. Architecture

The bridge introduces twelve boundary classes under worker.bridge.ros2:

┌─────────────────────────────────┐ │ Ros2ClientService │ │ • owns the transport │ │ • per-binding ingress queues │ │ • latest-message cache │ │ • bounded advisory queue │ │ • decimation + payload caps │ └─────┬───────────────┬───────────┘ │ │ SUBSCRIBE │ │ PUBLISH ──────────────────────────────────────▶│ │◀────────── │ │ ┌──────────────────────┐ │ │ │ rosbridge_server │ ─── /scan, /odom, … ──────│ │ │ ↑ │ │ │ │ │ DDS │ ─── tf, /battery, … ──────▶ │ │ │ │ │ │ │ ROS 2 graph │ ◀── /mission_advice ──── │ │ │ (turtlesim, │ advisory topic only │ │ │ Gazebo, real) │ │ │ └──────────────────────┘ │ │ ▼ ▼ ┌──────────────────────┐ ┌────────────────────┐ │ Ros2SensoryInput │ │ Ros2StateInput │ │ (IInitInput → │ │ (IInitInput → │ │ Sensory / │ │ Proprio / │ │ PeerObservation / │ │ Efficiency) │ │ Alarm) │ │ │ └──────────────────────┘ └────────────────────┘ ▼ [Standard Jneopallium pipeline] ▼ Ros2AdvisoryOutputAggregator │ ▼ Ros2ClientService.publish() (advisory namespace only) │ ▼ Ros2AuditOutput (JSONL + optional ROS 2 mirror)

Components:

| Class | Role |
| --- | --- |
| Ros2BridgeConfig | YAML record (immutable). Enforces ADVISORY ceiling and forbidden-topic list at construction. |
| Ros2BridgeConfigLoader | Jackson YAML loader. FAIL_ON_UNKNOWN_PROPERTIES = true. |
| Ros2Transport | Test-seam interface: connect / subscribe / advertise / publish / setHandler / isConnected / close. |
| RosbridgeTransport | Strategy B implementation. JDK WebSocket against rosbridge_websocket. |
| RcljavaClientService | Strategy A placeholder. Behind a feature flag; not built in v1. |
| Ros2TopicBinding | Resolved binding identity used at runtime by the service. |
| Ros2MessageMapper | Pure functions. ROS 2 JSON ↔ typed signal. Handles nav_msgs/Odometry, sensor_msgs/{JointState, LaserScan, Image, BatteryState}, std_msgs/String and similar. |
| Ros2ClientService | Owns the transport, the per-binding queues, the latest-message cache, the bounded advisory queue, decimation, payload caps, and the audit channel. AutoCloseable. |
| Ros2SensoryInput | IInitInput. Drains sensory bindings (SENSORY signalKind), peer observations (asPeerObservation: true), and bridge-level alarms each tick. |
| Ros2StateInput | IInitInput. Drains proprioception (PROPRIOCEPTIVE signalKind from JointState / Odometry) and energy (ENERGY signalKind from BatteryState). |
| Ros2AdvisoryOutputAggregator | IOutputAggregator. Drops on SHADOW, publishes on ADVISORY, audits every decision. Like the MQTT bridge, does NOT extend AbstractBridgeOutputAggregator because the advisory ceiling makes the autonomous-write algorithm moot. |
| Ros2AuditOutput | Local JSONL append-only audit plus optional ROS 2 mirror topic. |

The aggregator deliberately diverges from AbstractBridgeOutputAggregator. The reason mirrors MQTT: the universal §2.2 algorithm models a SHADOW → ADVISORY → AUTONOMOUS ladder, but this bridge has no AUTONOMOUS rung (except in simulatorOnly mode, which already changes the semantics). There are no interlock fail-safes to write into ROS 2, no operator override registry to consult, no rate-limiting against an actuator. The aggregator's job is narrower: drop on SHADOW, encode and publish on ADVISORY, audit every decision. The structural ceiling is enforced by the config record at load time, not by inheritance.

# 6. Strategy B vs Strategy A — choosing the transport

Two transport options exist; only one is built in v1.

Strategy B — rosbridge_suite over WebSocket (default). A rosbridge_websocket node runs in the ROS 2 graph and exposes a JSON-over-WS interface to the outside world. The bridge's RosbridgeTransport connects to that node, subscribes by topic, and publishes JSON-encoded messages. Pros: no native dependencies in the worker, the build is hermetic, rosbridge already exists in every modern ROS 2 deployment for web tooling, the same bridge works against any ROS 2 distribution that supports rosbridge_suite. Cons: extra hop adds 1–5 ms latency, message types serialise to JSON (so binary fields like images become base64 — see §10), rosbridge_websocket is a separate process to monitor.

Strategy A — embedded rcljava. rcljava is the Java client library for ROS 2 native — it bridges JNI to rcl and rmw so a Java process appears as a first-class ROS 2 node in the DDS graph. Pros: native DDS QoS, no extra hop, true peer discovery. Cons: requires a working ROS 2 install on the build host, the worker jar is no longer portable across distributions, JNI complications. The bridge ships a placeholder RcljavaClientService for the architecture intent; it is not built into the v1 worker jar.

Choose Strategy B for v1. Revisit Strategy A only if measured latency requirements demand it.

# 7. Configuration

Configuration is YAML, loaded once at startup through Ros2BridgeConfigLoader.load(Path). Hot-reload is not supported. The loader sets FAIL_ON_UNKNOWN_PROPERTIES = true.

## 7.1 Schema

The configuration has ten top-level keys.

- mode — ROSBRIDGE (default) or RCLJAVA.

- rosbridgeUrl — WebSocket URL of rosbridge_websocket (required when mode: ROSBRIDGE).

- domainId — ROS 2 ROS_DOMAIN_ID for Strategy A (advisory documentation under Strategy B).

- qosProfile — SENSOR_DATA, RELIABLE, or PARAMETERS.

- simulatorOnly — boolean. When true, unlocks /cmd_vel writes and per-tag AUTONOMOUS. Default false.

- reads — list of subscribe bindings.

- writes — list of advisory publish bindings.

- audit — local JSONL file path plus optional advisoryAuditTopic.

- perTagSafetyMode — per-binding SHADOW / ADVISORY (or AUTONOMOUS only when simulatorOnly: true).

- tickInterval — worker tick cadence (ISO-8601). Default PT0.25S.

## 7.2 Minimal example — turtlesim smoke test

The smallest useful configuration. One ingress binding from turtlesim, no writes, no simulator gates. Suitable as a first integration check.

mode: ROSBRIDGE rosbridgeUrl: "ws://localhost:9090" simulatorOnly: true reads: - bindingId: TURTLE-POSE topic: /turtle1/pose msgType: turtlesim/msg/Pose signalTag: TURTLE.T1.POSE writes: [] audit: localAuditFile: "/tmp/jneopallium-ros2-audit.jsonl" tickInterval: "PT0.25S"

## 7.3 Production-like example — fleet advisor against a Gazebo simulation

A more substantial config: subscribes to perception (LIDAR), proprioception (Odometry), and battery from one robot; subscribes to peer Odometry from two other robots in a fleet; publishes mission_advice and formation_advisory to advisory topics. simulatorOnly: true because the target is Gazebo.

mode: ROSBRIDGE rosbridgeUrl: "ws://gazebo-host:9090" domainId: 42 qosProfile: SENSOR_DATA simulatorOnly: true reads: - bindingId: SELF-LASER topic: /robot1/scan msgType: sensor_msgs/msg/LaserScan signalTag: ROBOT1.SCAN signalKind: SENSORY maxRangeBins: 360 decimateBy: 2 - bindingId: SELF-ODOM topic: /robot1/odom msgType: nav_msgs/msg/Odometry signalTag: ROBOT1.POSE signalKind: PROPRIOCEPTIVE - bindingId: SELF-BATTERY topic: /robot1/battery_state msgType: sensor_msgs/msg/BatteryState signalTag: ROBOT1.BATTERY signalKind: ENERGY - bindingId: PEER-ROBOT2-ODOM topic: /robot2/odom msgType: nav_msgs/msg/Odometry signalTag: PEER.ROBOT2.POSE asPeerObservation: true peerId: ROBOT2 - bindingId: PEER-ROBOT3-ODOM topic: /robot3/odom msgType: nav_msgs/msg/Odometry signalTag: PEER.ROBOT3.POSE asPeerObservation: true peerId: ROBOT3 writes: - bindingId: MISSION-ADVICE topic: /mission_advice/cmd_vel msgType: geometry_msgs/msg/Twist signalTag: ROBOT1.MISSION.ADV minClampValue: -0.5 maxClampValue: 0.5 rampRateMaxPerSec: 1.0 - bindingId: FORMATION-ADVISORY topic: /formation_advisory/slot msgType: std_msgs/msg/Int32 signalTag: FORMATION.SLOT audit: localAuditFile: "/var/log/jneopallium/ros2-audit.jsonl" advisoryAuditTopic: "/jneopallium/audit" perTagSafetyMode: MISSION-ADVICE: ADVISORY FORMATION-ADVISORY: ADVISORY tickInterval: "PT0.1S"

## 7.4 Field reference

mode chooses the transport. ROSBRIDGE is the only choice that ships built. RCLJAVA fails fast at startup until that strategy is wired in.

rosbridgeUrl is the full WebSocket URL — ws://host:9090 typically. rosbridge_websocket defaults to port 9090. Use wss:// for TLS in production.

domainId is informational under Strategy B; the value is passed through to audit records for traceability. Under Strategy A (when implemented) it sets the actual ROS_DOMAIN_ID for DDS discovery.

qosProfile mirrors ROS 2's named QoS presets. SENSOR_DATA (best-effort, volatile, depth 10) is appropriate for high-frequency telemetry like LIDAR. RELIABLE (reliable, transient_local, depth 10) is appropriate for low-frequency state like /joint_state or commands. PARAMETERS is the ROS 2 default for parameter servers. The bridge passes the choice through to rosbridge_websocket; ensure your rosbridge build supports per-topic QoS.

simulatorOnly is the master gate. When false (default), per-tag AUTONOMOUS and writes to /cmd_vel, /joint_trajectory, /joint_command are rejected at config load with an IllegalArgumentException naming the offending binding. When true, both are unlocked — but you should set this only when (a) the target topics live in a simulator namespace (Gazebo, Ignition, Webots) and (b) an external watchdog node will kill the bridge if heartbeats stop.

reads[].msgType is the ROS 2 message type name in canonical form: package/msg/TypeName. The signal kind is inferred from this when signalKind is omitted — see the inference rules in §9.

reads[].asPeerObservation: true is valid only for nav_msgs/msg/Odometry. It changes the emitted signal from ProprioceptiveSignal (own-robot pose) to PeerObservationSignal (peer-robot pose with peerId attached). For swarm models.

reads[].decimateBy drops all but every Nth message. Useful when subscribing to a high-rate topic (LIDAR at 40 Hz) but the model ticks slower (10 Hz). Default 1 (no decimation).

reads[].maxRangeBins is the LIDAR / point-cloud downsample limit. Messages with more bins are sliced to this size before encoding to a SensorySignal. Default 720.

reads[].maxPayloadBytes caps the JSON payload size after decoding. Messages larger than this are dropped with verdict=FAILED reason=PAYLOAD_TOO_LARGE. Default 1 048 576 (1 MiB). Useful for images and dense point clouds.

writes[].topic is the advisory topic. The forbidden-topic rule in Ros2BridgeConfig.FORBIDDEN_WRITE_TOPICS blocks /cmd_vel, /joint_trajectory, /joint_command unless simulatorOnly: true. The bridge does not enforce that advisory topics live under a specific namespace; by convention, prefix them with /mission_advice, /formation_advisory, or similar.

writes[].msgType is the encoded message type. geometry_msgs/msg/Twist is the typical advisory motor type. The mapper currently understands Twist, Int32, String, and a minimal set; new types extend Ros2MessageMapper.

writes[].minClampValue / maxClampValue / rampRateMaxPerSec / failSafeValue — same semantics as PLC4X / FMI. Even though there is no actuator on the other end, these are useful: the supervisor sees clean values, not occasional 99999 outliers.

perTagSafetyMode keys are bindingIds. Legal values: SHADOW, ADVISORY. AUTONOMOUS is only legal when simulatorOnly: true. Unlisted bindings default to SHADOW.

# 8. Wiring the Java side

The bridge does not ship a main class. A minimal driver:

package com.example.robot; import com.rakovpublic.jneuropallium.worker.bridge.common.AbstractBridgeAuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.ros2.RosbridgeTransport; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2AdvisoryOutputAggregator; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2AuditOutput; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2BridgeConfig; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2BridgeConfigLoader; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2ClientService; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2MessageMapper; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2SensoryInput; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2StateInput; import com.rakovpublic.jneuropallium.worker.bridge.ros2.Ros2Transport; import java.nio.file.Path; public final class Ros2Bootstrap { public static void main(String[] args) throws Exception { // 1. Load YAML. AUTONOMOUS and forbidden topics rejected unless // simulatorOnly: true. Ros2BridgeConfig cfg = Ros2BridgeConfigLoader.load(Path.of("ros2-bridge.yml")); // 2. Open the audit sink. try (AbstractBridgeAuditOutput audit = new Ros2AuditOutput( Path.of(cfg.audit().localAuditFile()))) { // 3. Build the transport. Strategy B: WebSocket against rosbridge. try (Ros2Transport transport = new RosbridgeTransport(cfg.rosbridgeUrl())) { // 4. Build the message mapper and the client service. Ros2MessageMapper mapper = new Ros2MessageMapper(); Ros2ClientService svc = new Ros2ClientService(cfg, transport, mapper, audit); // 5. Start: connect, subscribe to every read binding, // advertise every write binding. Idempotent. svc.start(); try { // 6. Build the inputs. Ros2SensoryInput sensIn = new Ros2SensoryInput("ros2-sens", svc); Ros2StateInput stateIn = new Ros2StateInput("ros2-state", svc); // 7. Build the aggregator. Ros2AdvisoryOutputAggregator agg = new Ros2AdvisoryOutputAggregator(svc, audit); // 8. Register with the worker. worker.registerInput(sensIn); worker.registerInput(stateIn); worker.registerOutputAggregator(agg); worker.run(); } finally { svc.close(); } } } } }

Three implementation notes worth highlighting. `svc.start()` does subscribe + advertise, not just connect. The client service owns the transport's connect() call, subscribes to every ingress binding's topic, advertises every advisory egress topic, and registers the message handler in one method. Subsequent reconnects re-subscribe automatically — rosbridge_websocket doesn't retain subscriptions across the WS reconnect. Two inputs, by signal kind. The split between Ros2SensoryInput (sensory + peer + alarms) and Ros2StateInput (proprioception + energy) mirrors the conceptual split in a robot — perception and own-state are independent layers. Wire both to the worker. `AutoCloseable` cascade. Closing the service closes the transport, which terminates the WebSocket. Failure to close leaves a rosbridge client session that the server eventually times out; harmless but noisy in logs.

# 9. Modeling guidance

## 9.1 Choosing what to bind on ingress

Start with the minimum a controller actually needs:

- One proprioception binding (/odom or /joint_states) — the robot's own state.

- One perception binding (/scan or /camera/depth/image_raw) — the world.

- One energy binding (/battery_state) — for energy-aware planning.

For a swarm, add asPeerObservation: true Odometry bindings for each peer you want the model to be aware of. The mapper emits a PeerObservationSignal with peerId attached, which downstream swarm-aware neurons can consume.

Resist the urge to bind every topic in the graph. ROS 2 systems often expose 30–100 topics per robot; subscribing to all of them through rosbridge saturates the WebSocket. The bridge subscribes only to topics with bindings; the rest go unread.

## 9.2 The signalKind inference table

When reads[].signalKind is omitted, Ros2BridgeConfig.ReadBindingConfig.inferKind(msgType) chooses:

| `msgType` | Inferred `signalKind` |
| --- | --- |
| sensor_msgs/msg/Image | SENSORY |
| sensor_msgs/msg/CompressedImage | SENSORY |
| sensor_msgs/msg/LaserScan | SENSORY |
| sensor_msgs/msg/PointCloud2 | SENSORY |
| sensor_msgs/msg/JointState | PROPRIOCEPTIVE |
| nav_msgs/msg/Odometry | PROPRIOCEPTIVE (or PEER_OBSERVATION if asPeerObservation: true) |
| sensor_msgs/msg/BatteryState | ENERGY |
| anything else | SENSORY (fallback) |

When the inference is wrong (you want /odom treated as a peer observation rather than own pose, for example), set signalKind explicitly. The mapper does the right thing per kind.

## 9.3 Adding a new sensor topic

The three-step procedure:

- YAML: add a reads: entry with a unique bindingId, the topic, the msgType, a stable signalTag, optional signalKind, and (for high-rate sources) decimateBy or maxRangeBins.

- Java neurons (optional): add a neuron that consumes the signal under the configured signalTag.

- Restart the worker. The bridge subscribes at startup; YAML changes are not picked up live.

The mapper handles common ROS 2 types. For unusual ones, extend Ros2MessageMapper in your bootstrap module.

## 9.4 Adding a new advisory write

- YAML — write binding. Choose topic so it lives under an advisory namespace. The forbidden-topic list (/cmd_vel, /joint_trajectory, /joint_command) is enforced at load time — use /mission_advice/cmd_vel, /formation_advisory/..., or similar.

- YAML — start in SHADOW. Audit-only, no protocol publishes.

- Observe. Run, inspect the audit JSONL. Every command should appear as REJECTED reason=SHADOW_MODE with the proposed value.

- Promote to ADVISORY. Change the per-tag mode, restart. Publishes go out.

- Stay in ADVISORY. AUTONOMOUS is unreachable on this bridge except with simulatorOnly: true. For real-world deployment, AUTONOMOUS belongs on a separately certified bridge — typically not this one.

## 9.5 Sensor downsampling

Two knobs control the bandwidth of high-rate sensors:

- decimateBy: N drops all but every Nth message before decoding. Cheap; reduces CPU.

- maxRangeBins: M (for LaserScan) and maxPayloadBytes: K (for everything) cap the decoded payload size. The mapper resamples LaserScan ranges to M bins; payloads above K bytes are dropped with audit. Useful for images — a 1920×1080 RGB frame as JSON is ~20 MB, so maxPayloadBytes: 5000000 rejects it; in practice, subscribe to CompressedImage instead and read the hash.

# 10. ROS 2 message types and JSON encoding

rosbridge_suite serialises ROS 2 messages as JSON. Most field types translate straightforwardly:

| ROS 2 type | JSON shape |
| --- | --- |
| primitives (int32, float32, bool, string) | direct |
| arrays of primitives | JSON array |
| Header | {stamp: {sec, nanosec}, frame_id} |
| nested messages | nested objects |
| uint8[] (image data, point cloud data) | base64-encoded string |
| time / duration | {sec, nanosec} |

The base64 encoding for byte arrays is significant. A sensor_msgs/Image carrying a 1920×1080 RGB frame is ~6 MB of raw bytes → ~8 MB of base64 → an ~8 MB JSON string per message. At 30 Hz that's 240 MB/s of WebSocket traffic. Never subscribe to raw image topics in production. Subscribe to compressed variants (/camera/image_raw/compressed), or have a ROS 2 node do feature extraction before publishing a compact PointStamped or similar.

For LIDAR, sensor_msgs/LaserScan arrives with a ranges[] array of floats. The bridge resamples to maxRangeBins and emits a SensorySignal whose payload is the resampled array.

# 11. The advisory egress algorithm

When the worker calls Ros2AdvisoryOutputAggregator.save(), the aggregator does the following for each result in the batch:

- `MotorCommandSignal`:

- Only commands with execute=true are considered — otherwise audit REJECTED reason=EXECUTE_FALSE.

- Resolve the binding by tag via Ros2ClientService.findWriteByTag(tag). If null → audit REJECTED reason=UNBOUND_SIGNAL_TYPE. No publish.

- Resolve the per-binding safety mode. If SHADOW → audit REJECTED reason=SHADOW_MODE. No publish.

- Clamp the dominant linear-x axis to [minClampValue, maxClampValue]. Audit reason carries CLAMPED_HIGH/CLAMPED_LOW if a clamp fired.

- Encode as the binding's msgType (typically geometry_msgs/msg/Twist).

- publish(topic, json) via the transport. On success → audit APPLIED reason=MOTOR_FRAME. On transport failure → audit FAILED reason=PUBLISH_ERROR:<detail>.

- `FormationSignal`:

- Encode the slot index as a small JSON (e.g. {"data": 3} for std_msgs/Int32).

- Same safety / publish / audit path.

- `HarmVetoSignal`:

- Encode the veto reason as std_msgs/String text.

- Publish on the binding's topic.

- Audited with INTERLOCK_TRIP verdict for downstream correlation. The veto does not drive any actuator through this bridge — it informs the autonomy supervisor.

- `TransparencyLogSignal`:

- If audit.advisoryAuditTopic is configured, publish the JSON-encoded record there.

- Always also write to the local JSONL.

There are no interlocks, no operator override registry, no rate-limit, no diff-suppress. The simpler algorithm reflects the simpler authority — recommendations to an external supervisor, not actuator drive.

# 12. Audit and transparency

The bridge writes one JSONL record per decision, conforming to the framework's BridgeAuditRecord schema:

{ "ts": 1740000000000, "run": 12345, "bridge": "ros2", "verdict": "APPLIED", "tag": "ROBOT1.MISSION.ADV", "proposed": 0.6, "effective": 0.5, "reason": "CLAMPED_HIGH", "safetyMode": "ADVISORY", "evidenceNeurons": ["Planning-7"] }

Records you will see:

| `verdict` | `reason` | Triggered by |
| --- | --- | --- |
| APPLIED | MOTOR_FRAME | Motor advisory published. |
| APPLIED | FORMATION | Formation advisory published. |
| APPLIED | CLAMPED_HIGH / CLAMPED_LOW | Value clamped before publish. |
| APPLIED | BRIDGE_RECONNECTED | Transport reconnected. |
| REJECTED | SHADOW_MODE | Binding in SHADOW. |
| REJECTED | EXECUTE_FALSE | Motor command with execute=false. |
| REJECTED | UNBOUND_SIGNAL_TYPE | Signal's tag has no writes: entry. |
| FAILED | PAYLOAD_TOO_LARGE | Inbound message exceeded maxPayloadBytes. |
| FAILED | PUBLISH_ERROR:<detail> | Transport error during publish. |
| FAILED | DECODER_ERROR:<detail> | Inbound JSON failed to decode. |
| INTERLOCK_TRIP | HARM_VETO | Harm-veto advisory published. |

# 13. Operating procedures

## 13.1 Smoke test against turtlesim

The fastest end-to-end check:

- Start a ROS 2 environment with rosbridge + turtlesim:

``bash    ros2 run rosbridge_server rosbridge_websocket &    ros2 run turtlesim turtlesim_node &    ``

- Save §7.2 minimal config to ros2-bridge.yml.

- Run Ros2Bootstrap (§8).

- Within ~2 s, Ros2StateInput.readSignals() returns ProprioceptiveSignals reflecting /turtle1/pose.

- Drive the turtle from the keyboard (ros2 run turtlesim turtle_teleop_key). The pose signal updates each tick.

## 13.2 Verifying simulator-only protections

A useful sanity check before promoting to a real robot:

- Set simulatorOnly: false and add a writes binding for /cmd_vel.

- Try to start the bridge. Config-load fails with IllegalArgumentException naming the binding.

- Set simulatorOnly: true. Bridge starts.

- Now try promoting a binding to AUTONOMOUS while simulatorOnly: false. Same failure.

The two gates are independent — both must be cleared.

## 13.3 ROS 2 networking checklist

Common issues that have nothing to do with the bridge code:

- `ROS_DOMAIN_ID` mismatch between rosbridge and the robot. Domains 0–101 are valid; pick one per deployment and use it everywhere.

- `ROS_LOCALHOST_ONLY=1` restricts DDS to loopback. Useful for development, breaks production multi-host setups. The runbook should pin one or the other.

- DDS multicast across machines requires a routed network. Many corporate networks block multicast; switch to a unicast discovery server (ROS_DISCOVERY_SERVER) when this is the case.

## 13.4 Promoting from SHADOW to ADVISORY

- Run for an observation period (24 h is typical) in SHADOW.

- Inspect the audit JSONL: every motor / formation / veto advisory should appear as REJECTED reason=SHADOW_MODE with the proposed value.

- Confirm with the operator and the autonomy-supervisor team that the proposed advisories are sensible.

- Stop. Edit YAML: change the binding's perTagSafetyMode to ADVISORY. Restart.

- Publishes begin. The supervisor sees them on the advisory topics.

There is no ADVISORY → AUTONOMOUS path unless simulatorOnly: true. In simulator mode this is a useful CI pattern; on a real robot, AUTONOMOUS is a separate bridge.

# 14. Limitations and future work

- Strategy A not built. Native ROS 2 via rcljava is a placeholder. For sub-millisecond latency to a real robot, you would either wait for Strategy A or run the bridge on the same host as rosbridge_websocket.

- JSON serialisation cost for binary topics. Images, dense point clouds, raw audio — rosbridge base64-encodes the bytes and the bridge decodes them. For high-rate binary traffic, prefer feature extraction in a ROS 2 node before crossing the bridge boundary.

- No per-topic QoS overrides yet. The qosProfile is a single global choice. ROS 2 supports per-topic QoS in principle; the bridge YAML does not expose this knob today.

- `asPeerObservation: true` is `Odometry`-only. Other peer-state messages (joint state of a peer arm) need a signalKind: PEER_OBSERVATION extension; that's future work.

- No structural identity validation. A shadow client with bridge credentials can publish to the same advisory topic the bridge writes to. The bridge audits its own writes; site-level ROS 2 SROS2 ACLs are the only defence.

# 15. Regulatory posture

| Aspect | Posture |
| --- | --- |
| Safety ceiling | ADVISORY. Structural — AUTONOMOUS rejected at config load unless simulatorOnly: true. |
| Default per-binding mode | SHADOW. Every binding must be explicitly promoted to ADVISORY. |
| Forbidden actuator topics | /cmd_vel, /joint_trajectory, /joint_command rejected at config load (compact constructor + Ros2BridgeConfig.FORBIDDEN_WRITE_TOPICS) unless simulatorOnly: true. |
| Interlock authority | None on the ROS 2 side. The robot's microcontroller is the SIL-rated interlock layer. |
| Operator override | Not applicable; advisory bridges do not write regulatory control. |
| Audit | One JSONL line per decision plus optional ROS 2 advisory audit topic. |
| MoC | Hot-reload not supported. YAML changes follow site MoC. |
| Network trust | DDS is peer-to-peer. Use SROS2 (sros2 keystore) for production deployments. |

For closed-loop control of a real robot, the path is: Jneopallium → ROS 2 advisory → autonomy supervisor → ROS 2 actuator topic. The bridge owns the first arrow; everything after is conventional robotics. Jneopallium's value here is providing the audit trail and the structural defences for the cognitive recommendations themselves.

# 16. References

- 04-ROS2-DDS.md — the bridge specification.

- 00-FRAMEWORK.md — the universal bridge contract.

- docs/ros2-bridge.md — domain-context companion in the repo.

- ROS 2 documentation — https://docs.ros.org/.

- DDS Foundation — https://www.dds-foundation.org/.

- rosbridge_suite — https://github.com/RobotWebTools/rosbridge_suite.

- rcljava — https://github.com/ros2-java/rcljava. Native Java client for Strategy A.

- SROS2 (security) — https://design.ros2.org/articles/ros2_dds_security.html.

- turtlesim tutorial — https://docs.ros.org/en/humble/Tutorials/Beginner-CLI-Tools/Introducing-Turtlesim/Introducing-Turtlesim.html.

- Gazebo — https://gazebosim.org/.

- Rakovskyi, D. (2024). Framework for Natural Neuron Network Modeling: The Jneopallium Approach. IJSR 13(7).